AMBALASCIENCEMARKET.COM- VIKRAM MITTAL /M-9812204116

# Central Welding Equipments

#### SMAW Equipment | GMAW Welding Equipment

GTAW Welding Equipment | SAW Welding Equipment

kitchen

Laundry

INDEX

Welding Equipment

Sr.

No.

PRODUCT	PAGE NO.

Sr.

No.

PRODUCT	PAGE NO.

##### 1	SOME BASICS TO KNOW

MOST COMMON TYPES OF INSULATION CLASSES USED IN WELDING EQUIPMENT

MOST COMMON TYPES OF ENCLOSURE PROTECTIONS USED IN WELDING EQUIPMENT

MOST COMMON TYPES OF DUTY CYCLES USED IN WELDING EQUIPMENT

ELECTRODE SELECTION TABLE HELP ME CHOOSE

22   SILENT CHALLENGER 501	49

3	23   SILENT CHALLENGER 2 X 301	51

- MAXIMIG 251	53

- MAXIMIG 400	55

3	25   STRIKER 400	57

- RANGER 600	59

- RANGER 600 MULTI	61

3	28   CHAMPMIG 250	63

- OMNIMIG 250	66

- CHAMPMIG 300	70

5	31   CHAMPMIG 400	72

32   CHAMPMIG 600	75

6	33   CHAMP MULTI 400, 600	78

- RED / TPA SERIES WELDING	8

TRANSFORMERS

- DOUBLE MUSTANG / DOUBLE RACER	12

DOUBLE HORSE / DOUBLE STALLION

- E-WELMAC 190	14

- GL SERIES WELDING RECTIFIERS	17

(DIODE BASED)

- THYROLUXE 401 / THYROLUXE 600	18

- THYROLUXE 1000 / THYROLUXE 1200	20

8	CHAMP 200	22

9	CHAMP 250	24

10   CHAMP 300	26

- CHAMP T400	28

- CHAMP 500 / CHAMP 600	30

- CHAMP 1200 - GOUGEMASTER	32

- CHAMPCUT 100	34

- CHAMPCUT 25C	36

- CHAMPCUT 150	37

17   CUTi 35, 70, 90, 120	39

- PLASMACUT - 25	41

- SUPERGEN 320	43

- SILENT CHALLENGER 301 S2	45

- SILENT CHALLENGER 401	47

34   CHAMP PULSE 500	83

35   HF 2000 / HF 2000 AD &	89

HF 3000 / HF 3000 AD

- CHAMPTIG 220P	91

- CHAMPTIG 300P	96

- CHAMPTIG 300 SP	98

- CHAMPTIG 400P	104

- CHAMPTIG 300 AD	107

- CHAMPTIG 500 AD	110

43   MAESTRO SERIES 800, 1000, 1200	114

44   UNI - MAESTRO RETROFIT	118

UNIT FOR SAW

46   THREE WHEEL TRACTOR	120

WELDING HEAD

48   MAESTRO 1200 (I)	121

49   ELECTRODE AND FLUX	125

DRYING OVENS

AMBALASCIENCEMARKET.COM- VIKRAM MITTAL /M-9812204116	www.modernkitchenequipments.com

# SOME BASICS TO KNOW

### MOST COMMON TYPES OF INSULATION CLASSES USED IN WELDING EQUIPMENT

The most common types of Insulation Classes, for which welding equipment are designed, are basically B, F and H. However electrical machines have following range of Insulation classes as per National and International standards. The maximum allowable temperatures are given below for these Insulation classes

- A = Class of Insulation --105 Deg. Centigrade

- E = Class of Insulation –120 Deg. Centigrade

- B = Class of Insulation –130 Deg. Centigrade

- F = Class of Insulation –155 Deg. Centigrade

- H = Class of Insulation –180 Deg. Centigrade

- C = Class of Insulation --220 Deg. Centigrade

### MOST COMMON TYPES OF ENCLOSURE PROTECTIONS USED IN WELDING EQUIPMENT

##### DEGREE OF INGRESS PROTECTION CLASS USED FOR ENCLOSURES OF WELDING EQUIPMENT

The most common Ingress Protection classes (IP class) used for the enclosure of the welding equipment are IP 21, IP 23 and IP 44. These protections classes of enclosures provide different degree of protection against Ingress of foreign body or particle and vertically falling water to the machines

- IP 21

- IP 23

- IP 44

##### FIRST DIGIT – Indicates protection against the ingress of foreign body or the particle

- 2 = Protection against ingress of foreign body / particle > 12 mm dia

- 4 = Protection against ingress of foreign body / particle > 1 mm dia

##### SECOND DIGIT – Indicates protection against the ingress of dripping or vertically falling water

- 1 = Protection against ingress of water i.e.dripping or vertically falling water

- 3 = Protection against ingress of spraying or splashing water up to 60 degrees

- 4 = Protection against the ingress of the splashing water from any direction i.e. all round the machine

### MOST COMMON TYPES OF DUTY CYCLES USED IN WELDING EQUIPMENT

##### DUTY CYCLES OF WELDING EQUIPMENT

Welding equipment are never used continuously for manual or semiautomatic welding processes. The common duty cycles observed for majority of welding applications are 25%, 35%, 40%, 60%. However all welding equipment are also supposed to be specified for 100% duty cycles. The duty cycle of the machine or application indicates how severely machines will be used to complete the jobs to be welded. A typical duty cycle is explained on next page, which indicate the WELD time and OFF time observed during the welding of job. The total CYCLE TIME consists of one WELD time and one OFF time associated with the respective WELD time. The CYCLE TIMES are 5 minute and 10 minute as per Indian and International standards respectively.

AMBALASCIENCEMARKET.COM- VIKRAM MITTAL /M-9812204116	www.modernkitchenequipments.com

3

# SOME BASICS TO KNOW

DUTY CYCLE

1 Load Cycle

Welding Current or

Load

ON

Time

6 min

OFF

Time

4 min

## t

% Duty cycle =

## =

(ON Time) X 100

## (ON Time + Off Time) 6 x 100 = 60%

(6 + 4)

### FINDING CURRENT FOR THE REQUIRED DUTY CYCLE WHEN CURRENT IS SPECIFIED AT DIFFERENT DUTY CYCLE

##### Present specs. are 400 A @ 60 % duty cycle

- Follow following steps to find the welding current at 100% duty cycle

- Losses @ d1 = Losses @ d2

##### Present specs. are 310 A @ 100 % duty cycle

- Follow following steps to find the welding current at 60% duty cycle

- Losses @ d1 = Losses @ d2

- (I1)2x r x t1 = (I2)2 x r x t2

- (400x400) x 6 = (I2)2  x 10

- (I2)2 = (6/10)x400x400

- I2 = sq. root of (6/10) x 400

- I2 = sq. root of 0.6 x 400

- I2 = 0.7746 X 400

- I2 = 309.8

- I2 = 310

- (I1)2x r x t1 = (I2)2 x r x t2

- (310x310) x 10 = (I2)2  x 6

- (I2)2 = (10/6) x 310 x 310

- I2 = sq. root of (10/6) x 310

- I2 = sq. root of 1. 66 x 310

- I2 = 1.29 x 310

- I2 = 400.2

- I2 = 400

AMBALASCIENCEMARKET.COM- VIKRAM MITTAL /M-9812204116	www.modernkitchenequipments.com

4

# SOME BASICS TO KNOW

### ELECTRODE TYPES / SIZES VS. RECOMMENDED WELDING CURRENTS

| SN. | ELECTRODE TYPE | AWS TYPE | WELDING CURRENTS (AMPS) FOR ELECTRODE SIZE (DIA) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 1.6MM | 2.0MM | 2.5MM | 3.2MM | 4.0MM | 5.0MM | 6.3MM |
| 1. | Superbond | E6013 | 30-50 | 55 40-60 | 80 60-90 | 125 100-130 | 170 140-180 | 220 180-240 | 275 |
| 2. | Superbond-S | E6013 | 30-50 | 55 40-60 | 80 60-90 | 125 100-140 | 170 140-180 | 220 180-250 | 250-330 |
| 3. | Superbond-SS | E6013 | 30-50 | 40-60 | 90 60-90 | 135 100-140 | 180 140-190 | 240 190-250 | 300 250-330 |
| 4. | Supabase | E7018 | 40-60 | 50-70 | 80 60-90 | 125 100-140 | 170 150-200 | 200 200-240 | 275 270-360 |
| 5. | Supabase-X-Plus | E7018 | 40-60 | 50-70 | 80 60-90 | 125 90-140 | 170 150-190 | 225 200-250 | 300 270-320 |
| 6. | Celwel 70G | E7010-G | - | - | 55 50-70 | 100 80-120 | 130 110-160 | 180 140-210 | - |
| 7. | Celwel 60 | E6010 | - | - | 55 50-70 | 100 80-120 | 130 110-160 | - | - |

GUIDING NOTES

##### Recommended Polarities

  - Superbond, Superbond-S, Superbond-SS = AC, DC (-)

  - Supabase, Supabase-X-Plus = AC (70 OCV), DC (+)

  - Celwel 60, Celwel 70G = DC(+) (for all passes), DC(-) (for root pass only)

##### Recommended Welding Positions

  - Superbond, Superbond-S, Superbond-SS, Celwel 60, Celwel 70G = All including vertical down (Vertical - Top to Bottom)

  - Supabase, Supabase-X-Plus = All except vertical down (Vertical - Top to Bottom)

##### Current Selection Generally Adapted

A) For Downhand flat, horizontal and overhead positions = Current at middle of the range is used.

- For Vertical up (Bottom to Top) = Current at maximum of the range is used.

- For vertical down (Top to Bottom) = Current at maximum of the range or slightly more than maximum is used.

- For the same diameter, as the thickness of parent material increases, employed current is increased.

AMBALASCIENCEMARKET.COM- VIKRAM MITTAL /M-9812204116	www.modernkitchenequipments.com

5

# SELECTION GUIDE

WELDING & CUTTING EQUIPMENT & PROCESSES

# HELP ME CHOOSE

THE BEST WELDING EQUIPMENT FOR DESIRED APPLICATION

STEP 1 METAL JOINING AND CUTTING PROCESS SELECTION

Select a process for a given metal you want to join or cut from the table given below

| WELDING PROCESS CUTTING PROCESS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| METAL TYPE | STICK | MIG | FCAW | SAW | AC-TIG | DC-TIG | CAC-A-AC | CAC-A-DC | PLASMA |
| STEEL |  |  |  |  |  |  |  |  |  |
| STAINLESS STEEL |  |  |  |  |  |  |  |  |  |
| ALUMINIUM |  |  |  |  |  |  |  |  |  |
| CAST IRON |  |  |  |  |  |  |  |  |  |
| COPPER, BRASS |  |  |  |  |  |  |  |  |  |
| TITANIUM |  |  |  |  |  |  |  |  |  |
| MAGNESIUM ALLOYS |  |  |  |  |  |  |  |  |  |
| SKILL LEVEL | MODERATE | LOW | LOW | MODERATE | HIGH | HIGH | MODERATE | MODERATE | HIGH |

### STEP 2 WELDING PROCESS SELECTION

It is possible that more than one welding process can be used for a given requirement.

Hence to facilitate selection, study and understand the advantages of each process as given below.

| A | SMAW / MMAW / STICK WELDING | Most suitable for outside/opendoor use One of jobs or less volume jobs. Better accessibility for intricate joints Ideal for dirty/rusty surfaces |
| --- | --- | --- |
| B | GMAW/MIG/MAG/SEMIAUTOMATIC/ CONTINUOUS WELDING | Suitable for high welding productivity Most suitable for relatively thinner sheets Very neat and clean welding without slag removable Easy to learn for welders |
| C | FLUX CORED (FCAW) MIG/MAG | Suitable for deep penetration for welding thick sections with less nos. of runs Provides higher welding deposition rates Gives excellent weld bead shape |
| D | SUBMERGED ARC WELDING (SAW) | Most suitable where excellent mechanical properties are required Ideally suitable for heavy duty longitudinal structures like girders High deposition rates gives higher welding productivity Gives excellent weld bead shape |
| E | TIG WELDING (GTAW) | Used where highest quality welding required Gives precise welding with very good welding finish Most suitable for thinnest sheets where better heat input controls are required and very minimal distortions allowed |
| F | AIR PLASMA ARC CUTTING | Used with almost all electrical conducting metals. Distortion free, dross free, burr free, narrow kert with clean cuts, compared to carbon ARC cutting. |

AMBALASCIENCEMARKET.COM- VIKRAM MITTAL /M-9812204116	www.modernkitchenequipments.com

6

2

SELECT WELDING / CUTTING PROCESS DEPENDING ON WELDING (OR CUTTING) PROCESS DESIGN (WELD PARAMETERS / ELECTRODE/WIRE DIA etc) AND PRODUCTION VOLUME REQUIREMENT, TARGET etc,  AFTER CONSIDERING THE MERITS OF EACH PROCESS

3

CARRY OUT ADDITIONAL CHECKS FOR POWER SUPPLY AND EVIRONMENT CONDITIONS, CONSULT

FACTORY / TECHNICAL MARKETING FOR ANY SPECIAL / CUSTOMISED REQUIREMENT OF APPLICATION IF ANY.

4

CHECK AND DECIDE WORK LOAD, DUTY CYCLE AND SEVERITY OF USAGE AS PER GUIDELINES GIVEN IN THE TABLE

5

ONCE ABOVE STEPS ARE COMPLETED, CHECK AC OR DC TYPE OF POWER SOURCE FOR SMAW/TIG PROCESSES AND THEN SELECT THE MODEL FROM THE TABLE GIVEN BELOW

# HELP ME CHOOSE

THE BEST WELDING EQUIPMENT FOR DESIRED APPLICATION

STEP 3 ADDITIONAL CHECK LIST

Also check for following along with above mentioned process advantages

| 1. | POWER SUPPLY CONDITIONS* | - Stable / Unstable / Very Low / Very High voltage condition |  |
| --- | --- | --- | --- |
| 2. | ENVIRONMENTAL CONDITIONS* | - Chemical (Acidic) Contamination. Conducting Dust Contamination. |  |
|  |  | - Economy |  |
| 3. | BUDGET |  |  |

* Consult factory / Technical Marketing Team for extreme supply and environmental conditions

### STEP 4 SELECTION OF WORK LOAD AND DUTY CYCLE

| Workload/usage/duty | Likely Duty Cycle | Welding Current Range Recommended |
| --- | --- | --- |
| Light Duty - Single Shift | 20 % to 35 % | Upto 225 Amps |
| Medium Duty - Single or Two Shift | >35 % to 60 % | 225 Amps to 300 Amps |
| Heavy Duty - Two or Three Shift | >60% to 100 % | More than 300 Amps |

STEP 5 MODEL SELECTION TABLE

Thus after selecting the process, duty cycle, and going through the additonal checklist, please verify once again the steps completed so far as per following FLOW CHART as a QUICK RE-CHECK and select the model for your welding/cutting need from the model selection table

EQUIPMENT SELECTION GUIDELINES-QUICK RE-CHECK

1

CHECK AND VERIFY THE WELDING/CUTTING PROCESS FOR THE METALS TO BE JOINED OR CUT FROM THE TABLE GIVEN

AMBALASCIENCEMARKET.COM- VIKRAM MITTAL /M-9812204116	www.modernkitchenequipments.com

7

WELDING TRANSFORMERS WITH FORCED AIR COOLING

# RED SERIES

WELDING TRANSFORMERS

RED 400 (S) / RED 401 (S)

400 Amp capacity World Class Welding Transformer

1

Phase

3

CC

Phase

- Single phase (2 lines of 3 phase) Welding Transformers

- User friendly AC ARC Welder

- Easily maneuverable from job to job on the shop- floor

SALIENT FEATURES

- Smooth welding current control by modern moving core magnetic shunt design

- Stepless, smooth and infinitely variable current regulation

- Adjustment of welding current possible even while welding is in progress

### SPECIFICATIONS

- Stepless, smooth and infinitely variable current regulation

- Forced air cooled transformers - less coil temperature resulting in longer service life

- Current Indicator provided on side panel

- ON-OFF switch control and current control provided on the front panel

- A Special design provides for excellent dynamic characteristic and ensures smooth and optimum metal transfer

| MODELS |  |  | UNIT | RED 400 (S) | RED 401 (S) |
| --- | --- | --- | --- | --- | --- |
| INPUT |  |  |  |  |  |
| Supply Voltage |  |  | Volts | 415 | 415 |
| Phases |  |  | No. | 2 lines of 3 phase | 2 lines of 3 phase |
| Frequency |  |  |  | 50 | 50 |
| Input KVA @ 100% Duty Cycle |  |  | KVA | 19 | 12.6 |
| Input switch fuse rating |  |  | A | 80 | 63 |
| Recommended Capacitor for P.F. improvement |  |  | KVAr | 8 | 8 |
| OUTPUT |  |  |  |  |  |
| Output Characteristic |  |  | Type | CC | CC |
| Operating Arc Voltage |  |  | Volts | 22-36 | 22-36 |
| Open circult voltage |  |  | Volts | 80 | 70 |
| Welding current range |  |  | A | 60-400 in two overlapping ranges | 60-400 in two overlapping ranges |
| Max. Continuous hand welding |  |  |  |  |  |
| current @ 60% Duty Cycle |  |  | A | 300 | 230 |
| current @100% Duty Cycle |  |  | A | 230 | 180 |
| Max. Intermittent welding current |  |  | A | 400 | 400 |
| Welding Electrode range |  |  |  |  |  |
| Minimum |  |  | mm | 2 | 2 |
| Maximum |  |  | mm | 6.3 | 5 |

AMBALASCIENCEMARKET.COM- VIKRAM MITTAL /M-9812204116	www.modernkitchenequipments.com

8

# RED 400 (S) / RED 401 (S)

400 Amp capacity World Class Welding Transformer

| GENERAL |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Cooling |  |  | Type | Forced Air | Forced Air |
| Protection class |  |  | Class | IP23 | IP23 |
| Dimensions (approx.): |  |  |  |  |  |
| Length |  |  | mm | 745 | 730 |
| Width |  |  | mm | 460 | 415 |
| Height |  |  | mm | 540 | 490 |
| Weight (approx.) |  |  | Kg. | 98 | 81 |

- Warranty : One year from the date of commissioning. ADOR WELDING LIMITED warrants that all new equipment sold from Plant/Area Offices / Authorised Distributors are free from defects in materials and workmanship and will perform in full accordance with applicable specifications.

- In view of continuous development, ADOR WELDING LIMITED reserves the right to modify/change the design and /or the specifications without any prior notice.

- Backed by dedicated customer care package.

AMBALASCIENCEMARKET.COM- VIKRAM MITTAL /M-9812204116	www.modernkitchenequipments.com

9

# RED SERIES

WELDING TRANSFORMERS

RED SERIES

A Wide Range of World Class Welding Transformers

1

Phase

3

CC

Phase

- Single phase (2 lines of 3 phase) Welding Transformers

- Class F Insulation - enhances services life

- Easily maneuverable from job to job on the shop- floor

- Stepless, smooth and infinitely variable current regulation

- Forced air cooled transformers - less coil temperature resulting in longer service life

- User friendly AC ARC Welders

# TPA SERIES

WELDING TRANSFORMERS

TPA SERIES

A Wide Range of World Class Welding Transformers

1	3	CC

Phase Phase

- Three phase Welding Transformers

- User friendly AC ARC Welders

- Stepless, smooth and infinitely variable current regulation

- Convenient and easily maneuverable from job to job on the shop-floor

- ‘Open-delta’ design enables conversion of three phase input into single phase output

- Forced air cooled transformers - less coil temperature resulting in longer service life

- Compact and lightweight

SALIENT FEATURES

- Smooth welding current control by moving core magnetic shunt design

- Stepless, smooth and infinitely variable current regulation

- Adjustment of welding current possible even while welding is in progress

- 1 swivel front wheel and 2 rear wheel arrangement for easy maneuverability

- ON-OFF switch control, current control and current indicator provided on the front panel

- A Special design provides for excellent dynamic characteristic and ensures smooth and optimum metal transfer - RED Series specifically recommended for low hydrogen electrodes

AMBALASCIENCEMARKET.COM- VIKRAM MITTAL /M-9812204116	www.modernkitchenequipments.com

10

# RED / TPA SERIES WELDING TRANSFORMERS

A Wide Range of World Class Welding Transformers

| MODELS |  |  | UNIT | RED 403 | RED 503 | RED 603 | TPA 403 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| INPUT |  |  | Volts | 415 | 415 | 415 | 415 |
| Supply Voltage | 1 |  |  |  |  |  |  |
| Phases |  |  | No. | 2 Lines of 3 Phase | 2 Lines of 3 Phase | 3 | 3 |
| Frequency |  |  | Hz | 50 | 50 | 50 | 50 |
| Input KVA @ 100 % Duty Cycle |  |  | KVA | 19 | 25 | 28.4 | 19.0 |
| Input switch fuse rating |  |  | A | 80 | 100 | 80 | 80 |
| Recommended Capacitor |  |  | KVAr | 8 | 10 | 10 | 10 |
| for P.F. improvement |  |  |  |  |  |  |  |
| OUTPUT |  |  |  |  |  |  |  |
| Output Characteristic |  |  | Type | CC | CC | CC | CC |
| Operating Arc Voltage |  |  | Volts | 22-36 | 23-40 | 24-44 | 22-36 |
| Open circult voltage |  |  | Volts | 80 | 80 | 80 | 80 |
| Welding current range |  |  | A | 60-400 | 80-500 | 100-600 | 60-400 |
| Max. Continuous hand welding |  |  |  |  |  |  |  |
| current @ 60% Duty Cycle |  |  | A | 300 | 400 | 450 | 300 |
| current @100% Duty Cycle |  |  | A | 230 | 300 | 350 | 230 |
| Max. Intermittent welding current |  |  | A | 400 | 500 | 600 | 400 |
| Welding Electrode range |  |  |  |  |  |  |  |
| Minimum |  |  | mms | 2 | 2.5 | 2 | 2 |
| Maximum |  |  | mms | 6.3 | 6.3 | 6.3 | 6.3 |
| GENERAL |  |  |  |  |  |  |  |
| Class of Insulation |  |  | Class | F | F | F | A |
| Cooling Type |  |  | Type | Forced Air | Forced Air | Forced Air | Forced Air |
| Protection class |  |  | Class | IP23 | IP23 | IP23 | IP23 |
| Dimensions (mms) L |  |  | mms | 720 | 745 | 820 | 800 |
| W |  |  | mms | 460 | 500 | 560 | 500 |
| H |  |  | mms | 750 | 780 | 850 | 850 |
| Weight (approx.) |  |  | kgs. | 110 | 132 | 160 | 160 |

- Warranty : One year from the date of commissioning. ADOR WELDING LIMITED warrants that all new equipment sold from Plant/Area Offices / Authorised Distributors are free from defects in materials and workmanship and will perform in full accordance with applicable specifications.

- In view of continuous development, ADOR WELDING LIMITED reserves the right to modify/change the design and /or the specifications without any prior notice.

- Backed by dedicated customer care package.

AMBALASCIENCEMARKET.COM- VIKRAM MITTAL /M-9812204116	www.modernkitchenequipments.com

11

WELDING TRANSFORMERS WITH NATURAL AIR COOLING

# DOUBLE MUSTANG / DOUBLE RACER DOUBLE HORSE / DOUBLE STALLION

Thoroughbred Series AC ARC Welding Transformers

CC

1

Phase

  - Light weight and compact transformers with robust construction - Ideal choice for site usage

  - Natural air cooled transformers - coils designed for natural air cooling which eliminates the use of fan and thus risk of coil burning whenever fan fails

  - Specially designed transformer coils with minimum joints and better interlayer insulation -eliminates the coil failure and enhances service life of transformer

  - Stepless, infinitely variable current adjustment through sturdy and smooth moving core mechanism

  - Stepless, smooth and infinitely variable current adjustment possible even while welding is in progress

SALIENT FEATURES

  - Ensures the fine control of welding current to minimise the heat input to the job which reduces the HAZ

  - Dynamic output characteristics ensures smooth and uninterrupted arc resulting excellent welding performance througout the entire welding range

  - Provided with input power ON-OFF switch

  - Welder friendly usage - easy to operate since all controls like ON-OFF switch, welding current controlling handle, current scale on the same side i.e. front side

AMBALASCIENCEMARKET.COM- VIKRAM MITTAL /M-9812204116	www.modernkitchenequipments.com

12

# DOUBLE MUSTANG / DOUBLE RACER DOUBLE HORSE / DOUBLE STALLION

Thoroughbred Series AC ARC Welding Transformers

| SPECIFICATIONS |  |  | UNIT | DOUBLE MUSTANG | DOUBLE RACER | DOUBLE HORSE | DOUBLE STALLION |
| --- | --- | --- | --- | --- | --- | --- | --- |
| INPUT |  |  | Volts | 230/400 | 415 | 415 | 415 |
| Voltage Supply |  |  | No. | 1 | 1 | 1 | 1 |
| Phases |  |  |  | (.......................... | ..........2 Lines of 3 | Phase AC................ | ......................) |
| Frequency |  |  | Hz | 50 | 50 | 50 | 50 |
| Input KVA @ 100% Duty Cycle |  |  | KVA | 10.5 | 16.0 | 21.0 | 25.5 |
| Recommended Capacitor |  |  |  |  |  |  |  |
| for PF improvement |  |  | KVAr | 5.0 | 8.0 | 9.0 | 11.0 |
| OUTPUT |  |  |  |  |  |  |  |
| Open Circuit Voltage |  |  | V | 66 | 80 | 80 | 80 |
| Welding Current Range |  |  | A | 35-250 | 50-300 | 60-400 | 80-500 |
| Maximum intermittent |  |  |  | (.......................... | ............Single Rang | e Control............... | .....................) |
| welding current at 35% duty |  |  | A | 250 | 300 | 400 | 500 |
| Maximum continuous |  |  |  |  |  |  |  |
| welding current |  |  |  |  |  |  |  |
| at 60% duty cycle |  |  | A | 190 | 225 | 300 | 375 |
| Maximum automatic |  |  |  |  |  |  |  |
| welding current |  |  |  |  |  |  |  |
| at 100% duty cycle |  |  | A | 150 | 175 | 230 | 300 |
| GENERAL SPECIFICATIONS |  |  |  |  |  |  |  |
| Type of Cooling |  |  |  | (................ | .............Natural Air | Cooled................... | ...........) |
|  |  |  | Class | H | H | H | H |
| Insulation |  |  |  |  |  |  |  |
| Recommended electrode |  |  |  |  |  |  |  |
| size/range |  |  | mm | 2-5 | 2-2.6 | 2-6.3 | 2.5-6.3 |
| Overall dimensions: |  |  |  |  |  |  |  |
| Length Width |  |  | mm mm | 590 425 | 650 475 | 650 500 | 650 550 |
| Height |  |  | mm | 660 | 700 | 725 | 755 |
| Weight (approx.) |  |  | kg | 92 | 120 | 140 | 170 |

- Warranty : One year from the date of commissioning. ADOR WELDING LIMITED warrants that all new equipment sold from Plant/Area Offices / Authorised Distributors are free from defects in materials and workmanship and will perform in full accordance with applicable specifications.

- In view of continuous development, ADOR WELDING LIMITED reserves the right to modify/change the design and /or the specifications without any prior notice.

- Backed by dedicated customer care package.

AMBALASCIENCEMARKET.COM- VIKRAM MITTAL /M-9812204116	www.modernkitchenequipments.com

13

# E-WELMAC 190

E-WELMAC AC ARC Welding Transformers

CC

1

Phase

- Light weight, compact AC arc welding transformer

- easy to mobilise at sites

- Single phase, natural air cooled transformer with robust construction

- Most reliable and simple transformer - very convenient and welder friendly to use at sites

- Infinitely variable current adjustment through sturdy and smooth moving core mechanism

SALIENT FEATURES

- Welding current adjustment possible even while welding is in progress

- Dynamic output characteristics ensures smooth and uninterrupted arc resulting excellent welding performance throughout the entire welding range

- H-class insulation of windings enhances reliability and service life

- Provided with input power ON-OFF switch

AMBALASCIENCEMARKET.COM- VIKRAM MITTAL /M-9812204116	www.modernkitchenequipments.com

14

# E-WELMAC 190

E-WELMAC AC ARC Welding Transformers

| MODELS |  |  | UNIT | E-WELMAC 190 |
| --- | --- | --- | --- | --- |
| INPUT |  |  | Volts | 230 |
| Input Supply: |  |  | No. | 1 |
| Voltage |  |  | Hz | 50 |
| Phases |  |  |  |  |
| Frequency |  |  | KVA | 5.5 |
| Input KVA @ 100% Duty Cycle |  |  |  |  |
| OUTPUT |  |  |  |  |
| Open Circuit Voltage |  |  | Volts | 50 |
| Welding Current Range |  |  | Amps | 30-150 |
| Current: |  |  |  |  |
| @20% duty cycle |  |  | Amps | 200 |
| @60% duty cycle |  |  | Amps | 115 |
| @100% duty cycle |  |  | Amps | 90 |
| Welding Electrode range |  |  | mm | 2.0-4.0 |
| GENERAL |  |  |  |  |
| Cooling |  |  | Type | ---------------Natural Air Cooled--------------- |
| Dimensions (approx.): |  |  | Class | H |
| Length |  |  | mm | 340 |
| Width |  |  | mm | 315 |
| Height |  |  | mm | 500 |
| Weight (approx.) |  |  | Kg. | 39 |

- Warranty : One year from the date of commissioning. ADOR WELDING LIMITED warrants that all new equipment sold from Plant/Area Offices / Authorised Distributors are free from defects in materials and workmanship and will perform in full accordance with applicable specifications.

- In view of continuous development, ADOR WELDING LIMITED reserves the right to modify/change the design and /or the specifications without any prior notice.

- Backed by dedicated customer care package.

AMBALASCIENCEMARKET.COM- VIKRAM MITTAL /M-9812204116	www.modernkitchenequipments.com

15

# GL SERIES WELDING RECTIFIERS (DIODE BASED)

A Wide of World Class Welding Rectifiers

CC

GOUGING

### SALIENT FEATURES

  - Transductor controlled rectifier units.

  - 600 Amps ideally suited for gouging application

  - Welding rectifiers based on most simple, reliable and maintenance free diode based technology.

  - These welding rectifiers are built with most robust and rugged mechanical construction which withstand rough handling at construction and project sites.

  - Welding current adjustment and regulation through electromagnetic transductor resulting stepless and smooth current control.

- These welding rectifiers are welder friendly - Very easy to use and operate, maintain and service by ordinary electricians.

- Single knob current control from the front panel or remote control unit (provided optionally.)

### APPLICATIONS

- Most simple, robust and reliable welding rectifiers ideally recommended for heavy duty structural welding of construction equipment like crane, steel foundries, steel and fertiliser plants, shipyards.

- These welding rectifiers are ideally suitable for heavy fabrication shops, project sites with harsh

### SPECIFICATIONS

- Easy parallel connection of these welding rectifiers possible for high current gouging applications.

- These welding rectifiers can be also used for DC TIG welding applications by connecting suitable TIG control units.

environment, where maintenance staff is not well equipped and qualified.

- GL 601 is most recommended for heavy duty welding and gouging applications in foundries, steel plants, heavy engineering workshops etc.

| Technical Specifications |  |  | Unit | GL 401 | GL 601 |
| --- | --- | --- | --- | --- | --- |
| Input |  |  |  |  |  |
| Input Supply : |  |  |  |  |  |
| Supply Voltage |  |  | Volt | 400/440 | 400/440 |
| Phase |  |  | No | 3 | 3 |
| Frequency |  |  | Hz | 50 | 50 |
| Input KVA @ 100% duty cycle |  |  | KVA | 26 | 38.5 |
| Input Current @ 100% duty cycle |  |  | A | 45 | 70 |
| Recommended switch fuse rating |  |  | A | TP50 | TP80 |
| Output |  |  |  |  |  |
| Output Characteristic |  |  | Type | CC | CC |
| Open circuit voltage |  |  | Volts | 80 | 80 |
| Welding current range |  |  | A | 50-400 | 80-600 |

AMBALASCIENCEMARKET.COM- VIKRAM MITTAL /M-9812204116	www.modernkitchenequipments.com

16

# GL SERIES WELDING RECTIFIERS (DIODE BASED)

A Wide of World Class Welding Rectifiers

Cont.

| @ 35% duty cycle |  |  | A | NA | NA |
| --- | --- | --- | --- | --- | --- |
| @ 60% duty cycle |  |  | A | 400 | 600 |
| @ 100% duty cycle |  |  | A | 310 | 465 |
| General |  |  |  |  |  |
| Class of Insulation |  |  | Class | B | B |
| Cooling Type |  |  | Type | Fan cooled |  |
| Protection class |  |  | Class | IP23 | IP23 |
| Dimensions (mm) |  |  |  |  |  |
| Length |  |  | mm | 840 | 907 |
| Height |  |  | mm | 705 | 750 |
| Width |  |  | mm | 905 | 1005 |
| Weight (approx.) |  |  | Kg | 275 | 430 |

- Warranty : One year from the date of commissioning. ADOR WELDING LIMITED warrants that all new equipment sold from Plant/Area Offices / Authorised Distributors are free from defects in materials and workmanship and will perform in full accordance with applicable specifications.

- In view of continuous development, ADOR WELDING LIMITED reserves the right to modify/change the design and /or the specifications without any prior notice.

- Backed by dedicated customer care package.

AMBALASCIENCEMARKET.COM- VIKRAM MITTAL /M-9812204116	www.modernkitchenequipments.com

17

THYROLUXE 401

THYROLUXE 600

THYROLUXE 401 / THYROLUXE 600

The world class Thyristor based DC welders

CC

GOUGING

### SALIENT FEATURES

#### Choice of heavy duty models to take care of both welding & gouging (for 600 A model only)

  - Superior welding performance for the complete welding current range

#### Ideal constant current drooping characteristics

  - Stepless control for current adjustments

#### Protections against input supply fluctuations i.e Under voltage, Over voltage, Single phasing, over load and short circuit

  - Power source provided with built in Hot start, Antistick and self controlled Arc force dynamics

#### Worry-free on maintenance

  - Smooth and stable arc with minimum spatters

#### Welder friendly Remote controller for easy and convenient setting of current from the workplace or the job

  - Easy arc striking, High OCV for ease in arc start / restart

AMBALASCIENCEMARKET.COM- VIKRAM MITTAL /M-9812204116	www.modernkitchenequipments.com

18

# THYROLUXE 401 / THYROLUXE 600

The world class Thyristor based DC welders

| SPECIFICATIONS TECHNICAL SPECIFICATIONS |  |  | UNIT | THYROLUXE 401 | THYROLUXE 600 |
| --- | --- | --- | --- | --- | --- |
| INPUT |  |  |  |  |  |
| Input Supply: Voltage |  |  | Volts | 415, +10, -15% | 415, +10, -15% |
| Phase |  |  | No. | 3 | 3 |
| Frequency |  |  | Hz. | 50/60 | 50/60 |
| Input KVA @100% duty cycle |  |  | KVA | 19.5 | 31.5 |
| Input current @100% duty cycle |  |  | Amps AC | 26.0 | 44 |
| Recommended switch fuse/rating |  |  | Amps AC | TP-35 | TP-60 |
| OUTPUT |  |  |  |  |  |
| Open Circuit Voltage |  |  | Volts DC | 100 | 100 |
| Welding Current Range |  |  | Amps DC | 100-400 | 20-600 |
| Current: |  |  |  |  |  |
| @40% duty cycle |  |  | Amps DC | - | - |
| @60% duty cycle |  |  | Amps DC | 400 | 600 |
| @100% duty cycle |  |  | Amps DC | 310 | 465 |
| GENERAL |  |  |  |  |  |
| Gouging Carbon Electrode size |  |  | mm Ø | - | 6 to 9 |
| Current Display |  |  | - | Analog | Analog |
| Ingress Protection |  |  | Class | IP23 | IP23 |
| Cooling |  |  | Type | Forced Air | Forced Air |
| Insulation |  |  | Class | H | H |
| Dimensions |  |  |  |  |  |
| Length |  |  | mm | 835 | 980 |
| Width |  |  | mm | 495 | 550 |
| Height |  |  | mm | 820 | 960 |
| Weight (approx.) |  |  | Kg. | 147 | 216 |

- Warranty : One year from the date of commissioning. ADOR WELDING LIMITED warrants that all new equipment sold from Plant/Area Offices / Authorised Distributors are free from defects in materials and workmanship and will perform in full accordance with applicable specifications.

- In view of continuous development, ADOR WELDING LIMITED reserves the right to modify/change the design and /or the specifications without any prior notice.

- Backed by dedicated customer care package.

AMBALASCIENCEMARKET.COM- VIKRAM MITTAL /M-9812204116	www.modernkitchenequipments.com

19

# THYROLUXE 1000

THYROLUXE 1200

The world class Thyristor based DC welders

CC

GOUGING

### SALIENT FEATURES

- Thyristor based heavy duty DC Welding and Gouging Rectifiers.

- Stepless control for current adjustments.

- Protections against supply fluctuations i.e Under voltage, Over voltage, Single phasing, over load and short circuit.

- Constant current drooping characteristics – Ideal for Welding and Gouging applications.

- Power source provided with built in Hot start, Antistick and self controlled Arc force dynamics.

- Smooth and stable arc with minimum spatters

- Welder friendly Remote controller for easy and convenient setting of current from the work place or the job

| SPECIFICATIONS TECHNICAL SPECIFICATIONS | UNIT | THYROLUXE 1000 | THYROLUXE 1200 |
| --- | --- | --- | --- |
| INPUT |  |  |  |
| Input Supply: | Volts | 415 , +10%, -10% | 415 , +10%, -10% |
| Voltage | No. | 3 | 3 |
| Phase | Hz | 50/60 | 50/60 |
| Frequency |  |  |  |
| Max. rated Input KVA | KVA | 81 | 96.5 |
| OUTPUT |  |  |  |
| Open Circuit Voltage | Volts DC | 100 | 100 |
| Welding Current Range | Amps DC | 100-1000 | 100-1200 |
| Current: |  |  |  |
| @40% duty cycle | Amps DC | 1000 | - |
| @60% duty cycle | Amps DC | 850 | 1200 |
| @100% duty cycle | Amps DC | 650 | 930 |

AMBALASCIENCEMARKET.COM- VIKRAM MITTAL /M-9812204116	www.modernkitchenequipments.com

20

# THYROLUXE 1000

THYROLUXE 1200

The world class Thyristor based DC welders

| SPECIFICATIONS TECHNICAL SPECIFICATIONS | UNIT | THYROLUXE 1000 | THYROLUXE 1200 |
| --- | --- | --- | --- |
| GENERAL |  |  |  |
| Suitable Gouging Carbon Electrode size | mmØ | 6 to 12 mm | 6 to 12 mm |
| Current Display |  | Analog | Analog |
| Ingress Protection | Class | IP23 | IP23 |
|  | Type | Forced Air | Forced Air |
| Cooling | Class | H | H |
| Insulation |  |  |  |
| Dimensions |  |  |  |
| Length | mm | 1130 | 1200 |
| Width | mm | 770 | 770 |
| Height | mm | 1080 | 1175 |
| Weight (approx.) | Kg. | 403 | 450 |

- Warranty : One year from the date of commissioning. ADOR WELDING LIMITED warrants that all new equipment sold from Plant/Area Offices / Authorised Distributors are free from defects in materials and workmanship and will perform in full accordance with applicable specifications.

- In view of continuous development, ADOR WELDING LIMITED reserves the right to modify/change the design and /or the specifications without any prior notice.

- Backed by dedicated customer care package.

AMBALASCIENCEMARKET.COM- VIKRAM MITTAL /M-9812204116	www.modernkitchenequipments.com

21

# CHAMP 200

The world class Inverter based DC Welder

CC

DC TIG

### SALIENT FEATURES

- Single phase High efficiency DC Welder

- High frequency IGBT based PWM Inverter

- Light weight, compact and portable for easy handling

- Compatible to power generator supply

### SPECIFICATIONS

- Hot start feature available

- Smooth and stable arc with minimum spatter

- TIG welding with scratch/Lift arc start

- Provided with the UV, OV thermal and line Voltage (415V) Protections

| Technical Specifications |  |  | Unit | CHAMP 200 |
| --- | --- | --- | --- | --- |
| Input |  |  |  |  |
| Input Supply : |  |  |  |  |
| Voltage |  |  | Volt | 240, +10%, -15% |
| Phase |  |  | No | 1 |
| Frequency |  |  | Hz | 50/60 |
| Max input KVA @ 100% duty cycle |  |  | KVA | 5 |
| Efficiency |  |  | % | >84 |
| Output |  |  |  |  |
| Open Circuit Voltage @ 240 Vac |  |  | Volt | 68 +/-3 |
| Welding Current Range |  |  | Amps | 10-200 |
| Welding Current (40°C 10 minute cycle) |  |  |  |  |
| @ 35% duty cycle |  |  | Amps | 200 |
| @ 60% duty cycle |  |  | Amps | 150 |
| @ 100% duty cycle |  |  | Amps | 120 |
| General |  |  |  |  |
| Suitable Welding Electrode size |  |  | mm Ø | 1.6, 2.5, 3.2 and intermittent 4 |
| Hot Start |  |  | - | 120% of the set current |
| Antistick |  |  | - | Built in |
| Ingress Protection |  |  | Class | IP23 |
| Cooling |  |  | Type | Forced Air |
| Insulation |  |  | Class | F |
| Welding Output Terminals |  |  |  | Stud type / Camlock |

Cont.

AMBALASCIENCEMARKET.COM- VIKRAM MITTAL /M-9812204116	www.modernkitchenequipments.com

22

# CHAMP 200

The world class Inverter based DC Welder

| Technical Specifications |  |  | Unit | CHAMP 200 |
| --- | --- | --- | --- | --- |
| General |  |  |  |  |
| Dimensions |  |  |  |  |
| Length |  |  | mm | 415 |
| Width |  |  | mm 1 | 155 |
| Height |  |  | mm | 300 |
| Weight (approx.) |  |  | Kg | 7 |

- Warranty : One year from the date of commissioning. ADOR WELDING LIMITED warrants that all new equipment sold from Plant/Area Offices / Authorised Distributors are free from defects in materials and workmanship and will perform in full accordance with applicable specifications.

- In view of continuous development, ADOR WELDING LIMITED reserves the right to modify/change the design and /or the specifications without any prior notice.

- Backed by dedicated customer care package.

AMBALASCIENCEMARKET.COM- VIKRAM MITTAL /M-9812204116	www.modernkitchenequipments.com

23

# CHAMP 250

The world class Inverter based DC Welder

CC

DC TIG

### SALIENT FEATURES

- Three phase PWM controlled inverter based, high efficiency and high power factor heavy duty DC welder

- Enhanced Reliability due to SMD technology

- High frequency IGBT based Rectifier

- Arc force adjustment on panel

- TIG Welding possible with External HF Unit

- Light weight, compact and portable for easy handling

- High efficiency

- Built in Hot Start

- Smooth and stable arc with minimum Spatter

### PROTECTIONS WITH AUTO RESET

- Input Supply Voltage protections for over and under Voltage

### SPECIFICATIONS

- Over Temperature

- Protection against Single phasing

| Technical Specifications |  |  | Unit | CHAMP 250 |
| --- | --- | --- | --- | --- |
| Input |  |  |  |  |
| Input Supply : |  |  |  |  |
| Voltage |  |  | Volt | 415, +15%, -10% |
| Phase |  |  | No | 3 |
| Frequency |  |  | Hz | 50/ 60 |
| Input KVA @ 415 Vac |  |  |  |  |
| @ 100% duty cycle |  |  | KVA | 7.5 |
| @ No Load |  |  | KVA | 0.15 |
| Power Factor |  |  | CosØ | 0.94 |
| Efficiency |  |  | % | 85 |
| Output |  |  |  |  |
| Open Circuit Voltage |  |  | Volt | 84 |
| Welding Current Range |  |  | Amps | 10-250 |
| Welding Current (40°C 10 minute cycle) |  |  |  |  |
| @ 60% duty cycle |  |  | Amps | 250 |
| @ 100% duty cycle |  |  | Amps | 195 |

Cont.

AMBALASCIENCEMARKET.COM- VIKRAM MITTAL /M-9812204116	www.modernkitchenequipments.com

24

# CHAMP 250

The world class Inverter based DC Welder

Cont.

| General |  |  |  |  |
| --- | --- | --- | --- | --- |
| Suitable Welding Electrode size |  |  | mm Ø | 1.6, 2.5, 3.2, 4 and intermittent 5 mm |
| Arc force setting |  |  | - | Adjustable by Potentiometer |
| Built in Hot Start |  |  | - | 25% more than set current for 2 seconds |
| Current Display (Actual) |  |  |  | 3 digit –7 segment digital panel pcb |
| Ingress Protection |  |  | Class | IP23 |
| Cooling |  |  | Type | Forced Air |
| Insulation |  |  | Class | H |
| Insulation |  |  |  |  |
| Dimensions |  |  |  |  |
| Length |  |  | mm | 520 |
| Width |  |  | mm | 260 |
| Height (with handle) |  |  | mm | 450 |
| Height (without handle) |  |  | mm | 410 |
| Weight (approx.) |  |  | Kg | 26 |

- Warranty : One year from the date of commissioning. ADOR WELDING LIMITED warrants that all new equipment sold from Plant/Area Offices / Authorised Distributors are free from defects in materials and workmanship and will perform in full accordance with applicable specifications.

- In view of continuous development, ADOR WELDING LIMITED reserves the right to modify/change the design and /or the specifications without any prior notice.

- Backed by dedicated customer care package.

AMBALASCIENCEMARKET.COM- VIKRAM MITTAL /M-9812204116	www.modernkitchenequipments.com

25

# CHAMP 300

The world class Inverter based DC Welder

CC

DC TIG

### TYPICAL APPLICATIONS

- Three phase inverter based, high efficiency and high power factor DC Welder

- High frequency IGBT based Rectifier

- Arc force adjustment on panel.

- TIG Welding possible with External HF Unit handling

- Light weight, compact and portable for easy handling

- Auto-selection of Remote control when Remote controller (optional) is plugged into the machine

- Can be equipped with VRD, ELCB, Current and Voltage Dual-display meters as optional features

### SALIENT FEATURES

- IGBT based latest PWM inverter technology

- High efficiency

- High OCV and suitable for all types of electrodes.

- Smooth and stable arc with minimum spatter

- Controls provided for adjustments of Arc force

- TIG operation possible with external HF TIG Control unit.

### PROTECTIONS WITH AUTO RESET

- Input Supply Voltage protections for over and under Voltage

### SPECIFICATIONS

- Over Temperature

- Protection against Single phasing

| Technical Specifications |  |  | Unit | CHAMP 300 |
| --- | --- | --- | --- | --- |
| Input |  |  |  |  |
| Input Supply : |  |  |  |  |
| Input Supply Voltage, Phase, Frequency |  |  | V Ac, No., Hz | 415 , +15%, -10%, 3, 50 / 60 |
| Input KVA @ 415V Supply |  |  | KVA | 0.2 @ No Load, 10.5 @ 100% Duty Cycle, 13.5 @ 60% Duty Cycle, 17 @ 35% Duty Cycle |
| Input Current @ 415V Supply |  |  | I AC | 0.24 @ No Load, 14.5 @ 100% Duty Cycle, 18.5 @ 60% Duty Cycle, 23.5 @ 35% Duty Cycle |
| Power Factor |  |  | λ | 0.72 Max. |

Cont.

AMBALASCIENCEMARKET.COM- VIKRAM MITTAL /M-9812204116	www.modernkitchenequipments.com

26

# CHAMP 300

The world class Inverter based DC Welder

Cont.

| Efficiency |  |  | % | >82 |
| --- | --- | --- | --- | --- |
| Output |  |  |  |  |
| Open Circuit Voltage |  |  | V DC | 88 V DC + / - 5 V |
| Welding Current Range |  |  | A DC | 10-300 |
| Welding Current (40°C, 10 Minute Duty Cycle) |  |  | A DC | 300 @ 35% Duty Cycle, 250 @ 60% Duty Cycle, 200 @ 100% Duty Cycle |
| Arc Force Control |  |  | A DC | 0 - 80 A More Than Set Current |
| General |  |  |  |  |
| Suitable Welding Electrode size |  |  | mm Ø | 2.5 - 5 mm For E-6013 & E - 7018, 2.5 - 3.2 For E - 6010 |
| Current Display (Set Current & Actual Current) |  |  | A | 3 Digit –7 Segment Digital Panel Meter |
| Remote Control (optional) |  |  | 10 Meter | Provided As Optional For Current Setting. |
| Front Panel Functions |  |  |  | - MMA / Tig Mode Selection Switch. - Current & Arc Force Adjustment Pot. - Mains on ‘green’ Colour Led Indication - Trip - ‘red’ Colour LED Indication of “Machine Under Protection Mode”. - Dpm For Display Of Set Current & Actual Current |
| Protection (Auto Resettable) |  |  |  |  |
| Ingress Protection |  |  | Degree | IP23 S |
| Cooling |  |  | Type | Forced Air |
| Insulation |  |  | Class | H |
| Dimensions (L x W x H) |  |  | mm | 536 x 200 x 345 |
| Weight (approx.) |  |  | Kg | 20 |

- Warranty : One year from the date of commissioning. ADOR WELDING LIMITED warrants that all new equipment sold from Plant/Area Offices / Authorised Distributors are free from defects in materials and workmanship and will perform in full accordance with applicable specifications.

- In view of continuous development, ADOR WELDING LIMITED reserves the right to modify/change the design and /or the specifications without any prior notice.

- Backed by dedicated customer care package.

AMBALASCIENCEMARKET.COM- VIKRAM MITTAL /M-9812204116	www.modernkitchenequipments.com

27

# CHAMP T400

The world class Inverter based DC Welder

CC

DC TIG

### SALIENT FEATURES

- Three phase IGBT inverter based, high efficiency and high power factor DC Welder

- High OVC and suitable for Long distance welding and cellulosic electrodes

- Enhanced Reliability due to SMD technology

- Capable of Welding with all types of cellulosic electrodes including 6010, 7010G and 8010G l HF Unit

- High frequency IGBT based Rectifier

### PROTECTIONS WITH AUTO RESET

- Input Supply Voltage protections for over and under Voltage

- Arc force adjustment on panel

- TIG Welding possible with External HF Unit

- Light weight, compact and portable for easy handling

- Capable of Welding with 100 meter + 100 meter welding and return cables

- Smooth and stable arc with minimum spatter

- Controls provided for adjustments of Arc force

- TIG operation possible with external HF TIG Control unit

  - Over Temperature

  - Protection against Single phasing

### SPECIFICATIONS

| Technical Specifications |  |  | Unit | CHAMP T400 |
| --- | --- | --- | --- | --- |
| Input |  |  |  |  |
| Input Supply : |  |  |  |  |
| Voltage |  |  | Volt | 415 , +15%, -10% |
| Phase |  |  | No | 3 |
| Frequency |  |  | Hz | 50/ 60 |
| Input KVA @ 415 Vac |  |  |  |  |
| @ 100% duty cycle |  |  | KVA | 14 |
| @ No Load |  |  | KVA | 0.24 |
| Power Factor |  |  | CosØ | Upto 0.93 |
| Efficiency |  |  | % | “ 85 |
| Output |  |  |  |  |
| Open Circuit Voltage |  |  | Volt | 85 V DC |
| Welding Current Range |  |  | Amps | 10-400 |
| Welding Current (40°C) |  |  |  |  |

Cont.

AMBALASCIENCEMARKET.COM- VIKRAM MITTAL /M-9812204116	www.modernkitchenequipments.com

28

# CHAMP T400

The world class Inverter based DC Welder

Cont.

| @ 60% duty cycle (10 minute cycle) |  |  | Amps | 400 |
| --- | --- | --- | --- | --- |
| @ 100% duty cycle |  |  | Amps | 310 |
| General |  |  |  |  |
| Suitable Welding Electrode size |  |  | mm Ø | 2.5, 3.2, 4, 5 and 6.3 mm |
| Arc force setting |  |  | - | Adjustable by Potentiometer |
| Current Display (Set Current & Actual Current) |  |  | A | 3 digit –7 segment digital panel pcb |
| Ingress Protection |  |  |  | IP23 |
| Cooling |  |  | Class | Forced Air |
| Insulation |  |  | Type | H |
| Welding Output Terminals |  |  | Class | Stud type for Lug type cable connections |
| Dimensions L x W x H |  |  | mm | 660 X 305 X 530 |
| Weight (approx.) |  |  | Kg | 40 |

- Warranty : One year from the date of commissioning. ADOR WELDING LIMITED warrants that all new equipment sold from Plant/Area Offices / Authorised Distributors are free from defects in materials and workmanship and will perform in full accordance with applicable specifications.

- In view of continuous development, ADOR WELDING LIMITED reserves the right to modify/change the design and /or the specifications without any prior notice.

- Backed by dedicated customer care package.

AMBALASCIENCEMARKET.COM- VIKRAM MITTAL /M-9812204116	www.modernkitchenequipments.com

29

# CHAMP 500 / CHAMP 600

The world class Inverter based DC Welder

CC

DC TIG        GOUGING

### SALIENT FEATURES

  - Three phase inverter based, high efficiency and high power factor DC Welders

  - Enhanced Reliability due to SMD technology

  - High frequency IGBT based Rectifier

  - Arc force adjustment on panel

  - TIG Welding possible with External HF Unit

  - Suitable for all electrodes including 6010, 7010G and 8010G

  - Latest PWM inverter technology

  - Smooth and stable arc with minimum spatter

  - Suitable for Gouging Application and long distant welding

  - Arc forceadjustmentsprovidedonthefrontpanel

  - TIG operation possible with external HF TIG Control unit.

### PROTECTIONS WITH AUTO RESET

- Input Supply Voltage protections for over and under Voltage

### SPECIFICATIONS

- Over Temperature

- Protection against Single phasing

| Technical Specifications |  |  | Unit | CHAMP 500 | CHAMP 600 |
| --- | --- | --- | --- | --- | --- |
| Input |  |  |  |  |  |
| Input Supply : |  |  |  |  |  |
| Voltage |  |  | Volt | 415 , +15%, -10% |  |
| Phase |  |  | No | 3 |  |
| Frequency |  |  | Hz | 50/ 60 |  |
| Input KVA @ 415 Vac |  |  |  |  |  |
| @ 100% duty cycle |  |  | KVA | 17.5 | 22.0 |
| @ No Load |  |  | KVA | 0.27 | 0.31 |
| Power Factor |  |  | CosØ | 0.93 |  |
| Efficiency |  |  | % | 87 |  |
| Output |  |  |  |  |  |
| Open Circuit Voltage |  |  | Volt | 89 |  |
| Welding Current Range |  |  | Amps | 20-500 | 20-600 |
| Welding Current (40°C 10 minute cycle) |  |  |  |  |  |
| @ 60% duty cycle |  |  | Amps | 500 | 600 |
| @ 100% duty cycle |  |  | Amps | 387 | 465 |

Cont.

AMBALASCIENCEMARKET.COM- VIKRAM MITTAL /M-9812204116	www.modernkitchenequipments.com

30

# CHAMP 500 / CHAMP 600

The world class Inverter based DC Welder

Cont.

| General |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Suitable Welding Electrode size |  |  | mm Ø | 2.5, 3.2, 4, 5 and 6.3 mm |  |
| Suitable for Gouging Electrode Size |  |  | mm Ø | Max. up to 9mm | Max. up to 9 mm 12mm –limited use |
| Arc force setting |  |  |  | Adjustable by Potentiometer |  |
| Current Display (Actual) |  |  |  | 4 digit –7 segment digital panel pcb |  |
| Ingress Protection |  |  | Class | IP23 |  |
| Cooling |  |  | Type | Forced Air |  |
| Insulation |  |  | Class | H |  |
| Welding Output Terminals |  |  |  | Stud type for Lug type cable connections |  |
| Dimensions |  |  |  |  |  |
| Length |  |  | mm | 700 |  |
| Width |  |  | mm | 460 |  |
| Height (with handle) |  |  | mm | 575 |  |
| Height (without handle) |  |  | mm | 630 |  |
| Weight (approx.) |  |  | Kg | 57 | 58 |

- Warranty : One year from the date of commissioning. ADOR WELDING LIMITED warrants that all new equipment sold from Plant/Area Offices / Authorised Distributors are free from defects in materials and workmanship and will perform in full accordance with applicable specifications.

- In view of continuous development, ADOR WELDING LIMITED reserves the right to modify/change the design and /or the specifications without any prior notice.

- Backed by dedicated customer care package.

AMBALASCIENCEMARKET.COM- VIKRAM MITTAL /M-9812204116	www.modernkitchenequipments.com

31

# CHAMP 1200 - GOUGEMASTER

The world class Inverter based DC Welder with High Current Gouging Facility

CC

DC TIG       GOUGING

### GENERAL

- Three phase inverter based, high efficiency and high power factor DC Welder

- Suitable for normal Electrode Welding as well as gouging at high currents

- Enhanced Reliability due to SMD technology

### SPECIFICATIONS

PROTECTIONS WITH AUTO RESET

- Input Supply Voltage protections for over and under Voltage

- Over Temperature

- Protection against Single phasing

| Technical Specifications | Unit | Champ 1200 |
| --- | --- | --- |
| Input |  |  |
| Input Supply : |  |  |
| Voltage | Volt AC | 415 , +15%, -10% |
| Phase | No | 3 |
| Frequency | Hz | 50/ 60 |
| Max Input KVA @ @ 100% duty cycle | KVA | 55 |
| Input Supply Current @ 100% duty cycle (1000 Amps) | Amps, AC | 76 |
| Input Supply Current @ 60% duty cycle (1200 Amps) | Amps, AC | 92 |
| Power Factor @ 100% duty cycle |  | 0.93 |
| Efficiency @ 100% duty cycle | % | “ 85 |
| Output |  |  |
| Open Circuit Voltage | Volt,DC | 90 |
| Welding Current Range | Amps,DC | 100-1200 |
| Welding Current (40°C 10 minute cycle) |  |  |
| @ 60% duty cycle | Amps,DC | 1200 |
| @ 100% duty cycle | Amps,DC | 1000 |
| General |  |  |
| Suitable Welding Electrode size | mm Ø | 3.2, 4, 5 and 6.3 mm |
| Suitable for Gouging Electrode Size | mm Ø | Up to 12 mm |
| Remote Control (Optional) |  | 10 meter Remote Control provided as optional for current setting |
| Ingress Protection | Class | IP23 |
| Cooling | Type | Forced Air |
| Insulation | Class | H |

Cont.

AMBALASCIENCEMARKET.COM- VIKRAM MITTAL /M-9812204116	www.modernkitchenequipments.com

32

# CHAMP 1200 - GOUGEMASTER

The world class Inverter based DC Welder with High Current Gouging Facility

Cont.

| Welding Output Terminals |  | Stud type for Lug type cable connections |
| --- | --- | --- |
| Dimensions |  |  |
| Length | mm | 930 |
| Width | mm | 525 |
| Height (with handle) | mm | 950 |
| Weight (approx.) | Kg | 115 |
| Front Panel Functions |  | 'MAINS ON' - Green LED Indication |
|  |  | 'MMA/GOUGING'-Red LED Indication |
|  |  | 'REMOTE ON' - Red LED Indication |
|  |  | 'TRIP' - Red LED Indication if Machine is under Protection Mode |
|  |  | Current Adjustment Encoder |
|  |  | 4 Digit Digital Display for Current and Voltage |
| Protections (Auto - Reset) |  | Over Voltage, Under Voltage, Single Phasing |
|  |  | Over Temperature |

- Warranty : One year from the date of commissioning. ADOR WELDING LIMITED warrants that all new equipment sold from Plant/Area Offices / Authorised Distributors are free from defects in materials and workmanship and will perform in full accordance with applicable specifications.

- In view of continuous development, ADOR WELDING LIMITED reserves the right to modify/change the design and /or the specifications without any prior notice.

- Backed by dedicated customer care package.

AMBALASCIENCEMARKET.COM- VIKRAM MITTAL /M-9812204116	www.modernkitchenequipments.com

33

# CHAMPCUT 100

The world class Inverter based Air Plasma Cutting Outfit

3 CC

TYPICAL APPLICATION

- Three	phase inverter based, high efficiency cutting outfit

- Ideal for Heavy duty production jobs

- Suitable for manual cutting as well as with CNC machine

- High frequency IGBT based PWM Inverter

- Enhanced reliability due to SMD technology

- Seven Segment LED display for set and actual cutting current for precise control of cutting current.

- Air pressure indicating meter is provided on front panel

- Emergency stop push button stops the cutting operation instantly in case of emergency.

### SALIENT FEATURES

- Latest inverter based technology

- Emergency stop push button is provided on front panel

- Micro-controller Based control provides precise control on sequence of operation and its interlocking.

- Capability to interface with CNC controllers

- Post flow cooling provides torch cooling after the cut for longer torch life

- Safety device signal provided on front panel for

- Low air pressure

- Thermal failure

- Mains voltage higher or lower than limit..

### PROTECTIONS WITH AUTO RESET

- Under-voltage /

Over-voltage protection.

### SPECIFICATIONS

- Thermal protection.

- Low air pressure

| Technical Specifications |  |  | Unit | CHAMPCUT 100 |
| --- | --- | --- | --- | --- |
| Input |  |  |  |  |
| Input Supply : |  |  |  |  |
| Voltage |  |  | Volts AC | 370 -470 |
| Phase |  |  | No | 3 |
| Frequency |  |  | Hz | 50/60 |
| Max input KVA @ 100% duty cycle |  |  | KVA | 15.4 |
| Current @ 100% Duty Cycle |  |  | Amps AC | 23 |

Cont.

AMBALASCIENCEMARKET.COM- VIKRAM MITTAL /M-9812204116	www.modernkitchenequipments.com

34

# CHAMPCUT 100

The world class Inverter based Air Plasma Cutting Outfit

Cont.

| Output |  |  |  |  |
| --- | --- | --- | --- | --- |
| Pilot ARC Current |  |  | Amps DC | “ 20 |
| Cutting Current (40°C) @100% Duty cycle |  |  | Amps DC | 100 |
| Open Current Voltage @415VAC Input Supply |  |  | Volts DC | 320 +/- 10V |
| Efficiency @ 100% duty cycle |  |  | % | >88 |
| Cutting Current Range |  |  | Amps DC | 20 - 100 |
| Maximum Cutting Thickness (Severance) |  |  | mm | 35 |
| Optimum Cutting Thickness |  |  | mm | 25 |
| Machine Used Protect Cutting Thickness |  |  | mm | “ 20 |
| General |  |  |  |  |
| Front Panel Functions |  |  |  | - Cutting Current Adjustment Pot - Mains LED, Green - Trip LED, Red - Cutting LED Yellow - Switch to Select 2T/4T Functions - Switch for Air Check 70 LED Display for Displaying The Cutting Range |
| Protection (Auto Resettable) |  |  |  | Thermal, UV,-OV, Low Pressure, Emergency Stop Switch |
| Cooling |  |  | Type | Forced Air |
| Class of Insulation |  |  |  | H |
| Degree of Protection |  |  |  | IP21 |
| Air Pressure |  |  | MPA/BAR | 0.45-0.55 / 4.5-5.5 |
| Dimensions (L x W x H) |  |  | mm | 620 x 300 x 526 |
| Weight (approx.) |  |  | Kg | 43 |

- Warranty : One year from the date of commissioning. ADOR WELDING LIMITED warrants that all new equipment sold from Plant/Area Offices / Authorised Distributors are free from defects in materials and workmanship and will perform in full accordance with applicable specifications.

- In view of continuous development, ADOR WELDING LIMITED reserves the right to modify/change the design and /or the specifications without any prior notice.

- Backed by dedicated customer care package.

AMBALASCIENCEMARKET.COM- VIKRAM MITTAL /M-9812204116	www.modernkitchenequipments.com

35

# CHAMPCUT 25C

The world class inverter based Plasma Cutting Machine

3 CC

TYPICAL APPLICATION

- Three phase inverter based, soft switching technology high efficiency and high cutting capacity Plasma Cutting Machine

- Suitable for both manual and mechanized cutting operation with suitable Cutting Torch

- High Frequency arc striking for ease of operation

- Digital display for precise setting of cutting current

- Time delay function provides effective protection to cutting torch

- Self Hold ON / OFF setting feature makes themachine typically suitable for use with Automatic CNC Cutting outfit

### SPECIFICATIONS

| Technical Specifications | Unit | CHAMPCUT 25C |
| --- | --- | --- |
| Characteristic Control |  | IGBT Inverter Based |
| Characteristics | Type | Constant Current |
| Input |  |  |
| Input Supply Voltage | Volts AC | 380 - 415 |
| Phase |  | 3 |
| Frequency | No. | 50/60 |
| Rated Input Power | Hz | 14.5 |
| Output |  |  |
| Cutting Current Range | Amps, DC | 30-100 |
| Cutting Current @100% Duty Cycle | Amps, DC | 100 |
| General |  |  |
| Maximum Severing Capacity | mm | 40 |
| Optimal Cutting Capacity | mm | 1-20 |
| Maximum Machine (Automatic) Cutting | mm | “ 15 |
| Nominal Air Pressure | Mpa/Bar | 0.45 / 4.5 |
| Dimensions (L X W X H) | mm | 550 / 300 / 500 |
| Weight (Approx) | kg | 40 |

- Warranty : One year from the date of commissioning. ADOR WELDING LIMITED warrants that all new equipment sold from Plant/Area Offices / Authorised Distributors are free from defects in materials and workmanship and will perform in full accordance with applicable specifications.

- In view of continuous development, ADOR WELDING LIMITED reserves the right to modify/change the design and /or the specifications without any prior notice.

- Backed by dedicated customer care package.

AMBALASCIENCEMARKET.COM- VIKRAM MITTAL /M-9812204116	www.modernkitchenequipments.com

36

# CHAMPCUT 150

The world class Inverter based Air Plasma Cutting Outfit

3 CC

TYPICAL APPLICATION

•		Three phase inverter based, high efficiency cutting outfit

•	Ideal for Heavy duty production jobs

•	Suitable for manual cutting as well as with CNC machine

•	High frequency IGBT based PWM Inverter

•	Enhanced reliability due to SMD technology

• Seven Segment LED display for set and actual cutting current for precise control of cutting current.

• Air pressure indicating meter is provided on front panel

• Emergency stop push button stops the cutting operation instantly in case of emergency.

### SALIENT FEATURES

• Latest inverter based technology

• Emergency stop push button is provided on front panel

• Micro-controller Based control provides precise control on sequence of operation and its interlocking.

• Capability to interface with CNC controllers

• Post flow cooling provides torch cooling after the cut for longer torch life

• Safety device signal provided on front panel for

- Low air pressure

- Thermal failure

- Mains voltage higher or lower than limit..

### PROTECTIONS WITH AUTO RESET

• Under-voltage / Over-voltage protection.

### SPECIFICATIONS

• Thermal protection.

• Low air pressure

| Technical Specifications - Plasma Cutting Power Source |  |  | Unit | CHAMPCUT 150 |
| --- | --- | --- | --- | --- |
| Input |  |  |  |  |
| Input Supply : |  |  |  |  |
| Voltage |  |  | Volts AC | 415 + / -10% |
| Phase |  |  | No | 3 |
| Frequency |  |  | Hz | 50/60 |
| Max input KVA @ 100% duty cycle |  |  | KVA | 26.4 |
| Current @ 100% Duty Cycle |  |  | Amps AC | 38 |

Cont.

AMBALASCIENCEMARKET.COM- VIKRAM MITTAL /M-9812204116	www.modernkitchenequipments.com

37

# CHAMPCUT 150

The world class Inverter based Air Plasma Cutting Outfit

Cont.

| Output |  |  |  |  |
| --- | --- | --- | --- | --- |
| Open Current Voltage @415VAC Input Supply |  |  | Volts DC | 330 +/- 10V |
| Cutting Current Range |  |  | Amps DC | 30 - 50 |
| Cutting Current (40°C) @100% Duty cycle |  |  | Amps DC | 150 |
| Recommended Cutting With Rage (With CNG) |  |  | mm | 5 -35 |
| Maximum Cutting Capacity (Severance) For as MS Material |  |  | mm | 50 |
| Maximum Piercing Capacity |  |  | mm | 20 |
| Machine Used Protect Cutting Capacity (For M.S.) |  |  | mm | “ 25 |
| General |  |  |  |  |
| Type of End Connector for Plasma Cutting Torch |  |  |  | Euro |
| Type of Cooling |  |  | Type | Forced Air |
| Class of Insulation |  |  |  | H |
| Ingress Protection |  |  |  | IP23S |
| Air Pressure |  |  | MPA/BAR | 0.45-0.55 / 4.5-5.5 |
| Weight (approx.) |  |  | Kg | 60 |
| Plasma Cutting Power Source |  |  | Unit | CCT 150 |
| Voltage Glass |  |  |  | M |
| Standard Length |  |  | Mtr | 6 |
| Air Pressure |  |  | BAR | 5 |
| Air Consumption |  |  | I/Min | 230 |
| Duty Cycle 60% |  |  | A | 150 |
| Duty Cycle 100% |  |  | A | 150 |

• Warranty : One year from the date of commissioning. ADOR WELDING LIMITED warrants that all new equipment sold from Plant/Area Offices / Authorised Distributors are free from defects in materials and workmanship and will perform in full accordance with applicable specifications.

• In view of continuous development, ADOR WELDING LIMITED reserves the right to modify/change the design and /or the specifications without any prior notice.

• Backed by dedicated customer care package.

AMBALASCIENCEMARKET.COM- VIKRAM MITTAL /M-9812204116	www.modernkitchenequipments.com

38

# CUTi 35, 70, 90, 120

The world Class Inverter based Plasma Cutting Machines

SALIENT FEATURES

• The small power packages of the CUTi series are easy to handle and thus especially suitable for mobile use. From among the models of the series, the user may choose the unit which meets his individual requirements best.

• Using compressed Air as the Plasma Gas, mild steel, stainless steel, aluminium, brass, copper and other electrically conductive materials can be cut.

• All CUTi inverters operate with gas-cooled plasma torches and external compressed air supply.

• Due to the sinusoidal power consumption with PFC (power factor correction), the inverter CUTi 35 draws its maximum power from the single-phase 230 V mains, without major reactive power loss.

• Light, portable, easy to handle

• Contains inlet FRL Unit to ensure flow of clean, dry air

• Productive due to high cutting speed

• Energy saving due to modern inverter technique

• High cut quality

• Versatile use due to a large variety of accessories

• Safe working due to safety shut-down

### DELIVERY CONTENTS OF OUTFIT:

• Plasma Power Source with input cable and power supply plug

• Plasma Cutting Torch (Air cooled) with cutting accessories

• Return cable with Earth clamp

• Air supply hose

### APPLICATION AREAS

• Convenient Hand Torches

• Plasma gouging does not require after-treatment and produces less smoke compared to gouging with carbon electrodes.

• Suitable for industry and craft

• In workshops and training centres

• For repairing and servicing

• At assembly workplaces, on construction sites

• Straight, profile and contour cutting, also with templates

• For piercing and hole cutting

•  Bevel cutting at any angle for weld preparation, possible with appropriate accessories

• Plasma gouging in preparation of weld joints, fettling, removal of welding mistakes and surface defects with CUTi 90 and CUTi 120

AMBALASCIENCEMARKET.COM- VIKRAM MITTAL /M-9812204116	www.modernkitchenequipments.com

39

# CUTi 35, 70, 90, 120

The world Class Inverter based Plasma Cutting Machines

ERGONOMIC HANDLE DESIGN

• Thanks to the ergonomic handle design and the low weight of the KjellCut plasma torches work is easy. In addition to the convenient operation, safety is also of prime importance here.

### TECHNICAL SPECIFICATIONS

ACCESSORIES FOR CUTi

• A large variety of accessories is available for the flexible use of the CUTi units.

| Parameter | UNIT | CUTi 35 | CUTi 70 | CUTi 90 | CUTi 120 |
| --- | --- | --- | --- | --- | --- |
| Input |  |  |  |  |  |
| Mains Voltage | VOLTS, AC | 1 X 230 V | 3 X 400 V | 3 X 400 V | 3 X 400 V |
| Fuse, Slow | AMPS, AC | 16 A | 16 A | 25 A | 32 A |
| Connected Load, Max. | KVA | 3.7 KVA | 11.1 KVA | 15 KVA | 16.7 KVA |
| Output |  |  |  |  |  |
| Cutting Current Range | AMPS, DC | 5-35 A | 26 -70 A | 26 -90 A | 25 -120 A |
| Duty Cycle | 10 min duty cycle | 35 A @ 40% | 70 A @ 35% | 90 A @ 40% | 120 A @ 35% |
|  |  | 28 A @ 60% | 60 A @ 60% | 74 A @ 60% | 95 A @ 60% |
|  |  | 22 A @ 100 % | 50 A @ 100 % | 55 A @ 100 % | 80 A @ 100 % |
| General |  |  |  |  |  |
| Protection Class | CLASS | IP 21 | IP 21 | IP 21 | IP 21 |
| Cutting Range | MM | 12 MM | 30 MM | 35 MM | 50 MM |
| Ignition | TYPE | DRAWN ARC | HIGH VOLTAG | HIGH VOLTAGE | HIGH VOLTAGE |
| Plasma Gas | TYPE | AIR | AIR | AIR | AIR |
| Pressure | MPA | 0.4 MPA | 0.5 MPA | 0.5 MPA | 0.5 MPA |
| Air Consumption | L / MIN | 70 L / MIN | 140 L / MIN | 195 L / MIN | 195 L / MIN |
| Dimensions (L X W X H) | MM | 480 X 150 X 225 MM | 470 X 180 X 250 MM | 470 X 180 X 270 MM | 610 X 230 X 410 MM |
| Weight | KG | 10 KG | 16.4 KG | 17 KG | 28.5 KG |

• Warranty : One year from the date of commissioning. ADOR WELDING LIMITED warrants that all new equipment sold from Plant/Area Offices / Authorised Distributors are free from defects in materials and workmanship and will perform in full accordance with applicable specifications.

• In view of continuous development, ADOR WELDING LIMITED reserves the right to modify/change the design and /or the specifications without any prior notice.

• Backed by dedicated customer care package.

AMBALASCIENCEMARKET.COM- VIKRAM MITTAL /M-9812204116	www.modernkitchenequipments.com

40

# PLASMACUT - 25

Air plasma cutting system with unmatched cutting speed

SALIENT FEATURES

• Most dependable and reliable power source

• Distortion-free, dross-free, burr-free, narrow kerf width clean cuts

• Suitable for cutting all conducting metals

• Simple and easy to use

### PLASMACUT - 25

for all your Cutting Needs Wheather it’s clean cuts that you want or seperation cuts

PLASMACUT - 25 works with equal ease.

### TYPICAL APPLICATION

•	Automotive Repairs

•	General Fabrication

•	Bus and coach Body Building

•	Railway and Road Transport Workshops

•		Power Stations, Chemical, Petrochemicals or Fertilizer Industry

•	Sheet Metal Fabrication

•	Construction and Maintenance Workshops

•		Manufacture of Food Processing and Pharmaceutical equipment using SS

•	Scrap Cutting Demolition, ship-breaking etc.

### MACHINE FEATURES

•	Rugged and robust Design

•	User Friendly, requires minimum operator training

•	Special “Safety-interlock”for operator protection

•	Railway and Road Transport Workshops

•		Lightweight and Flexible Torch with 8 m long hose assemble

•	Air presssure safety switch for “fail-safe cutting”

• Double air fliter-cum-pressure regular unit to ensure clean compressed air for excellent cut quality

• Easy manoeuvrability on the shopfloor

• Full range of accessories

AMBALASCIENCEMARKET.COM- VIKRAM MITTAL /M-9812204116	www.modernkitchenequipments.com

41

# PLASMACUT - 25

Air plasma cutting system with unmatched cutting speed

SPECIFICATIONS

| Technical Specifications - |  |  | Unit | PLASMACUT 150 |
| --- | --- | --- | --- | --- |
| Input |  |  |  |  |
| Input Supply : |  |  |  |  |
| Voltage |  |  | Volts AC | 415 |
| Phase |  |  | No | 3 |
| Frequency |  |  | Hz | 50 |
| Max input KVA @ 100% duty cycle |  |  | KVA | 28 |
| Recommended switch fuse rating |  |  | Amps AC | TP-45 |
| Output |  |  |  |  |
| Cutting Current |  |  | Amps | 100 |
| Open Current Voltage |  |  | DC Volts | 300 |
| Cutting Current @60% Duty cycle |  |  | Amps | 100 |
| Pilot arc open circuit voltage |  |  | DC Volts | 300 |
| General |  |  |  |  |
| Insulation |  |  | Class | ‘B’ |
| Type of Cooling |  |  | Type | Forced Air |
| Overall Dimensions |  |  |  |  |
| Length |  |  | mm | 800 |
| Width |  |  | mm | 370 |
| Height |  |  | mm | 825 |
| Weight (approx.) |  |  | Kg | 165 |
| Torch |  |  | Model : CCT-25 (A) |  |
| Max Current / Voltage @ 60% duty cycle |  |  | Amps/ Volts | 100/110 |
| Cooling |  |  | Type | Air |
| Air to be used (Plasma+Cooling) |  |  |  |  |
| Flow |  |  | Lit/Min | 120 to 150 |
| Pressure |  |  | kg/cm2 | 6 |
| Cable Hose Length |  |  | Mtrs. | 6 |

• Warranty : One year from the date of commissioning. ADOR WELDING LIMITED warrants that all new equipment sold from Plant/Area Offices / Authorised Distributors are free from defects in materials and workmanship and will perform in full accordance with applicable specifications.

• In view of continuous development, ADOR WELDING LIMITED reserves the right to modify/change the design and /or the specifications without any prior notice.

• Backed by dedicated customer care package.

AMBALASCIENCEMARKET.COM- VIKRAM MITTAL /M-9812204116	www.modernkitchenequipments.com

42

# SUPERGEN 320

Performance Proven Motor Generator Welding Set

Choice of the finest equipment for superior weld quality and efficiency

CC

DC TIG

### SALIENT FEATURES

• The set consists of three-phase motor as prime mover and DC welding generator of a special patented design

• Excellent high quality welding with big savings in power bills

• Positive protection against overload and single phasing

•	Mobile and mounted on rubber-tyred wheels

•		Ideal for welding with cellulosic electrodes for cross country pipelines and thermal/nuclear power plants.

### QUANTUM LEAP IN WELDING PRODUCTION FEATURE-WISE COMPARISON

| Features you want | Brand X | Our new technology break through |
| --- | --- | --- |
| Welding with 100 mtr cable length without any current drop | Current drops | Constant current even with longer cable length |
| Low no load power consumption | 3.2 KW | 1.1 KW - less than half power consumption and energy cost |
| Low power consumption at | 22.23 KW 320 Amp.welding current | 15.5 KW Reduction by 30% in energy bill Saving upto Rs. 46,000 in a year |
| Low maintenance cost | Periodic wear’n tear of brushes | Brushless’ technology eliminates frequent Maintenance and down time costs |
| Freedom from stator failures | No protection | Built-in protection against single phasing, over voltage andunder voltage |
| Insulation class | “B” | “H"- provides longer service life- more reliable |
| Suitable for welding in open/ dusty sites | S.P.D.P.* Open type design - sucks in dirt and dust | Totally enclosed preventing entry of dirt and dust insidel winding - ideal for site conditions |
| Ease of mobility | 330 kgs in weight | Only 265 kgs in weight i.e.25% lighter-easier to move about |
| Availability of Spares & service | Spares subject to imports- Limited service network | Easy availability of spares Large network of authorised service centres. |

Cont.

AMBALASCIENCEMARKET.COM- VIKRAM MITTAL /M-9812204116	www.modernkitchenequipments.com

43

# SUPERGEN 320

Performance Proven Motor Generator Welding Set

Choice of the finest equipment for superior weld quality and efficiency

SPECIFICATIONS

Cont.

CC

DC TIG

| Technical Specifications |  |  | Unit | Value |
| --- | --- | --- | --- | --- |
| Input |  |  |  |  |
| Input Supply : |  |  |  |  |
| Voltage |  |  | Volt | 415 |
| Phase |  |  | No | 3 |
| Frequency |  |  | Hz | 50 |
| Power |  |  | Kw/HP | 15.5/20.78 |
| Speed (Synchronous) |  |  | RPM | 3000 |
| Starting |  |  | - | Star / Delta starter |
| Recommended switch fuse rating |  |  | - | TP40 |
| Protection to Machine |  |  | - | Machine provided with built in Single Phasing, Under voltage (340V), Over voltage (480 V) and thermal protection |
| Output |  |  |  |  |
| Open Circuit Voltage |  |  | Volt | 100 |
| Welding Current Range |  |  | Amps | 35-320 |
| Welding Current |  |  |  |  |
| @ 60% duty cycle |  |  | Amps | 320 |
| @ 100% duty cycle |  |  | Amps | 250 |
| Welding Electrode Range |  |  | mm | 2.5-6.3 |
| General |  |  |  |  |
| Insulation |  |  | Class | H |
| Cooling |  |  | Type | Fan Cooled |
| Enclosure |  |  | Type | Totally enclosed |
| Protection |  |  | Class | IP44 (excluding Fan side) |
| Dimensions L X W X H |  |  | mm | 1065 x 540 x 840 |
| Weight (approx) |  |  | Kg | 265 |

• Warranty : One year from the date of commissioning. ADOR WELDING LIMITED warrants that all new equipment sold from Plant/Area Offices / Authorised Distributors are free from defects in materials and workmanship and will perform in full accordance with applicable specifications.

• In view of continuous development, ADOR WELDING LIMITED reserves the right to modify/change the design and /or the specifications without any prior notice.

• Backed by dedicated customer care package.

AMBALASCIENCEMARKET.COM- VIKRAM MITTAL /M-9812204116	www.modernkitchenequipments.com

44

# SILENT CHALLENGER 301 S2

New Generation Diesel Engine Driven Silent Welding Set Conforming to Latest CPCB II Norms

CC

TIG

### SALIENT FEATURES

• Conforms to latest CPCB norms for noise & exhaust emission level

• Versatile applications, including cross country, inplant pipe and tube welding

• Ideal for heavy fabrication & site applications

• Highly reliable even in hostile site conditions

• Big savings in fuel & longer runs before next refuelling

• Specially proven with Cellulosic (6010, 7010G & 8010G types) and other special types of electrodes

• Excellent Pre and Post Sales Services

• Super Silent Operations

### SPECIFICATIONS

| Technical Specifications | Unit | Value |
| --- | --- | --- |
| Machine Name |  | SC 301 S2 |
| CPCB compliance | Type | Meets CPCB - II compliance for Exhaust Emission and Noise within 75 dba at 1 mtr |
| Welding Generator |  |  |
| Welding Generator | Type | Brushless type, high frequency generator, specially designd for welding applications, constant current (CC) characteristic with winding less, commutator less, magnet less, slip-ring less and rotating diode less rotor construction for maintenance free and reliable operation |
| Welding process | Type | SMAW (Shielded Metal Arc Welding ) |
| Operators | No | Single |
| Welding current range | Amps | 40 - 300 A |
| Max. hand welding current | Amps | 300 A @ 32 V |
| Max. hand welding current @ 60% DC | Amps | 230 A @ 29V |
| Max. hand welding current @ 100% DC | Amps | 310 |
| Open circuit voltage (Max. / Min) | Volts DC | 100 / 45 VDC |
| Generally, Conforms To | IS | 26-35 |
| Diameter of coated electrode | mm | 2.5, 3.15, 4, 5, 6 |
| Insulation Class | Type | H |
| Auxiliary Power Source (built In) |  |  |
| Rating 3 Phase / Single Phase | KVA | 10 / 3 |
| Voltages (3 Phase/1 Phase) | VOLTS | 415 / 240 |
| MCB Rating | AMPS | 16 AMPS |

AMBALASCIENCEMARKET.COM- VIKRAM MITTAL /M-9812204116	www.modernkitchenequipments.com

45

# SILENT CHALLENGER 301 S2

New Generation Diesel Engine Driven Silent Welding Set Conforming to Latest CPCB II Norms

Cont.

| Engine Make, Type |  | SIMPSON, SC213 CPCB-II COMPLIANT |
| --- | --- | --- |
| Cylinder | Nos. | 2 |
| Engine Cooling | Type | Water Cooled |
| Engine Rating | BHP | 23 BHP @ 1800 RPM |
| Speed | RPM | 1800 |
| Conforms to | Standard | 10002/81 |
| Starting (12 V) | Type | Electric |
| Fuel tank Capacity | Ltrs. | 50 |
| Meters/Gauges/Indications | Type/Detail | Lube oil pressure, Fuel level Indication, Battery charging current, Charging failure warning lamp, Engine speed, Hour Meter etc. |
| OPTIONAL Engine Safety Protection | Type | engine auto shut off in the event of low lube oil pressure (or) high cylinder head temperature |
| General (Dimensions & Weight) |  |  |
| Skid Mounted : |  |  |
| Length/Width/Height | mm | 1700 X 820 X 1130 |
| Weight (Net) | Kg | 695 |
| Two Wheel Mounted : |  |  |
| Length/Width/Height | mm | 2540 X 1455 X 1725 |
| Weight (Net) | Kg | 770 |
| Four Wheel Mounted : |  |  |
| Length/Width/Height | mm | 3200 X 1555 X 1990 |
| Weight (Net) | Kg | 845 |

• Warranty : One year from the date of commissioning. ADOR WELDING LIMITED warrants that all new equipment sold from Plant/Area Offices / Authorised Distributors are free from defects in materials and workmanship and will perform in full accordance with applicable specifications.

• In view of continuous development, ADOR WELDING LIMITED reserves the right to modify/change the design and /or the specifications without any prior notice.

• Backed by dedicated customer care package.

AMBALASCIENCEMARKET.COM- VIKRAM MITTAL /M-9812204116	www.modernkitchenequipments.com

46

# SILENT CHALLENGER 401

New Generation Diesel Engine Driven Silent Welding Set Conforming to Latest CPCB II Norms

CC

DC TIG

### SALIENT FEATURES

• Conforming to latest CPCB norms for noise & exhaust emission level

• Versatile applications, including cross country, inplant pipe and tube welding

• Ideal for heavy fabrication & site applications

• Highly reliable even in hostile site conditions

• Big savings in fuel & longer runs before next refueling

• Specially proven with Cellulosic (6010, 7010G & 8010G types) and other special types of electrodes

• Excellent Pre and Post Sales Services

• Super Silent Operations

• Capable of Arc Cutting with up to 4 mm Arc Cutting Electrode

### SPECIFICATIONS

| Technical Specifications | Unit | Value |
| --- | --- | --- |
| Machine Name |  | SC 401 |
| CPCB compliance | Type | Meets CPCB - II compliance for Exhaust Emission and Noise within 75 dba at 1 mtr |
| Welding Generator |  |  |
| Welding Generator | Type | Brushless type, high frequency generator, specially designd for welding applications, constant current (CC) characteristic with winding less, commutator less, magnet less, slipring less and rotating diode less rotor construction for maintenance free and reliable operation |
| Welding process | Type | SMAW (Shielded Metal Arc Welding ) |
| Operators | No | Single |
| Welding current range | Amps | 50 - 400 |
| Max. hand welding current | Amps | 400 |
| Max. hand welding current @ 60% DC | Amps | 400 |
| Max. hand welding current @ 100% DC | Amps | 310 |
| Open circuit voltage (Max. / Min) | Volts | 100/45 |
| Diameter of coated electrode | mm | 2.5-6.3 |
| Insulation Class | Type | H |
| Engine Make, Type |  | SIMPSON, SJ327T CPCB-II Compliant |
| Cylinder | No. | 3 |
| Rated engine Speed | RPM | 1800 |
| Engine Cooling | Type | Water Cooled |
| Engine Rating | BHP | 38.5 BHP @ 1800 RPM |
| Conforms to | Standard | IS 10002(BI) |
| Starting | Type | Electric (Battery 12 V, 75 AH) |

AMBALASCIENCEMARKET.COM- VIKRAM MITTAL /M-9812204116	www.modernkitchenequipments.com

47

# SILENT CHALLENGER 401

New Generation Diesel Engine Driven Silent Welding Set Conforming to Latest CPCB II Norms

Cont.

| Fuel Consumption @ 60% Duty Cycle | Ltrs./Hrs. | 4.0 ltrs/hr @ 100% Duty Cycle |  |
| --- | --- | --- | --- |
| Fuel tank Capacity | Ltrs. | 70 |  |
| Meters/Gauges/Indications | Type/Detail | Lube oil pressure, Fuel level Indication, Battery charging current, Charging failure warning lamp, Engine speed, Hour Meter etc. |  |
| OPTIONAL Engine Safety Protection | Type | Engine Auto Shut off in the event of low lube oil presssure |  |
| Arc Cutting |  | Arc Cutting with upto 4 mm Cutting Electrode |  |
| Auxiliary Power Source (Indicates rating with simultaneous welding load) |  |  |  |
| Rating | KVA | 10 | 3 |
| Voltage | Volts | 415 | 240 |
| Phases | No | 3 | 1 |
| MCCB Rating | Amps | 16 |  |
| Output Sockets | No | 2 |  |
| General (Dimensions & Weight) |  |  |  |
| Skid Mounted : |  |  |  |
| Length/Width/Height | mm | 2000 x 820 x 1460 |  |
| Weight (Net) | Kg | 1100 |  |
| Two Wheel Mounted : |  |  |  |
| Length/Width/Height | mm | 2900 x 1455 x 1990 |  |
| Weight (Net) | Kg | 1240 |  |
| Four Wheel Mounted : |  |  |  |
| Length/Width/Height | mm | 2900 x 1555 x 1990 |  |
| Weight (Net) | Kg | 1340 |  |

• Warranty : One year from the date of commissioning. ADOR WELDING LIMITED warrants that all new equipment sold from Plant/Area Offices / Authorised Distributors are free from defects in materials and workmanship and will perform in full accordance with applicable specifications.

• In view of continuous development, ADOR WELDING LIMITED reserves the right to modify/change the design and /or the specifications without any prior notice.

• Backed by dedicated customer care package.

AMBALASCIENCEMARKET.COM- VIKRAM MITTAL /M-9812204116	www.modernkitchenequipments.com

48

# SILENT CHALLENGER 501

New Generation Diesel Engine Driven Silent Welding Set Conforming to Latest CPCB II Norms

CC

DC TIG

### SALIENT FEATURES

•   Conforming to latest CPCB norms for noise	& exhaust emission level

• Versatile applications, including cross country, inplant pipe and tube welding

• Big savings in fuel and longer runs before next refueling

• Arc Cutting and Gouging capability. Maximum size of Gouging Electrode is 6 mm

### SPECIFICATIONS

• Specially proven with Cellulosic (6010, 7010G & 8010G types) and other special types of electrodes

• Excellent Pre and Post Sales Services

• Ideal for heavy fabrication & site applications

• Highly reliable even in hostile site conditions

• Super Silent Operations

| Technical Specifications | Unit | Value |
| --- | --- | --- |
| Machine Name |  | SC 501 |
| CPCB compliance | Type | Meets CPCB - II compliance for Exhaust Emission and Noise within 75 dba at 1 mtr |
| Welding Generator |  |  |
| Welding Generator | Type | Brushless type, high frequency generator, specially designd for welding applications, constant current (CC) characteristic with winding less, commutator less, magnet less, slipring less and rotating diode less rotor construction for maintenance free and reliable opertaion |
| Welding process | Type | SMAW (Shielded Metal Arc Welding ) |
| Operators | No | Single |
| Welding current range | Amps | 50 - 500 |
| Max. hand welding current | Amps | 500 |
| Max. hand welding current @ 60% DC | Amps | 500 |
| Max. hand welding current @ 100% DC | Amps | 400 |
| Open circuit voltage (Max. / Min) | Volts | 100/45 |
| Diameter of coated electrode | mm | 2.5-6.3 |
| Insulation Class | Type | H |
| Engine Make, Type |  | SIMPSON, SJ327T CPCB-II Compliant |
| Cylinder | No. | 3 |
| Rated engine Speed | RPM | 1800 |
| Engine cooling | Type | Water Cooled |
| Engine Rating | BHP | 38.5 @ 1800 RPM |
| Conforms to | Standard | IS 10002(BI) |
| Starting | Type | Electric (Battery 12 V, 75 AH) |

Cont.

AMBALASCIENCEMARKET.COM- VIKRAM MITTAL /M-9812204116	www.modernkitchenequipments.com

49

# SILENT CHALLENGER 501

New Generation Diesel Engine Driven Silent Welding Set Conforming to Latest CPCB II Norms

Cont.

| Fuel Consumption @ 60% Duty Cycle | Ltrs./Hrs. | 4.0 ltrs/hr @ 100% Duty Cycle |  |
| --- | --- | --- | --- |
| Fuel tank Capacity | Ltrs. | 70 |  |
| Meters/Gauges/Indications | Type/Detail | Lube oil pressure, Fuel level Indication, Battery charging current, Charging failure warning lamp, Engine speed, Hour Meter etc. |  |
| OPTIONAL Engine Safety Protection | Type | Engine Auto shut of in the event of low lube oil pressure or High Cylinder Head Temperature |  |
| Gouging and Arc Cutting |  | Capable of Gouging with upto 6 mm Gouging Electrode |  |
| Auxiliary Power Source (Indicates rating with simultaneous welding load |  |  |  |
| Rating | KVA | 10 | 3 |
| Voltage | Volts | 415 | 240 |
| Phases | Nos | 3 | 1 |
| MCCB Rating | Amps | 16 |  |
| Output Sockets | Nos | 2 |  |
| General (Dimensions & Weight) |  |  |  |
| Skid Mounted : |  |  |  |
| Length/Width/Height | mm | 1995 X 820 X 1520 |  |
| Weight (Net) | Kg | 1100 |  |
| Two Wheel Mounted : |  |  |  |
| Length/Width/Height | mm | 2900 x 1455 x 1990 |  |
| Weight (Net) | Kg | 1240 |  |
| Four Wheel Mounted : |  |  |  |
| Length/Width/Height | mm | 3435 x 1555 x 1990 |  |
| Weight (Net) | Kg | 1340 |  |

• Warranty : One year from the date of commissioning. ADOR WELDING LIMITED warrants that all new equipment sold from Plant/Area Offices / Authorised Distributors are free from defects in materials and workmanship and will perform in full accordance with applicable specifications.

• In view of continuous development, ADOR WELDING LIMITED reserves the right to modify/change the design and /or the specifications without any prior notice.

• Backed by dedicated customer care package.

AMBALASCIENCEMARKET.COM- VIKRAM MITTAL /M-9812204116	www.modernkitchenequipments.com

50

# SILENT CHALLENGER 2 X 301

The New generation Diesel Engine Driven Silent Welding Set Conforming To Latest CPCB II norms

CC

DC TIG

### SALIENT FEATURES

• Conforms to latest CPCB norms for noise & exhaust emission level

• Versatile applications, including cross country, in- plant pipe and tube welding

•  Ideal for heavy fabrication & site applications

• Highly reliable even in hostile site conditions

• Big savings in fuel and longer runs before next re- fueling

• Specially proven with Cellulosic (6010, 7010G & 8010G types) and other special types of electrodes

• Excellent Pre and Post Sales Services

• Super Silent Operations

### SPECIFICATIONS

| Technical Specifications | Unit | Values |  |
| --- | --- | --- | --- |
| Welding Generator | Type | Double Operator | Single Operator |
|  |  | Manual (CC) | Manual (CC) |
| Welding Current Range | Amps, DC | 15x300 A | 30-600 Amps |
| Max. Hand Welding Current @ 40% DC | Amps, DC | 2x300 A @29V | 600 Amps @30V |
| Max. Hand Welding Current @ 60% DC | Amps, DC | 2x250 A @26.5V | 500 Amps @30V |
| Max. Hand Welding Current @ 100% DC | Amps, DC | 2x200 A @24V | 400 Amps@36V |
| Open Circuit Voltage (Max) | Volts, DC | 92 VDC | 92 VDC |
| Generally Conforms To | IS | 26-35 |  |
| Insulation | Type | H |  |
| Engine Make, Type |  | SIMPSON, SJ327T CPCB-II Compliant |  |
| Cylinder | Nos. | 3 |  |
| Engine Colling | Type | Water Cooled |  |
| Engine Rating | BHP | 38.5 BHP @ 1800 RPM |  |
| Speed | RPM | 1800 |  |
| Conforms to | IS | 10002/82 |  |
| Starting (12V) | Type | Electric |  |
| Fuel Consumption | Ltrs,/Hrs. | 4 |  |
| Fuel tank Capacity | Ltrs. | 75 |  |
| Meters/Gauges/Indications | Type/Detail | Lube oil pressure, Fuel level Indication, Battery charging current, Charging failure warning lamp, Engine speed, Hour Meter etc. |  |
| OPTIONAL Engine Safety Protections | Type | Engine Auto shut off in the event of low lube oil pressure (or) High Cylinder Head Temperature |  |

Cont.

AMBALASCIENCEMARKET.COM- VIKRAM MITTAL /M-9812204116	www.modernkitchenequipments.com

51

# SILENT CHALLENGER 2 X 301

The New generation Diesel Engine Driven Silent Welding Set Conforming To Latest CPCB II norms

Cont.

| Auxiliary Power Source (Built In) |  |  |  |
| --- | --- | --- | --- |
| Mode |  | Weld Load together with Auxiliary Load | Auxiliary Mode without Weld Load |
| Rating (3 Phase) | KAV/KW | 8 | 10 & 8 KVA from each socket Respectively (18 KVA Total) |
| OR |  |  |  |
| Rating (Single Phase) | KAV/KW | 3 | 3 KVA from each socket Respectively (6 KVA Total) |
| Votage (3 Phase / 1 Phase) | Volts | 415/240 |  |
| Phases | No | Three / Single |  |
| MCCB Rating | Amps | 10/12.5 | 27/25 |
| Dimensions, Rate & Mounting Details |  |  |  |
| Skid Mounted |  |  |  |
| Length/Width/Height | mm | 1995 x 820 x 1520 |  |
| Weight (Net) | Kgs | 1008 |  |
| Two Wheel Mounted |  |  |  |
| Length/Width/Height | mm | 2905 x 1455 x 2100 |  |
| Weight (Net) | Kgs | 1150 |  |
| Four Wheel Mounted |  |  |  |
| Length/Width/Height | mm | 3435 x 1555 x 2100 |  |
| Weight (Net) | Kgs | 1250 |  |

• Warranty : One year from the date of commissioning. ADOR WELDING LIMITED warrants that all new equipment sold from Plant/Area Offices / Authorised Distributors are free from defects in materials and workmanship and will perform in full accordance with applicable specifications.

• In view of continuous development, ADOR WELDING LIMITED reserves the right to modify/change the design and /or the specifications without any prior notice.

• Backed by dedicated customer care package.

AMBALASCIENCEMARKET.COM- VIKRAM MITTAL /M-9812204116	www.modernkitchenequipments.com

52

# MAXIMIG 251

The Workhorse Diode based Tap controlled MIG / MAG Welding Rectifier

CV

### SALIENT FEATURES

• Recommended for All position MIG / MAG welding applications

• Suitable for welding of wide range of metals like Carbon Steel, Stainless Steel and other steel alloys, with appropriate or recommended consumable

• High volume thin sheet metal fabrication welding as in Auto Industries and other engineering industries, maintenance and repair workshops

• Most ideal for Auto Ancillary units - for increased productivity and quality welding applications

• Most reliable and dependable workhorse welding machines with proven economy

• Diode based power source with traditional tapped transformer, provided with switch on front panel for step control of voltage, enabling welder to obtain the desired arc voltage with ease

• User friendly selection of 2 track / 4track operation for zero defect GMAW welding

• Good and consistent weld quality with low hydrogen content

• Dependable Thyristorized wire-feeder system ensures long, uninterrupted welding

• Quick release wire feed roller mechanism enables operator to change the wire spool quickly, minimising the unproductive time and thus increasing the productivity on the shop floor

• Standard Package comprises of Power Source, Wire feeder (2 Roll Drive) with 5 meter interconnection and Welding Torch only

• Rugged design, reliable performance and superior arc characteristics

### SPECIFICATIONS

| Technical Specifications |  |  | Unit | Maximig 251 |
| --- | --- | --- | --- | --- |
| Chracteristic Control |  |  |  | Tap Controlled Diode Based |
| Characteristics |  |  | Type | Constant Potential |
| Input |  |  |  |  |
| Input Supply: |  |  |  |  |
| Voltage |  |  | Volts, Ac | 380 / 415 , +10%, -10% |
| Phase |  |  | No | 3 |
| Frequency |  |  | Hz | 50/60 |
| Maximum Input Current |  |  | Amps, AC | 13 |
| Input Kva @ 100% Duty Cycle |  |  | KVA | 10 |
| Input Kva @ 60% Duty Cycle |  |  | KVA | 7 |
| No. Of Welding Steps |  |  |  | 16 |

Cont.

AMBALASCIENCEMARKET.COM- VIKRAM MITTAL /M-9812204116	www.modernkitchenequipments.com

53

# MAXIMIG 251

The Workhorse Diode based Tap controlled MIG / MAG Welding Rectifier

Cont.

| Output |  |  |  |  |
| --- | --- | --- | --- | --- |
| Open Current Voltage |  |  | Volts, DC | 16-36 |
| Welding Current Range |  |  | Amps, DC | 50-250 |
| Welding Current @ 100% duty cycle |  |  | Amps, DC | 195 @ 23.8 V |
| Welding Current @ 60% duty cycle |  |  | Amps, DC | 250 @ 26.5 V |
| General |  |  |  | 120% of the set current |
| Cooling |  |  | Type | Forced Air |
| Insulation |  |  | Class | F |
| Welding Output Terminals |  |  | Type | Stud Type For Lug Type Cable Connections |
| Dimensions (L X W X H) |  |  | mm | 770 x 400 x 740 |
| Weight (Approx) |  |  | Kg | 93 |

• Warranty : One year from the date of commissioning. ADOR WELDING LIMITED warrants that all new equipment sold from Plant/Area Offices / Authorised Distributors are free from defects in materials and workmanship and will perform in full accordance with applicable specifications.

• In view of continuous development, ADOR WELDING LIMITED reserves the right to modify/change the design and /or the specifications without any prior notice.

• Backed by dedicated customer care package.

AMBALASCIENCEMARKET.COM- VIKRAM MITTAL /M-9812204116	www.modernkitchenequipments.com

54

# MAXIMIG 400

The Workhorse Diode based Tap controlled MIG / MAG Welding Rectifier

CV

### SALIENT FEATURES

• Most reliable and dependable workhorse MIG / MAG welding machines proven for wide range of applications

• Rugged design, reliable performance and superior arc characteristics – minimum maintenance requirements

• Diode based power source with traditional tapped transformer, provided with switch on front panel for step control of voltage enabling welder to obtain the desired arc voltage with ease

• User friendly selection of 2track / 4track operation for zero defect GMAW welding

• Good and consistent weld quality with low hydrogen content

• Dependable Thyristorized wire feeder system ensures long, uninterrupted welding

• Quick release wire feed roller mechanism enables operator to change the wire spool quickly, minimizing the unproductive time and thus increasing the productivity on the shop floor

• Standard Package comprises of Power Source, Wire feeder (2 Roll Drive) with 5 meter interconnection and Welding Torch only

• Recommended for All position MIG / MAG welding applications

• Suitable for wide range of metals like Carbon Steel, Stainless Steel, with appropriate or recommended consumable

• Ideally recommended for heavy duty MIG / MAG welding applications like crane structure components manufacturing, 3 shift use, rough handling on the shop-floor and fabrication shops where maintenance staff is not very qualified

### SPECIFICATIONS

| Technical Specifications |  |  | Unit | Maximig 400 |
| --- | --- | --- | --- | --- |
| Chracteristic Control |  |  |  | Tap Controlled Diode Based |
| Characteristics |  |  | Type | Constant Potential |
| Input |  |  |  |  |
| Input Supply: |  |  |  |  |
| Input Supply Voltage |  |  | Volts, Ac | 415 , +10%, -10% |
| Phase |  |  | No | 3 |
| Frequency |  |  | Hz | 50/60 |
| Input Kva @ 100% Duty Cycle |  |  | KVA | 17.5 |
| No. Of Welding Steps |  |  |  | 32 |

Cont.

AMBALASCIENCEMARKET.COM- VIKRAM MITTAL /M-9812204116	www.modernkitchenequipments.com

55

# MAXIMIG 400

The Workhorse Diode based Tap controlled MIG / MAG Welding Rectifier

Cont.

| Output |  |  |  |  |
| --- | --- | --- | --- | --- |
| Open Circuit Voltage |  |  | Volts, DC | 18-54 |
| Welding Current Range |  |  | Amps, DC | 50-400 |
| Welding Current @ 100% duty cycle |  |  | Amps, DC | 310 @ 29.5 V |
| Welding Current @ 60% duty cycle |  |  | Amps, DC | 400 @ 34.0 V |
| General |  |  |  |  |
| Cooling |  |  | Type | Forced Air |
| Insulation |  |  | Class | H |
| Welding Output Terminals |  |  | Type | Stud Type |
| Dimensions (L X W X H) |  |  | mm | 835 x 435 x 820 |
| Weight (Approx) |  |  | Kg | 136 |
| Wire Feeder |  |  | Model | FEEDLITE 40 NEM(C) - 4 ROLL |
| Weight (Without Spool) |  |  | Kg | 16 KG (Approx) |
| Dimensions (LXWXH) |  |  | MM | 563 X 230 X 410 |
| Suitable for wire spool capacity |  |  | Kg | 15 Kg |
| Wire Feeder Fitted with rollers |  |  | mm | 1.2/1.6 for solid wire 2 No. |
| Wire Feeder Motor Voltage |  |  | Volts DC | 42 V DC |
| Wire Drive Motor |  |  | Type | Permanent Magnet DC Type |
| Wire Roll Drive |  |  | No | Four |
| Wire Feed Speed |  |  | Mtr/ min | 1.5 to 18 |
| Suitable for wire sizes |  |  | mm | 0.8,1,1.2,1.6 |
| Torch (3 Meter) |  |  |  | ADOR TW 402 (E) / ADOR MTG 400 (E) |

• Warranty : One year from the date of commissioning. ADOR WELDING LIMITED warrants that all new equipment sold from Plant/Area Offices / Authorised Distributors are free from defects in materials and workmanship and will perform in full accordance with applicable specifications.

• In view of continuous development, ADOR WELDING LIMITED reserves the right to modify/change the design and /or the specifications without any prior notice.

• Backed by dedicated customer care package.

AMBALASCIENCEMARKET.COM- VIKRAM MITTAL /M-9812204116	www.modernkitchenequipments.com

56

# STRIKER 400

Thyristor based welding power source, suitable for

MIG/MAG Semiautomatic and Automatic welding applications

CV

### SALIENT FEATURES

• Light weight, Compact design of Power source, Wire feeder and Torch

• Crater Voltage and Current control possible with ON/OFF switch

• LED Indications for Mains ON and Trip signal

• Globule Detachment Technique keeps the tip ready without globule formation for next welding cycle

• Protection against overheating and very high secondary short circuit current

•		Flux core or solid wire selection switch and Gas check Toggle switch on front panel

•		Special construction of transformer – resulting in faster response time and Excellent Arc dynamics

•		Digital voltage and current meters on front panel

•		Wheels for easy movement of machine on the shop floor

##### Details Of Complete System:

The complete system consists of power source, wire feeder, interconnecting cables and Torch as option. Remote control is provided along with the outfit to enable the user to set voltage and current while welding.

| Technical Specifications | Unit | TCVR405 (STRIKER) 400 |
| --- | --- | --- |
| Input |  |  |
| Voltage | Volts, Ac | 415 , +10%, -10% |
| Phase | No. | 3 |
| Frequency | Hz | 50 |
| Input Kva | KVA | 18.7 @ 100% Duty Cycle, 23 @ 60% Duty Cycle |
| Rated Input Current (max.) | Amps, Ac | 32 |
| Output |  |  |
| Open Circuit Voltage | Volts, Dc | 55 V |
| Welding Current Range | Amps, Dc | 50 - 400 |
| Welding Current @ 100% Duty Cycle | Amps, Dc | 310 |
| Welding Current @ 60% Duty Cycle | Amps, Dc | 400 |
| Welding Voltage Range | Volts, Dc | 16 - 39 |

Cont.

AMBALASCIENCEMARKET.COM- VIKRAM MITTAL /M-9812204116	www.modernkitchenequipments.com

57

# STRIKER 400

Thyristor based welding power source, suitable for MIG/MAG Semiautomatic and Automatic welding applications

Cont.

| General |  |  |
| --- | --- | --- |
| Ingress Protection | Class | IP23 |
| Cooling | Type | Forced Air |
| Insulation | Class | H |
| Dimensions (l X W X H) | mm | 780 X 450 X 915 |
| Weight | Kg | 157 |

| Wire Feeder | Unit | Feedlite 20 ST | Feedlite 40 ST |
| --- | --- | --- | --- |
| Drive Unit | Type | 2 Roll Print Motor | 4 Roll Print Motor |
| Motor Voltage | Volts, Dc | 18.3 | 18.3 |
| Wire Speed | Meters Per Minute | 1.5 - 18 | 1.5 - 18 |
| Wire Spool Capacity | Kg | 15 | 15 |
| Suitable For Wire Diameter | mm | 0.8, 1.0, 1.2 And 1.6 | 0.8, 1.0, 1.2 AND 1.6 |
| Inch Switch |  | On Wire Feeder | On Wire Feeder |
| Dimensions (l X W X H) | mm | 530 X 230 X 315 | 530 X 230 X 315 |
| Weight | Kg | 9 | 9 |

• Warranty : One year from the date of commissioning. ADOR WELDING LIMITED warrants that all new equipment sold from Plant/Area Offices / Authorized Distributors are free from defects in materials and workmanship and will perform in full accordance with applicable specifications.

• In view of continuous development, ADOR WELDING LIMITED reserves the right to modify/change the design and

/or the specifications without any prior notice.

• Backed by dedicated customer care package.

AMBALASCIENCEMARKET.COM- VIKRAM MITTAL /M-9812204116	www.modernkitchenequipments.com

58

# RANGER 600

The Heavy Duty Thyristorized MIG/MAG Welding Rectifier

CV

### SALIENT FEATURES

• Most reliable and dependable thyristor based power source for high quality MIG / MAG semi automatic manual and semiautomatic mechanized welding applications

• Hex phase, full wave full control bridge rectification circuit reduces ripple in the welding output - resulting in spatter free welding

• Special design and construction of Transformer - resulting faster response time and excellent Arc dynamics

• Light weight, Compact design of power source, wire feeder and Torch

• Adjustment of Crater Voltage, Crater Current possible with regulating pots provided on the front panel

• Globule detach technique keeps the tip ready for next welding cycle

• Choice of different types of wire feeder motors and welding torches available for different industrial applications

• Quick release wire feed roller mechanism enables operator to change the wire spool quickly, minimizing the unproductive time and thus increasing the productivity on the shop floor

• Power source protected from Over Voltage, Under Voltage supply, Single phasing, Over load / overheating and very high secondary short circuit current

• Suitable for ALL position MIG / MAG welding applications - both within shop-floor as well as at open yards and project sites (with due protection or shield against windy air)

• Suitable for wide range of metals like Carbon Steel, Stainless Steel, Aluminium and its alloys, with appropriate or recommended consumable

• Specifically designed for heavy duty, structural welding, machine building applications

### SPECIFICATIONS

| Technical Specifications | Unit | TCVR 600 (Ranger 600) |
| --- | --- | --- |
| Characteristics Control |  | Constant Potential Thyristorized Phase Control |
| Input |  |  |
| Input Supply: |  |  |
| Voltage | Volts, Ac | 415 , +10%, -10% |
| Phase | No | 3 |
| Frequency | Hz | 50/60 |

Cont.

AMBALASCIENCEMARKET.COM- VIKRAM MITTAL /M-9812204116	www.modernkitchenequipments.com

59

# RANGER 600

The Heavy Duty Thyristorized MIG/MAG Welding Rectifier

Cont.

| Maximum Input Current | Amps, AC | 55 |
| --- | --- | --- |
| Input Kva @ 100% Duty Cycle | KVA | 31.7 |
| Input Kva @ 60% Duty Cycle |  | 40 |
| Output |  |  |
| Open Circuit Voltage | Volts, DC | 65 V MAXIMUM |
| Welding Current Range | Amps, DC | 60-600 |
| Welding Current @ 100% duty cycle | Amps, DC | 465 |
| Welding Current @ 60% duty cycle | Amps, DC | 600 |
| General |  | 120% of the set current |
| Ingress Protection |  | IP23 |
| Cooling | Class | Forced Air |
| Insulation | Type | H |
| Welding Output Terminals | Class | Stud Type For Lug Type Cable Connections |
| Dimensions (L X W X H) | mm | 980 x 550 x 960 |
| Weight (Approx) | Kg | 230 |
| Wire Feeder |  | 120% of the set current |
| Weight (without spool) | Kg | 16 Kg Approx |
| Dimensions (L X W X H) | mm | 563X230X410 |
| Suitable for wire spool capacity | Kg | 15 Kg |
| Wire Feeder Fitted with Rollers | mm | 1.2/1.6 for Solid Wire 2 No. |
| Wire Feeder Motor Voltage | V DC | 42 V DC |
| Wire Driver Motor | Type | Permanent Magnet DC Type |
| Wire Roll Driver | No | Four |
| Wire Feed Speed | mtr/min | 1.5 to 18 |
| Suitable for Wire sizes | mm | 0.8,1,1.2,1.6 |

• Warranty : One year from the date of commissioning. ADOR WELDING LIMITED warrants that all new equipment sold from Plant/Area Offices / Authorised Distributors are free from defects in materials and workmanship and will perform in full accordance with applicable specifications.

• In view of continuous development, ADOR WELDING LIMITED reserves the right to modify/change the design and /or the specifications without any prior notice.

• Backed by dedicated customer care package.

AMBALASCIENCEMARKET.COM- VIKRAM MITTAL /M-9812204116	www.modernkitchenequipments.com

60

# RANGER 600 MULTI

The Heavy Duty Multipurpose Thyristorized Welding Rectifier for MMA / MIG / TIG / GOUGING Applications

CC/CV

TIG

GOUGING

### SALIENT FEATURES

• Heavy duty Thyristorized Multipurpose Welding Rectifier with Double-star configuration

• Suitable for multi process welding applications like SMAW / GMAW / FCAW / GTAW and GOUGING processes within shop floor as well as at open yards and project sites (with due protection or shield against windy air)

• Suitable for wide range of metals like Carbon Steel, Stainless Steel, Aluminium and its alloys, with appropriate or recommended consumable

• Specifically designed for heavy duty, structural welding, machine building application

• Rugged construction -most reliable and dependable thyristor based power source for high quality welding processes

• Hex- phase, full wave full control bridge rectification circuit reduces ripple in the welding output- resulting spatter-free welding

• Automatic Globule Detachment technique keeps the tip ready for next welding cycle

• Choice of 2 roll and 4 roll wire-feeders and different torches available for different industrial applications

• Quick release wire feed roller mechanism enables operator to change the wire spool quickly, minimising the unproductive time and thus increasing the productivity on the shop floor

• Power source protected from Over Voltage, Under Voltage supply, Single phasing, Over load/ overheating and very high secondary short circuit current

### SPECIFICATIONS

| Technical Specifications | Unit | TCCVR 600 |
| --- | --- | --- |
| Characteristics Control | CC/CV | Constant Potential Thyristorized Phase Control |
| Input |  |  |
| Input Supply: |  |  |
| Voltage | Volts, Ac | 415 , +10%, -10% |
| Phase | No | 3 |
| Frequency | Hz | 50/60 |
| Maximum Input Current | Amps, AC | 55 |
| Recommended Switch Fuse Rating | Amps, AC | 60 |
| Input Kva @ 100% Duty Cycle | KVA | 31.7 |
| Input Kva @ 60% Duty Cycle |  | 40 |

Cont.

AMBALASCIENCEMARKET.COM- VIKRAM MITTAL /M-9812204116	www.modernkitchenequipments.com

61

# RANGER 600 MULTI

The Heavy Duty Multipurpose Thyristorized Welding Rectifier for MMA / MIG / TIG / GOUGING Applications

Cont.

| Output |  |  |
| --- | --- | --- |
| Open Circuit Voltage | Volts, DC | 80 V (CC Mode) |
|  |  | 65 V MAXIMUM |
| Welding Current Range | Amps, DC | 20-600 (CC Mode) |
|  |  | 75-600 (CV Mode) |
| Welding Current (40°, 10 Min duty cycle) o o o o o ooo |  |  |
| @ 60% duty cycle | Amps, DC | 600 |
| @ 100% duty cycle | Amps, DC | 465 |
| General |  |  |
| Ingress Protection |  | IP23 |
| Cooling | Class | Forced Air |
| Insulation | Type | H |
| Welding Output Terminals | Class | Stud Type For Lug Type Cable Connections |
| Dimensions |  |  |
| Lenght | mm | 980 |
| Width | mm | 550 |
| Height (With Handel) | mm | 960 |
| Wire Feeder |  | FEEDLITE 40 NEH(C) |
| Weight (without spool) | Kg | 16 Kg Approx |
| Dimensions (L X W X H) | mm | 563X230X410 |
| Suitable for wire spool capacity | Kg | 15 Kg |
| Wire Feeder Fitted with Rollers | mm | 1.2/1.6 for Solid Wire 2 No. |
| Wire Feeder Motor Voltage | V DC | 42 V DC |
| Wire Driver Motor | Type | Permanent Magnet DC Type |
| Wire Roll Driver | No | Four |
| Wire Feed Speed | mtr/min | 1.5 to 18 |
| Suitable for Wire sizes | mm | 0.8,1,1.2,1.6 |

Weight (Approx)

Kg

227

• Warranty : One year from the date of commissioning. ADOR WELDING LIMITED warrants that all new equipment sold from Plant/Area Offices / Authorised Distributors are free from defects in materials and workmanship and will perform in full accordance with applicable specifications.

• In view of continuous development, ADOR WELDING LIMITED reserves the right to modify/change the design and /or the specifications without any prior notice.

• Backed by dedicated customer care package.

AMBALASCIENCEMARKET.COM- VIKRAM MITTAL /M-9812204116	www.modernkitchenequipments.com

62

# CHAMPMIG 250

The world class Inverter based MIG Welder

CV

### INTRODUCTION

CHAMP MIG 250 is IGBT inverter based welding outfit suitable for GMAW applications. The IGBT Power module, ferrite core and fast recovery diode are used as key device for power conversion and trans-mission to assure better efficiency and performance. The complete system consists of power source, wire feeder and MIG torch and interconnecting cables.

### DETAILS OF COMPLETE SYSTEM

| Sr. No. | Description | CHAMPMIG 250 A | CHAMPMIG 250 B |
| --- | --- | --- | --- |
| 1 | Power Source | ICVR 250 | ICVR 250 |
| 2 | Wire feeder | FEEDLITE 20 (NELR) | FEEDLITE 20 (NELR) |
| 3 | Torch | ADOR HIPRO 253 (E) | ADOR MTG 250 (E) |

SALIENT FEATURES

• Inverter based GMAW outfit

• High efficiency and high power factor – resulting energy saving

• Auto “Weld Stop” when welding torch is taken away from weld job

• Compatible to Power Generator Supply

• Enhanced Reliability due to SMD technology

• Digital Panel for adjusting the welding parameters

• 30% more Energy efficient than conventional machines

• Maximum Power factor is 0.93

• Excellent dynamic response   enables   superior arc characteristics. 2T, 4T operating modes

• Electronic choke adjustment for better arc control

• Crater voltage and Crater current adjustment through digital panel

• Unique feature of Globule Detachment to stop formation of globules at the tip of wire at the end of welding

• Automatic “Weld Stop” facility

### PROTECTION

##### The equipment is provided with following protections.

- Under Voltage and Over Voltage: Error message is displayed and the equipment shuts down if the supply voltage exceeds the positive or negative limits specified in the Technical Specifications

- Over Temperature: Error message is displayed and the equipment shuts down if the temperature of the main power components exceeds the safe limits

- Single Phasing Protection: Error message is displayed and the equipment shuts down if any one of the three phase supply line is absent (single-phasing prevention / protection)

AMBALASCIENCEMARKET.COM- VIKRAM MITTAL /M-9812204116	www.modernkitchenequipments.com

63

# CHAMPMIG 250

The world class Inverter based MIG Welder

SPECIFICATIONS

| Technical Specifications |  |  | Unit | ICVR 250 |
| --- | --- | --- | --- | --- |
| Input |  |  |  |  |
| Input Supply : |  |  |  |  |
| Voltage |  |  | Volt | 415, +15%, -10% |
| Phase |  |  | No | 3 |
| Frequency |  |  | Hz | 50/ 60 |
| Input KVA |  |  |  |  |
| @ 100% duty cycle |  |  | KVA | 6.5 |
| @ No Load |  |  | KVA | 0.19 |
| Output |  |  |  |  |
| Open Circuit Voltage |  |  | Volts +/-5% | 65 |
| Welding Current Range |  |  | Amp | 50-250 |
| Welding Current (40°C 10 minutes cycle) |  |  |  |  |
| @ 100% duty cycle |  |  | Amp | 195 |
| Welding Voltage Range |  |  | Volt | 16-34 |
| Crater Current Range |  |  | Amp | 50-250 |
| Crater Voltage Range |  |  | Volt | 16-34 |
| Mode of operation |  |  |  | 2 track, 4 track, Gas check, OCV check facility |
| Remote Control (on wire feeder) |  |  |  | For setting voltage & current |
| Power Factor |  |  |  | Max 0.93 |
| Efficiency |  |  | % | 83% @ 100% duty cycle |
| General |  |  |  |  |
| Compatibility to International standard |  |  |  | As per Std. EN60974-1 |
| Wire feed speed |  |  | m/min | 1.5-18 |
| Suitable Welding Wire Diameter |  |  | mm | Steel 0.8,1.0 and 1.2 |
| Ingress Protection |  |  | Class | IP23 |
| Cooling |  |  | Type | Forced Air |
| Dimensions (Approx.) |  |  |  |  |
| Length |  |  | mm | 500 |
| Width |  |  | mm | 260 |
| Height |  |  | mm | 470 |
| Weight |  |  | Kg | 26 |
| Audio Noise Emission |  |  |  | 70 |

Cont.

AMBALASCIENCEMARKET.COM- VIKRAM MITTAL /M-9812204116	www.modernkitchenequipments.com

64

# CHAMPMIG 250

The world class Inverter based MIG Welder

| Technical Specifications | Unit | FEEDLITE 20 (NELR) |
| --- | --- | --- |
| Suitable for wire | mm | 0.8,1,1.2 |
| Wirefeed speed | mtr/min | 1.5 to 18 |
| Wire roll drive | - | TWO |
| Wire drive motor | Type | Permanent Magnet DC |
| Wire feeder Motor voltage | V | 42 |
| Wire feeder fitted with rollers | - | 0.8/1.0/1.2 for sold wire 1no |
| Suitable for Wire Spool capacity | Kg | 15 |
| Dimensions |  |  |
| Length | mm | 460 |
| Width | mm | 210 |
| Height | mm | 280 |
| Weight (without spool) | Kg | 10 (Approx) |

| Torch Specification |  |  |  |
| --- | --- | --- | --- |
| Technical Specifications | Unit | ADOR HIPRO 253 (E) | ADOR MTG 250 (E) |
| Rating | A | 250A @ 60% Duty cycle for CO2 | 250A @ 60% Duty cycle for CO2 |
| End Connection to Torch | Type | Euro | Euro |
| Suitable for Wire Ø | mm | 0.8, 1.0, 1.2 | 0.8, 1.0, 1.2 |
| Torch Length | meter | 3 | 3 |

• Warranty : One year from the date of commissioning. ADOR WELDING LIMITED warrants that all new equipment sold from Plant/Area Offices / Authorised Distributors are free from defects in materials and workmanship and will perform in full accordance with applicable specifications.

• In view of continuous development, ADOR WELDING LIMITED reserves the right to modify/change the design and /or the specifications without any prior notice.

• Backed by dedicated customer care package.

AMBALASCIENCEMARKET.COM- VIKRAM MITTAL /M-9812204116	www.modernkitchenequipments.com

65

# OMNIMIG 250

The new Inverter based Compact MIG Welder

1

CV

### SALIENT FEATURES

• Latest Inverter based technology.

• Low Power Consumption due to Power Factor Correction Technique that brings the Power Factor nearly equal to one.

• Single point controlled synergic MIG welding operation.

• 3-digit Digital Panel Meters to display the set voltage, actual voltage, set current and actual current while welding.

• LED Indications for Mains ON and Trip signal.

• Protections: Over-Voltage, Under -Voltage, Over- Temperature & Output Short Circuit Protection.

• Compact Design with built-in Wirefeeder.

• Gas Trolley for easy movement of machine on the shop floor.

### TECHNICAL SPECIFICATIONS - POWER SOURCE

| Technical Specifications |  |  | Unit | OMNIMIG |
| --- | --- | --- | --- | --- |
| Input |  |  |  |  |
| Input Supply : |  |  |  |  |
| Voltage |  |  | Volts, Ac | 230 V +15%, -10%, |
| Phase |  |  |  | 1 |
| Frequency |  |  | HZ | 50/ 60 |
| Input Power @ 230VAC in MIG Mode |  |  |  | CC - CV |
| @100% duty cycle |  |  | KVA | 4.6 Max. |
| @ 60% duty cycle |  |  | KVA | 6.5 Max. |
| @ 35% duty cycle |  |  | KVA | 8.5 Max. |
| @ No Load |  |  | KVA | 0.15 |
| Input Current 415V Supply |  |  |  |  |
| @ 100% duty cycle |  |  | Amps, Ac | 18.9 |
| @ 60% duty cycle |  |  | Amps, Ac | 27.3 |
| @ 35% duty cycle |  |  | Amps, Ac | 36.2 |
| @ No Load |  |  | Amps, Ac | 0.3 |
| Power Factor |  |  |  | 0.99 Max. |
| Efficiency |  |  | % | >82 |
| Output |  |  |  |  |
| Open Circuit Voltage @230V Input Supply |  |  | Volts, DC | 75V DC (+/-5V) |
| Welding Voltage Range |  |  | Volts, DC | 14 - 34 |
| Welding Current Range |  |  | Amps, DC | 50 - 250 |

Cont.

AMBALASCIENCEMARKET.COM- VIKRAM MITTAL /M-9812204116	www.modernkitchenequipments.com

66

# OMNIMIG 250

The new Inverter based Compact MIG Welder

Cont.

| Crater Voltage Range |  |  | Volt, DC | 14 - 34 |
| --- | --- | --- | --- | --- |
| Crater Current Range |  |  | Amps, DC | 50 - 250 |
| Welding Current (40°C 10 minutes cycle) |  |  |  |  |
| @ 100% duty cycle |  |  | Amps, DC | 160 |
| @ 60% duty cycle |  |  | Amps, DC | 195 |
| @ 35% duty cycle |  |  | Amps, DC | 250 |
| General |  |  |  |  |
| Gas Preheater Rating |  |  | V / A | 90 V DC / 1.1 Amps |
| Welding Wire Sizes (Diameter) |  |  | mm | 0.8, 1.0, 1.2 |
| Protections (Auto Reset) |  |  | - | Over Voltage, Under Voltage, Over Temperature, Output Short Circuit |
| Front Panel Functions |  |  |  | - 2 Track/ 4 track control and Crater Setting - Creep Feed - Adjustable Burn-back - Display - set Voltage and Wire Speed, actual Voltage and Welding Current - Pre flow and Post flow - Wire Diameter Selection- 0.8, 1.0 & 1.2 - Shielding Gas Selection- Co2 And Mixed Arco2 |
| Cooling |  |  | Type | Forced Air |
| Class of Insulation |  |  | - | H |
| Degree of Protection |  |  | - | IP23S |
| Dimensions (L x W x H) Power Source |  |  | mm | 600 x 330 x 530 |
| Dimensions (L x W x H) With Trolley |  |  | Kg | 1040 x 475 x 1140 |
| Weight (Approx. only Power Source) |  |  | Kg | 32 |
| Weight (Approx. Trolley) |  |  | Kg | 77 |

Cont.

AMBALASCIENCEMARKET.COM- VIKRAM MITTAL /M-9812204116	www.modernkitchenequipments.com

67

# OMNIMIG 250

The new Inverter based Compact MIG Welder

FRONT CONTROL PANEL – OMNIMIG 250

Cont.

### TECHNICAL SPECIFICATIONS – BUILT-IN WIRE FEEDER

| Wire Feeder (built - in) |  |  |
| --- | --- | --- |
| Parameter | Unit | Value |
| Drive Unit | - | 2 Roll PMDC Motor |
| Motor Voltage | Volt, Dc | 42 V DC ± 0.5 V |
| Rated Power | Watts | 90 W |
| End Connection For Torch | - | Euro Type |
| Suitable For Wire Sizes | mm | 0.8, 1.0, 1.2 |
| Wire Drive Motor. | Type | Permanent Magnet DC Type With Encoder Feedback |
| Rated Power | Watts |  |
| Wire Roll Drive | No | Four |
| Wire Feed Speed | Mtrs. / Min. | 2-22 (For 0.8 mm Wire), 2-18 (For 1.0 mm Wire), 2-8 (For 1.2 mm Wire) |

Cont.

AMBALASCIENCEMARKET.COM- VIKRAM MITTAL /M-9812204116	www.modernkitchenequipments.com

68

# OMNIMIG 250

The new Inverter based Compact MIG Welder

Cont.

TECHNICAL SPECIFICATIONS – WELDING TORCH

| Mig Welding Torch |  |  |  |
| --- | --- | --- | --- |
| Parameter | Unit | Value |  |
| Maximum Current @ 60% Duty Cycle | Amps | 250 Amps With Co2 Gas, 200 Amps With Mixed Gas |  |
| Torch Cable Length | Meter | 3 |  |
| Torch Adapter | Type | Euro Type |  |
| Suitable For Wire Size Diameter | mm | 0.8, 1.0, 1.2 |  |
| Standard Accessories: |  |  |  |
| Gas Hose With Both Side End Connectors |  |  | 2 Mtr. |

• Warranty : One year from the date of commissioning. ADOR WELDING LIMITED warrants that all new equipment sold from Plant/Area Offices / Authorised Distributors are free from defects in materials and workmanship and will perform in full accordance with applicable specifications.

• In view of continuous development, ADOR WELDING LIMITED reserves the right to modify/change the design and /or the specifications without any prior notice.

• Backed by dedicated customer care package.

AMBALASCIENCEMARKET.COM- VIKRAM MITTAL /M-9812204116	www.modernkitchenequipments.com

69

# CHAMPMIG 300

The world class Inverter based MIG / MMA Welder

CC/CV

### TYPICAL APPLICATION

• Three phase inverter based high efficiency and high power factor DC Welder with high frequency switching technology.

• Suitable for both MIG and MMA welding operation.

• User friendly digital front panel interface with 7 segment LED display and multi function encoders.

• Anti stick function to protect the machine from short circuit condition in MMA mode.

• Option of Remote control unit with digital display for setting wire feed speed/current and voltage remotely in MIG/MMA mode.

### SPECIFICATIONS

| Technical Specifications |  |  | Unit | CHAMPMIG 300 |
| --- | --- | --- | --- | --- |
| Input |  |  |  |  |
| Input Supply : |  |  |  |  |
| Voltage |  |  | Volts, Ac | 415, +15%, -10% |
| Phase |  |  |  | 3 |
| Frequency |  |  | HZ | 50/ 60 |
| Characteristics |  |  |  | CC - CV |
| Input KVA @ 100% duty cycle |  |  | KVA | 8.1-MIG Mode, 10.66 MMA Mode |
| Input KVA @ 60% duty cycle |  |  | KVA | 10.9-MIG Mode, 13.08 MMA Mode |
| Input KVA @ 35% duty cycle |  |  | KVA | 12.9-MIG Mode, 17.45 MMA Mode |
| Input Current @ 100% duty cycle |  |  | Amps, Ac | 11.2-MIG Mode, 14.9 MMA Mode |
| Input Current @ 60% duty cycle |  |  | Amps, Ac | 15.1-MIG Mode, 18.4 MMA Mode |
| Input Current @ 35% duty cycle |  |  | Amps, Ac | 18.2-MIG Mode, 24.6 MMA Mode |
| Power Factor |  |  | % | Up to 85% |
| Efficiency |  |  |  | 0.84 Max. |
| Output |  |  |  |  |
| Open Circuit Voltage |  |  | Volts, DC | 65 V (+/-3V) - MIG mode, 88V (+/-5V) MMA Mode |
| Welding Current Range |  |  | Amps, DC | 50-250 |
| Welding Current (40°C 10 minutes cycle) |  |  |  |  |
| @ 100% duty cycle |  |  | Amps, DC | 195 |
| @ 60% duty cycle |  |  | Amps, DC | 16-34 |
| @ 35% duty cycle |  |  | Amps, DC | 50-250 |
| Auxiliary Pre Heating Supply |  |  | Volts, AC, Watts | 16-34 |

Cont.

AMBALASCIENCEMARKET.COM- VIKRAM MITTAL /M-9812204116	www.modernkitchenequipments.com

70

# CHAMPMIG 300

The world class Inverter based MIG / MMA Welder

Cont.

| Front Panel Functions |  |  |
| --- | --- | --- |
| 1. Creep On/Off Switch | 2. Crater Current & Voltage Adjustment | 3. MMA/MIG Selection Switch |
| 4. 2T/4T selection switch | 5. GAS/OVC Check Switch | 6. GAS CO2/ARCO2 Selection Switch |
| 7. Segment LED display for displaying set Voltage & Wire Feed speed, while Welding display Actual Current & Voltage | 8. Wire Selection 0.8/1.0/1.2 mm selection switch | 9. Menu Switch Selecting all Functions - Gas Pre Flow Time, Weld on Current & Voltage, Burn Back Time, Gas Post Flow Time |
| 10. Encoder for selection parameter Value Increment / Decrement | 11. 10 memory Location with Save & recall function | 12. Remote connector (10 meter remote control can be provide as an option for setting Voltage & wire feed speed) |
| 13. Output Terminal KNOBS | 14. Mains on ‘Green’ Color indication | 17. Trip ‘Red’ Color LED Protection Mode |
| 16. Torch switch gear | 17. Gas Out |  |

| General | Unit | Value |
| --- | --- | --- |
| Ingress Protection |  | IP23S |
| Cooling | Class | Forced Air |
| Insulation | Type | H |
| Dimensions (L x W x H) | mm | 575 x 225 x 450 |
| Weight (approx.) | Kg | 28 |
| MIG Welding Parameter | Unit | Value |
| Gas Pre Flow Time | Sec. | 0 to 10 |
| Current Range in MIG | Amp. | 10 to 300 |
| Voltage Range in MIG | Volts. | 14 to 40 |
| Crater Current in MIG | Amp. | 10 to 300 |
| Crater Voltage in MIG | Volts. | 14 to 40 |
| Burn Back Time | Sec. | 0 to 2 |
| Gas Post Flow Time | Sec. | 0 to 10 |

• Warranty : One year from the date of commissioning. ADOR WELDING LIMITED warrants that all new equipment sold from Plant/Area Offices / Authorised Distributors are free from defects in materials and workmanship and will perform in full accordance with applicable specifications.

• In view of continuous development, ADOR WELDING LIMITED reserves the right to modify/change the design and /or the specifications without any prior notice.

• Backed by dedicated customer care package.

AMBALASCIENCEMARKET.COM- VIKRAM MITTAL /M-9812204116	www.modernkitchenequipments.com

71

# CHAMPMIG 400

The world class Inverter based MIG Welder

CV

### INTRODUCTION

CHAMP MIG 400 is IGBT inverter based welding outfit suitable for GMAW applications. The IGBT Power module, ferrite core and fast recovery diode are used as key device for power conversion and transmission to assure better efficiency and performance.

The complete system consists of power source, wire feeder, MIG torch and interconnecting cables.

### DETAILS OF COMPLETE SYSTEM

| Sr. No. | Description | CHAMPMIG 400 | CHAMPMIG 400 |
| --- | --- | --- | --- |
| 1 | Power Source | ICVR 400 | ICVR 400 |
| 2 | Wire feeder | FEEDLITE 40 (NEMRC) | FEEDLITE 40 (NEMRC) |
| 3 | Torch | ADOR HIPRO 403 (E) | ADOR MTG 400 (E) |

SALIENT FEATURES

• Inverter based digitally controlled GMAW outfit

• Digital Panel for adjusting the welding parameters

• 25% more Energy efficient than conventional machines

• Maximum Power factor is 0.94

• Excellent dynamic response enables superior arc characteristics

• 2T, 4T operating modes

• Welding dynamic adjustment for better arc control

• Crater voltage and Crater current adjustment through digital panel

• Unique feature of Fresh Tip Transfer (FTT) to avoid globule formation

• Automatic “Weld Stop” facility

### PROTECTION

##### The equipment is provided with following protections.

- Under Voltage and Over Voltage: Error message is displayed and the equipment shuts down if the supply voltage exceeds the positive or negative limits specified in the Technical Specifications

- Over Temperature: Error message is displayed and the equipment shuts down if the temperature of the main power components exceeds the safe limits

- Single Phasing Protection: Error message is displayed and the equipment shuts down if any one of the three phase supply line is absent (single-phasing prevention / protection)

AMBALASCIENCEMARKET.COM- VIKRAM MITTAL /M-9812204116	www.modernkitchenequipments.com

72

# CHAMPMIG 400

The world class Inverter based MIG Welder

SPECIFICATIONS

| Technical Specifications |  |  | Unit | ICVR 400 |
| --- | --- | --- | --- | --- |
| Input |  |  |  |  |
| Input Supply : |  |  |  |  |
| Voltage |  |  | Volt | 415 , +15%, -10% |
| Phase |  |  | No | 3 Ø |
| Frequency |  |  | Hz | 50/ 60 |
| Input KVA |  |  |  |  |
| @ 100% duty cycle |  |  | KVA | 12 |
| @ No Load |  |  | KVA | 0.24 |
| Output |  |  |  |  |
| Open Circuit Voltage |  |  | Volts +/-5% | 65 |
| Welding Current Range |  |  | Amp | 50-400 |
| Welding Current (40°C 10 minutes cycle) |  |  |  |  |
| @ 60% duty cycle |  |  | Amp | 400 |
| @ 100% duty cycle |  |  | Amp | 310 |
| Welding Voltage Range |  |  | Volt | 16-39 |
| Crater Current Range |  |  | Amp | 50-400 |
| Crater Voltage Range |  |  | Volt | 16-39 |
| Mode of operation |  |  |  | 2 track, 4 track, Gas check, OCV check facility |
| Remote Control |  |  |  | For setting voltage & current |
| Power Factor |  |  |  | Max 0.93 |
| Efficiency |  |  | % | 87% @ 100% duty cycle |
| General |  |  |  |  |
| Compatibility to International standard |  |  |  | As per Std. EN60974-1 |
| Wire feed speed |  |  | m/min | 1.5-18 |
| Suitable Welding Wire Diameter |  |  | mm | Aluminum 1.2 – 1.6 Steel 0.8,1.0,1.2 & 1.6 |
| Ingress Protection |  |  | Class | IP23 |
| Cooling |  |  | Type | Forced Air |
| Dimensions (Approx.) |  |  |  |  |
| Length |  |  | mm | 700 |
| Width |  |  | mm | 450 |
| Height |  |  | mm | 600 |
| Weight |  |  | Kg | 44 |
| Audio Noise Emission |  |  | dB | 70 |

Cont.

AMBALASCIENCEMARKET.COM- VIKRAM MITTAL /M-9812204116	www.modernkitchenequipments.com

73

# CHAMPMIG 400

The world class Inverter based MIG Welder

| Technical Specifications | Unit | FEEDLITE 40 (NEMRC) |
| --- | --- | --- |
| Suitable for wire Ø | mm | 0.8,1,1.2,1.6 |
| Wirefeed speed | mtr/min | 1.5 to 18 |
| Wire roll drive | - | Four |
| Wire drive motor | Type | Permanent Magnet DC |
| Wire feeder Motor voltage | V | 42 |
| Wire feeder fitted with rollers | - | 1.2/1.6 for sold wire 2 no |
| Suitable for Wire Spool capacity | Kg | 15 |
| Dimensions |  |  |
| Length | mm | 563 |
| Width | mm | 230 |
| Height | mm | 410 |
| Weight (without spool) | Kg | 16 (Approx) |

TORCH SPECIFICATION

| Technical Specifications | Unit | ADOR HIPRO 403 (E) | ADOR MTG 400 (E) |
| --- | --- | --- | --- |
| Rating | A | 400A @ 60% Duty cycle for CO2 | 400A @ 60% Duty cycle for CO2 |
| End Connection to Torch | Type | Euro | Euro |
| Suitable for Wire Ø | mm | 0.8, 1.0, 1.2, 1.6 | 0.8, 1.0, 1.2, 1.6 |
| Torch Length | meter | 3 | 3 |

• Warranty : One year from the date of commissioning. ADOR WELDING LIMITED warrants that all new equipment sold from Plant/Area Offices / Authorised Distributors are free from defects in materials and workmanship and will perform in full accordance with applicable specifications.

• In view of continuous development, ADOR WELDING LIMITED reserves the right to modify/change the design and /or the specifications without any prior notice.

• Backed by dedicated customer care package.

AMBALASCIENCEMARKET.COM- VIKRAM MITTAL /M-9812204116	www.modernkitchenequipments.com

74

# CHAMPMIG 600

The world class Inverter based MIG Welder

CV

### INTRODUCTION

CHAMP MIG 600 is an IGBT inverter based welding power source, suitable for GMAW applications. The IGBT Power module, high frequency transformer and fast recovery diode are used as key device for power conversion and transmission to assure better efficiency and performance.

The complete system consists of power source, wire feeder, water cooled or gas cooled torch, water cooling unit (for water cooled systems) and interconnecting cables.

### DETAILS OF COMPLETE SYSTEM

| Sr. No. | Description | CHAMPMIG 600 A | CHAMPMIG 600 E |
| --- | --- | --- | --- |
| 1 | Power Source | ICVR 600 | ICVR 600 |
| 2 | Wire feeder | FEEDLITE 40 (NEH-RC) | FEEDLITE 40 (NEH-RC) |
| 3 | Torch | ADOR-MTG 600 E. | ADOR TW 502 (E) |

SALIENT FEATURES

• Inverter based digitally controlled GMAW outfit

• Auto “Weld Stop” when welding torch is taken away from weld job

• Digital Panel for adjusting the welding

• 25% more Energy efficient than conventional machines.

• Maximum Power factor is 0.95

• Excellent dynamic response   enables   superior arc characteristics. 2T, 4T operating modes

• Dynamic Inductance adjustment for better arc control

• Crater voltage and Crater current adjustment through digital panel. Unique feature of Pinch off pulse to avoid globule formation.

• Automatic “Weld Stop” facility.

• Normal/ Creep feeding selection available on on front panel.

PROTECTION

##### The equipment is provided with following protections.

- Under Voltage and Over Voltage: If supply voltage goes lower or higher than set limit, error message is displayed on Display

- Over Temperature: If the temperature of the main power elements is more than safety limits, error message is displayed

- Single Phasing Protection: If any one of the three phases (R, Y, and B) is absent, welding will stop. Welding current would not be available in this condition)

AMBALASCIENCEMARKET.COM- VIKRAM MITTAL /M-9812204116	www.modernkitchenequipments.com

75

# CHAMPMIG 600

The world class Inverter based MIG Welder

SPECIFICATIONS

| Technical Specifications |  |  | Unit | ICVR 600 |
| --- | --- | --- | --- | --- |
| Input |  |  |  |  |
| Input Supply : |  |  |  |  |
| Voltage |  |  | Volt | 415, +15%, -10% |
| Phase |  |  | No | 3 Ø |
| Frequency |  |  | Hz | 50/ 60 |
| Max Input KVA |  |  |  |  |
| @ 100% duty cycle |  |  | KVA | 21.5 |
| @ No Load |  |  | KVA | 0.31 |
| Output |  |  |  |  |
| Open Circuit Voltage |  |  | Volts +/-5% | 65 |
| Welding Current Range |  |  | Amp | 65-600 |
| Welding Current (40°C 10 minutes cycle) |  |  |  |  |
| @ 60% duty cycle |  |  | Amp | 600 |
| @ 100% duty cycle |  |  | Amp | 465 |
| Welding Voltage Range |  |  | Volt | 16-45 |
| Crater Current Range |  |  | Amp | 65-600 |
| Crater Voltage Range |  |  | Volt | 16-45 |
| Mode of operation |  |  |  | 2 track, 4 track, Gas check, OCV check facility |
| Remote Control |  |  |  | For setting voltage & current |
| Power Factor |  |  |  | Max 0.94 |
| Efficiency |  |  | % | 89% @ 100% duty cycle |
| General |  |  |  |  |
| Compatibility to International standard |  |  |  | As per Std. EN60974-1 |
| Wire feed speed |  |  | m/min | 1.5-18 |
| Suitable Welding Wire Diameter |  |  | mm | Aluminum 1.2 – 1.6 Steel 0.8,1.0,1.2 & 1.6 |
| Ingress Protection |  |  | Class | IP23 |
| Cooling |  |  | Type | Forced Air |
| Dimensions (Approx.) |  |  |  |  |
| Length |  |  | mm | 670 |
| Width |  |  | mm | 450 |
| Height |  |  | mm | 620 |
| Weight |  |  | Kg | 56 |
| Audio Noise Emission |  |  | dB | 70 |

Cont.

AMBALASCIENCEMARKET.COM- VIKRAM MITTAL /M-9812204116	www.modernkitchenequipments.com

76

# CHAMPMIG 600

The world class Inverter based MIG Welder

| Technical Specifications | Unit | Feedlite 40 (NEHR)C |
| --- | --- | --- |
| Suitable for wire Ø | mm | 0.8,1,1.2,1.6 mm |
| Wirefeed speed | mtr/min | 1.5 to 18 mtr/min |
| Wire roll drive | - | Four |
| Wire drive motor | Type | Permanent Magnet DC |
| Wire feeder Motor voltage | V | 42 |
| Wire feeder fitted with rollers | - | 1.2/1.6 for sold wire 2 no |
| Suitable for Wire Spool capacity | Kg | 15 |
| Dimensions |  |  |
| Length | mm | 563 |
| Width | mm | 230 |
| Height | mm | 410 |
| Weight (without spool) | Kg | 16 (Approx) |

• Warranty : One year from the date of commissioning. ADOR WELDING LIMITED warrants that all new equipment sold from Plant/Area Offices / Authorised Distributors are free from defects in materials and workmanship and will perform in full accordance with applicable specifications.

• In view of continuous development, ADOR WELDING LIMITED reserves the right to modify/change the design and /or the specifications without any prior notice.

• Backed by dedicated customer care package.

AMBALASCIENCEMARKET.COM- VIKRAM MITTAL /M-9812204116	www.modernkitchenequipments.com

77

# CHAMP MULTI 400, 600

The world class Inverter based Multipurpose Welder

TIG

CC/CV

### SALIENT FEATURES

• CHAMPMULTI- 400 and CHAMPMULTI- 600 are Multi process welding outfits with Inverter based welding power source

• The IGBT power module, High frequency transformer and fast recovery diode are used as key device for power conversion and transmission to assure better Efficiency and performance

• The welding power source has both constant current (CC) and constant voltage characteristics (CV), which are suitable for MMA and MIG/MAG and FCAW applications

• Set output parameters are constant against input supply variations

• Power source is protected against single phasing, under voltage, over voltage, short circuit and temperature rise

• Water cooled version available for CHAMP MULTI 600 as option

• MMA process with this outfit is most suitable for all kinds of electrodes including CELWEL for fabrication work, pipe welding, site construction etc

• GMAW process is suitable for welding in semiautomatic / automatic mechanism for welding MS, SS and Al materials with Solid and Flux core wires (FCAW Mode)

• Both the models can operate with single point Synergic control in MIG/MAG mode

• The complete system consists of Power Source, wire feeder, torch and inter connecting cables and control cables between wire feeder and power source

• Available with both Gas cooled and Water cooled MIG Torches for 600 Amp model

• High efficiency (>85%)

• Single point Synergic control in GMAW

• User friendly Digital front panel and analog remote controller

• Auto “Weld Stop” when welding torch is taken away from work piece

• 2T, 4T and SPOT and Multi Spot operating modes in MIG Mode as well as FCAW mode

• Dynamic Inductance adjustment in GMAW process and Arc force adjustment in MMA process for better arc control

• Crater voltage and Crater current adjustment through digital panel

• Unique feature of Pinch-off pulse to avoid globule formation

AMBALASCIENCEMARKET.COM- VIKRAM MITTAL /M-9812204116	www.modernkitchenequipments.com

78

# CHAMP MULTI 400, 600

The world class Inverter based Multipurpose Welder

PROTECTIONS WITH AUTO RESET

##### The Equipment is provided with following protections:

• Under / Over Input supply Voltage :

  - Red LED glows if input supply voltage goes below 330V AC

  - Red LED glows if input supply voltage goes above 480V AC

• Over Temperature Trip :

If the temperature of the Semiconductor Component is increased above safety limits then machine goes in safety mode (Trip Mode). In this condition welding voltage will not be available and welding will stop.

- Single phasing protection :

If any one of three phases of input supply (R, Y, B) is absent, then Machine will Trip and Red LED will glow. In this condition welding voltage will not be available and welding will stop.

- Output  Short      Circuit      Protection      : Welding Output is protected against any Short circuit.

- No Output Voltage will be available in both conditions.

### DETAILS OF COMPLETE SYSTEM

| Sr. No. | Description | Model |  |
| --- | --- | --- | --- |
|  |  | Champ Multi 400 | Champ Multi 600/600 (W) |
| 1 | Power Source | ICCCVR- 401 | ICCCVR- 600 |
| 2 | Wire Feeder | FEEDLITE 40 (NEM)- C | FEEDLITE 40 (NEH - C) / NEHW) |
| 3 | Torch | HIPRO 403 (E) / MTG 400 (E) | TW 502 (E) / MTG 600 (E) for Gas cooled systems TW 600 (WE) for Water cooled systems |
| 4 | Gas Pressure Regulator Cum Flow Meter (optional) | ARGON Or CO2 Regulator | ARGON Or CO2 Regulator |
| 5 | Gas Heater (optional) | 110V AC, In Case of CO2 Regulator | 110V AC, In Case of CO2 Regulator |
| 6 | Water Cooling Unit (optional) |  | WCU-302 For Water Cooled Version Only |

| Sr. No. | Error Code | Error |
| --- | --- | --- |
| 1 | ERR 001 | Under Voltage Error |
| 2 | ERR 002 | Over Voltage Error |
| 3 | ERR 003 | Thermal Trip Error |
| 4 | ERR 004 | No Current Flow Error |
| 5 | ERR 005 | Feeder Motor Overloading Error |
| 6 | ERR 006 | Wire Feeder Error |
| 7 | ERR 007 | Water Presure Error |
| 8 | ERR 008 | Communication Error |

AMBALASCIENCEMARKET.COM- VIKRAM MITTAL /M-9812204116	www.modernkitchenequipments.com

79

# CHAMP MULTI 400, 600

The world class Inverter based Multipurpose Welder

SPECIFICATIONS

| Parametre | Unit | Value |  |
| --- | --- | --- | --- |
|  |  | CHAMPMULTI 400 | CHAMPMULTI 600 |
| Nominal Input Voltage | V AC | 415 V, 3 PH |  |
| Input Voltage Range | V AC | 415 (+15% - 10%) |  |
| Phase | No | 3 |  |
| Frequency | Hz | 50/ 60 |  |
| Efficiency @ 100% duty cycle | % | >90 | >89 |
| Power Factor @ 100% duty cycle |  | 0.93 Max |  |
| Open Circuit Voltage In MMA Mode @415v, 3 Phase | V DC | 84 V | 90 V |
| Open Circuit Voltage In MIG Mode @415v, 3 Phase |  | 55 V | 65 V |
| Open Circuit Voltage In TIG Mode @415v, 3 Phase |  | 84 V | 90 V |
| Welding Current Range In MMA Mode | A DC | 50-400 A | 50-600 |
| Welding Current Range In TIG Mode |  | 10-400 | 10-600 |
| Welding Current Range In MIG Mode |  | 40-400 | 40-600 |
| Welding Current @ 100% duty cycle (10 minute cycle) | A DC | 310 | 465 |
| Welding Current @ 60% duty cycle (10 minute cycle) |  | 400 | 600 |
| Input Power (In MMA Mode) @ 100% duty cycle | KVA | 12.0 | 22.0 |
| Input Power (In MIG Mode) @ 100% duty cycle |  | 11.0 | 22.0 |
| Input Power (In TIG Mode) @ 100% duty cycle |  | 9.0 | 16.5 |
| Crater Current Range In MIG Mode |  | 500-400 A | 65-600 A |
| Crater Current Voltage In MIG Mode | - | 14-40 V | 14-44 V |
| Cooling | Type | Forced Air |  |
| Class of Insulation | Class | H |  |
| Degree of Protection | - | IP23 |  |
| Protections Auto Resettable | - | Over Voltage, Undervoltage, Single– Phasing, Over Temperature |  |
| Suitable Welding Electrode Size Dia. - In MMA Mode | mm | 2.5, 3.2 , 4, 5, 6 MM Diameter |  |
| Suitable Welding Electrode Size Dia. - In MIG Mode | mm | 0.8, 1.0, 1.2, 1.6 MM Diameter |  |
| Dimensions L x W x H | mm | 650 x 450 x 570 | 700 x 460 x 650 |
| Auxiliary Outputs On Back Panel | V AC | 110 V – Gas Heater, 230 V – Water Cooling Unit |  |
| Front Panel Functions | - | Ø MMA /TIG/ MIG /FCAW Process Selection |  |
|  | MIG / FCAW MODE | Ø Separate Switches To Check OCV, Gas Flow & Wire Inch. |  |
|  |  | Ø Selection Switches For Wire Dia, Material & Gas For Synergic Application. |  |
|  |  | Ø Welding Mode 2t / 4t / Spot / Multispot Selection Switch. |  |
|  |  | Ø Auto/manual Selection Switch. |  |

Cont.

AMBALASCIENCEMARKET.COM- VIKRAM MITTAL /M-9812204116	www.modernkitchenequipments.com

80

# CHAMP MULTI 400, 600

The world class Inverter based Multipurpose Welder

Cont.

| Front Panel Functions | MIG / FCAW Mode | Ø Save & Recall Switches To Save & Recall 10 Programs. |  |
| --- | --- | --- | --- |
|  |  | Ø Facility To Set Preflow, Post Flow Burnback, Spot & Pause Time Before Welding. |  |
|  | - | Ø 3 Digits Digital Display For Voltage and Current. |  |
|  | MMA / TIG Mode | Ø Current Setting By Encoder |  |
|  |  | Ø Encoder For Arc Force Setting With On/ Off Facility Switch Only In MMA Mode. |  |
| Remote Control | - | Remote Control With Cable For Setting Voltage and Current |  |
| Mounting Wheels Of The Power Source | - | Wheel Mount |  |
|  |  | Front: Swivel, Rear: Fixed |  |
| Lifting Arrangement |  | Handle Provided |  |
| Weight (Approx.) | Kg. | 52 | 57 |

SYNERGIC FUNCTIONS

Select the right program

The Synergic Panel shows weld-programs loaded inside the power source

  - Shielding Gas selection- ArCO2 / CO2 / Ar / Ar O2

  - Weld Material selection- Fe / Al / SS

  - Wire Sizes selection- 0.8 / 1.0 / 1.2 / 1.6

Wire Feed Speed is the dominant control parameter for each wire feed speed, a corresponding voltage is programmed into the machine.

Turning the knob, the voltage can be trimmed to suit the best welding if required.

Cont.

AMBALASCIENCEMARKET.COM- VIKRAM MITTAL /M-9812204116	www.modernkitchenequipments.com

81

# CHAMP MULTI 400, 600

The world class Inverter based Multipurpose Welder

TECHNICAL SPECIFICATIONS OF WATER COOLING UNIT – TYPE WCU 302

Cont.

| Sr. No. | Parameter | Unit | Value |
| --- | --- | --- | --- |
| 1 | Input Supply | V AC | 240 |
| 2 | Phase | No. | 1 |
| 3 | Frequency | Hz | 50 |
| 4 | Cooling Capacity | W | 750-1000 |
| 5 | Maximum Motor Power | W | 220 |
| 6 | Maximum Pump Pressure | Bar | 2.5 |
| 7 | Maximum Pump Flow Rate | L/min | 10 |
| 8 | Tank Capacity | L | 8 |
| 9 | Connector Size | In/out | 1/4 ² X 9 |
| 10 | Dimensions L X W X H | mm | 560 X 325 X 285 |
| 11 | Weight | Kg. | 18 |

### TECHNICAL SPECIFICATIONS OF WIRE FEEDER – TYPE FEEDLITE 40 NEM(C) / NEH(C)

| Sr. No. | Parameter | Unit | Value |
| --- | --- | --- | --- |
| 1 | Suitable for Wire Sizes | mm |  |
|  | Steel |  | 0.8, 1.0, 1.2, 1.6 |
|  | FCW |  | 1.2, 1.6 |
|  | Aluminium |  | 1.2, 1.6 |
| 2 | Wire Feed Speed | m / min | 0 - 18 |
| 3 | Wire Roll Drive | Type | 4 Roll |
| 4 | Wire Drive Motor | Type | PMDC, 42 V, 80 W |
| 5 | Dimensions L X W X H | mm | 563 X 230 X 410 |
| 6 | Weight | Kg. | 16 |

  - Warranty : One year from the date of commissioning. ADOR WELDING LIMITED warrants that all new equipment sold from Plant/Area Offices / Authorised Distributors are free from defects in materials and workmanship and will perform in full accordance with applicable specifications.

  - In view of continuous development, ADOR WELDING LIMITED reserves the right to modify/change the design and /or the specifications without any prior notice.

  - Backed by dedicated customer care package.

AMBALASCIENCEMARKET.COM- VIKRAM MITTAL /M-9812204116	www.modernkitchenequipments.com

82

# CHAMP PULSE 500

The New Indigenous World Class Inverter Based MULTIPROCESS PULSED MIG Welder

CC/CV

### INTRODUCTION:

ADOR’s CHAMP PULSE 500 outfit is an indigenously developed inverter based system with advanced IGBT protection mechanism. Outfit supports different welding process modes: SMAW, GTAW, GMAW, PULSE MIG with single and double pulse mode along with preprogrammed synergic data. Front panel comes with G-LCD and digital encoder which makes it user friendly. Built in various protection features allows it to work in the harsh environment. Machine can be used for manual as well as automatic welding application. With its different welding modes, the machine is suitable for welding with different materials like MS / SS / AL / FCAW / CORTEN STEEL / Al+Mg / Al+Si / DCu. This complete system comes with Power Source, Wire Feeder, Water Cooling unit and Water Cooled MIG torch.

### SALIENT FEATURES:

- Inverter based indigenous advanced digitally controlled SMAW, GTAW, GMAW & SINGLE/TWIN PULSE MIG welding outfit

- Synergic mode of operation for single point control that allows Automatic parameter selection (Synergic) in MMA, TIG, MIG and Pulse MIG mode

- Advanced digital control algorithms enable superior arc characteristics

- Digital control of inverter for spatter less MIG welding application

- Fine Arc length control in Pulse Mig and MIG welding mode for different types of welding application

- Excellent arc force control in MMA mode for low current application

- Twin pulse mode in Pulse Mig with low heat input makes it possible to weld the thin plates

- Digital pulse feedback of motor from wire feeding motor for accurate control of wire feeding, even at lowspeed

- With built in Water-on-Demand and VRD functionality for energy saving

- Advanced IGBT (VCE (sat)) fault detection mechanism for enhancing reliability of machine

- Power source with Graphical-LCD for displaying machine’s settings along with dual 7 segments LED display for actual current and voltage display

- Digital Encoder and switches for setting the machine

- Wire Feeder with digital console for remote parameter setting

- Facility to store 10 welding programs (weld parameter) for easy save and recall operation

- Provision to measure the Welding Arc on Time for productivity measurement

- Available feature like settable pre-flow, Burn-back, post-flow & adjustment of welding voltage & wire speed / Current in MIG / Pulsed MIG welding mode

- CRATER OFF (2T), CRATER ON (4T) mode of operation in MIG & Pulse MIG

- Details of Synergic Mode

- In MMA welding, Automatic welding current (Synergic) setting is done by selection of Type of welding material (MS/SS/CI), Diameter of Electrode (2.5/ 3.2/4.0/5.0), thickness of welding plate (1.6/ 2.4/3.2/4.8/6.4/12.8) and Type of Electrode (E7018, E6013, E6010, E308L, NICL)

Cont.

AMBALASCIENCEMARKET.COM- VIKRAM MITTAL /M-9812204116	www.modernkitchenequipments.com

83

# CHAMP PULSE 500

The New Indigenous World Class Inverter Based MULTIPROCESS PULSED MIG Welder

Cont.

- In TIG welding, Automatic welding current(Synergic) setting is done by selection of Type of welding material (SS/DCu), Welding plate thickness (2.5,3.2,4.0,5.0) & Type of welding joint (Butt/Lap/Corner/Fillet).

- In MIG welding, Automatic welding current (Synergic) setting is done by selection of welding material (MS/SS/ALU), Diameter of welding wire (0.8/1.0/1.2/1.6) & welding gas (Arco2/Aro2/Co2 /Ar).

- In PULSEMIG welding, Automatic welding current (Synergic) setting is done by selection of type of welding material (MS/Corten/FCAW/SS19/SS/ Al+SI / Al+Mg), diameter of welding wire (0.8/1.0/1.2/1.6) & welding gas (ArCO2/Co2/ArO2/Ar).

PROTECTION:

##### The equipment is provided with following protections.

- Under Voltage and Over Voltage: If supply voltage goes lower or higher than set limit, RED colour Trip LED will glow on the front panel and display will show the corresponding error.

- Over Temperature: If the temperature of the main power elements is more than safety limits, RED Colour Trip LED will glow on the front panel, display will show the corresponding error.

- Single Phasing Protection: If any one of the three phases (R, Y, and B) is absent, welding will stop.

Welding current would not be available in this condition; RED colour Trip LED will glow on the front panel, display will show the corresponding error.

- IGBT VCE-Sat Protection: This protection mechanism continuously monitors the IGBT’s VCE voltage and if this voltage goes beyond the permissible limit of IGBT due to over current through IGBT, display will show the  corresponding error.

### WELDING PARAMETER SPECIFICATIONS

| Mma Welding Parameter Specifications |  |  |
| --- | --- | --- |
| Parameter | UNIT | VALUE |
| Welding Current | AMP | 50 - 500 |
| Arc Force | % | 0 - 100 |

| Tig Welding Parameter Specifications |  |  |
| --- | --- | --- |
| Parameter | UNIT | VALUE |
| Welding Current | AMP | 20 - 500 |

| Mig Welding Parameter Specifications |  |  |
| --- | --- | --- |
| Parameter | UNIT | VALUE |
| Gas Pre Flow Time | SEC. | 0 – 10 |
| ARC Length | % | 0 – 40 |
| Welding Current | AMP. | 50 – 500 |
| Welding Voltage | VOLT, DC | 14.0 – 46.0 |
| Crater Current | AMP | 50 - 500 |

Cont.

AMBALASCIENCEMARKET.COM- VIKRAM MITTAL /M-9812204116	www.modernkitchenequipments.com

84

# CHAMP PULSE 500

The New Indigenous World Class Inverter Based MULTIPROCESS PULSED MIG Welder

Cont.

| Crater Voltage | Volt, DC | 14.0 – 46.0 |
| --- | --- | --- |
| Burn - Back Time | Sec. | 0.1 - 2.0 |
| Gas Post Flow Time | Sec. | 0 – 10 |

| Pulsed Mig Welding Parameter Specifications |  |  |
| --- | --- | --- |
| Parameter | Unit | Value |
| Gas Pre-flow Time | Sec | 0 - 10 |
| Welding Current | AMP | 40 - 500 |
| Welding Voltage | Volt, DC | 14.0 – 46.0 |
| Arc Length | % | - 40 TO + 40 |
| Crater Current | AMP | 40 - 500 |
| Crater Voltage | VOLT, DC | 14.0 – 46.0 |
| Burn - Back Time | SEC. | 0.1 - 2.0 |
| Gas Post Flow Time | SEC. | 0 – 10. |
| Twin Pulse Frequency | HZ | 1.0 – 10.0 |
| Twin Pulse Duty Cycle | % | 20 - 80 |
| Twin Pulse Current Ratio | % | 10 - 50 |

EQUIPMENT TECHNICAL SPECIFICATIONS

| Power Source | Model | Champ Pulse 500 |
| --- | --- | --- |
| Parameter | UNIT | Value |
| Input |  |  |
| Supply Voltage, Phase, Frequency | VOLT, AC | 415 V +15%, -10%, 3 Phase, 50 / 60 Hz |
| Max. Input Kva @ 415 V Supply |  |  |
| @ 100% DUTY CYCLE | KVA | Mma / Tig / Pulsed Mig Modes - 20, Mig Mode - 17.5 |
| @ 60% DUTY CYCLE | KVA | Mma / Tig / Pulsed Mig Modes - 30, Mig Mode - 25.5 |
| Input Current @ 415 V Supply |  |  |
| @ 100% DUTY CYCLE | AMP, AC | Mma / Tig / Pulsed Mig Modes - 28, Mig Mode - 24 |
| @ 60% DUTY CYCLE | AMP, AC | Mma / Tig / Pulsed Mig Modes - 41, Mig Mode - 35 |
| Power Factor |  | 0.8 Maximum |
| Efficiency | % | Upto 85 |

Cont.

AMBALASCIENCEMARKET.COM- VIKRAM MITTAL /M-9812204116	www.modernkitchenequipments.com

85

# CHAMP PULSE 500

The New Indigenous World Class Inverter Based MULTIPROCESS PULSED MIG Welder

Cont.

| Output |  |  |
| --- | --- | --- |
| Open Circuit Voltage @ 415v Input Supply | Volt, DC | MMA / TIG / Pulsed Mig Modes - 98, MIG Mode - 65 |
| Welding Current Range | Amp, DC | MMA Mode 50 - 500, TIG Mode 20 - 500, MIG Mode 50 - 500, Pulsed MIG Mode 40 - 500 |
| Welding Current At 40 Deg C, 10 Minute Cycle |  |  |
| @ 100% Duty Cycle | Amp, DC | 387 |
| @ 60% Duty Cycle | Amp, DC | 500 |
| General |  |  |
| Wire Feeder (for Mig / Pulsed Mig Welding) | 5 Meter Interconnection | Provided With Digital Front Panel Control For Setting Wire Speed / Current And Voltage/ Arc Length |
| Protections | - | Over Voltage, Under Voltage, Single- Phasing, Over Temperature, Over Current |
| Front Panel Functions |  | - MMA/ TIG / MIG / Pulsed MIG Selection Switch - 2T/ 4T Selection Switch - Gas Check / Wire Inch / Ocv Check Switches - Material / Wire Diameter / Shielding Gas Selection Switches - Gas Cooling / Watr Cooling Selection Switch - Wire Speed / Current / Plate Thickness Selection Switch - Up / Down Switches Are Provided To Set The Gas Pre-flow, Gas Post- flow & Burn Back Times |

Cont.

AMBALASCIENCEMARKET.COM- VIKRAM MITTAL /M-9812204116	www.modernkitchenequipments.com

86

# CHAMP PULSE 500

The New Indigenous World Class Inverter Based MULTIPROCESS PULSED MIG Welder

Cont.

| Front Panel Functions |  | - 3 Separate Encoders Are Provided To Set Respectively Current, Volrage And Additional Parameters - Connectors For Wire Feeder And Remote Control - Output Terminals (stud Type) - Mains On - Green Led Indicator - Trip - Red Led Indicator |
| --- | --- | --- |
| Program Storage Facility |  | 13. Jobs Along With Parameter Locking Facility |
| Auxiliary Power Supply For Water Cooling Unit | VAC, VA | 240 V, 300 VA |
| Cooling | TYPE | Forced Air |
| Ambient Temperature Rating | ºC | 40 |
| Class Of Insulation | - | H |
| Degree Of Protection | - | IP23S |
| Compatibility To International Standards |  |  |
|  |  | AS PER EN 60974 -10 (EMI / EMC Standard) |
| Dimensions L X W X H (without Wire Feeder At Top) | MM | 1140 X 475 X 1130 |
| Weight (approx Without Wire Feeder.) | KG | 105 |
| Wire Feeder | Model | Feedlite 40 (nepm - 5) |
| Parameter | Unit | Value |
| Weight (without Spool, And With 5 Meter Interconnection) | Kg | 20 KG (APPROX) |
| Dimensions (l X W X H) | mm | 535 X 255 X 410 |
| Suitable For Wire Spool Capacity | Kg | 15 KG |
| Wire Spool Diameter | mm | 300 |
| Wire Feeder Motor Voltage | Volts, DC | 42 V DC |
| Wire Drive Motor. | Type | Permanent Magnet Dc Type With Encoder Feedback |

Cont.

AMBALASCIENCEMARKET.COM- VIKRAM MITTAL /M-9812204116	www.modernkitchenequipments.com

87

# CHAMP PULSE 500

The New Indigenous World Class Inverter Based MULTIPROCESS PULSED MIG Welder

Cont.

| Rated Power | Watt | 80 |
| --- | --- | --- |
| Wire Roll Drive | No | Four |
| Wire Feed Speed | Meters / Minute | 2 - 22 |
| Suitable For Wire Sizes | mm | Solid Wire - 0.8,1,1.2,1.6; Fcw - 1.2, 1.6; Aluminium - 1.2, 1.6 |
| Torch End Adatper | Type | Euro |
| Front Panel |  | 7dual -segment Display And Encoder For Setting Parameters |
| Water Cooling Unit | Model | WCU - 302 |
| Parameter | Unit | Value |
| Supply Voltage, Phase, Frequency | Volt, AC | 230 V, 1 PHASE, 50 / 60 HZ |
| Cooling Capacity | W | 750-1000 |
| Max Motor Power | W | 180 |
| Max Pump Pressure | BAR | 3.2 |
| Max Pump Flow Rate | L/Min | 10 |
| Tank Capacity | L | 8 |
| Connector Size | In/Out | 1/4 ² BSP, 19TPI |
| Dimensions (l X W X H) | mm | 540 X 315 X 255 |
| Weight(approx.) | Kg | 14 |
| Mig Welding Torch | Model | Ador Tw 600 (we) |
| Parameter | Unit | Value |
| Maximum Current @ 60% Duty Cycle | Amp | 550 (with Co2 Gas); 500 (with Mixed Gas) |
| Torch Cable Length | METERS | 3 |
| Torch Cable Type | - | Co-axial |
| Torch Adapter | TYPE | EURO |
| Cooling | TYPE | Water |
| Suitable For Wire Size Diameter | MM | 0.8, 1.0, 1.2, 1.6 |

  - Warranty : One year from the date of commissioning. ADOR WELDING LIMITED warrants that all new equipment sold from Plant/Area Offices / Authorised Distributors are free from defects in materials and workmanship and will perform in full accordance with applicable specifications.

  - In view of continuous development, ADOR WELDING LIMITED reserves the right to modify/change the design and /or the specifications without any prior notice.

  - Backed by dedicated customer care package.

AMBALASCIENCEMARKET.COM- VIKRAM MITTAL /M-9812204116	www.modernkitchenequipments.com

88

# HF 2000 / HF 2000 AD & HF 3000 / HF 3000 AD

The high efficiency, low power consumptionWorld Class DC and AC/DC TIG control units

CC

DC TIG	AC TIG

### TYPICAL APPLICATIONS

  - Light Weight, Compact & Portable for greater flexibility in operation

  - Suitable for TIG Welding application with SMAW Power Source of any make

### SALIENT FEATURES

  - Portable & Light Weight

  - Built in high frequency unit for easy arc striking in AC/DC TIG welding process

  - HF on/off switch to prevent high frequency interference (In DC mode only)

  - Gas flow through solenoid valve for Economical consump[tion of costly Agron gas

  - Gas Preflow / Postflow facility

  - Gas flow through solenoid valve for Economical consumption of costly Argon gas

  - Auto HF cut off if arc does not strike within 10 seconds

  - Built in gas pre-flow / post-flow facility

  - In built water cooling unit with HF 3000/	HF 3000AD

### SPECIFICATIONS

| Technical Specifications |  |  | Unit | HF 2000 | HF 2000 AD | HF 3000 | HF 3000 AD |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Input |  |  |  | Value | Value | Value | Value |
| Input Supply : |  |  |  |  |  |  |  |
| Voltage |  |  | Volt | 230 | 230 | 230 | 230 |
| Phase |  |  | No | 1 | 1 | 1 | 1 |
| Frequency |  |  | Hz | 50 | 50 | 50 | 50 |
| Rating |  |  |  |  |  |  |  |
| Welding Current DC |  |  |  |  |  |  |  |
| @ 60% Duty Cycle |  |  | Amps | 200 | 200 | 300 | 300 |
| @ 100% Duty Cycle |  |  | Amps | 150 | 150 | 230 | 230 |

AMBALASCIENCEMARKET.COM- VIKRAM MITTAL /M-9812204116	www.modernkitchenequipments.com

89

# HF 2000 / HF 2000 AD & HF 3000 / HF 3000 AD

The high efficiency, low power consumptionWorld Class DC and AC/DC TIG control units

| Technical Specifications |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Model |  |  | Unit | HF 2000 | HF 2000 AD | HF 3000 | HF 3000 AD |
| Input |  |  | Unit |  |  |  |  |
| I/P Supply Voltage |  |  | V | 230 | 230 | 230 | 230 |
| Phase |  |  | No | 1 | 1 | 1 | 1 |
| Frequency |  |  | Hz | 50 | 50 | 50 | 50 |
| Rating |  |  |  |  |  |  |  |
| Welding Current DC |  |  |  |  |  |  |  |
| @ 60% Duty Cycle |  |  | Amps | 200 | 200 | 300 | 300 |
| @ 100% Duty Cycle |  |  | Amps | 150 | 150 | 230 | 230 |
| Welding Current AC |  |  |  |  |  |  |  |
| @ 60% Duty Cycle |  |  | Amps | - | 150 | - | 250 |
| @ 100% Duty Cycle |  |  | Amps | - | 110 | - | 180 |
| Gas Preflow Time |  |  | Sec. | 0.1-5 | 0.1-5 | 0.1-5 | 0.1-5 |
| Gas Postflow Time |  |  | Sec. | 1-30 | 1-30 | 1-30 | 1-30 |
| General |  |  |  |  |  |  |  |
| Water Tank Capacity |  |  | Liter | - | - | 8 | 8 |
| Protection Class |  |  | Class | IP23 | IP23 | IP23 | IP23 |
| Mode of Operation |  |  | Type | 2T / 4T | 2T / 4T | 2T / 4T | 2T / 4T |
| Arc Striking |  |  | Type | HF/Touch | HF/Touch(DC) | HF/Touch | HF/Touch(DC) |
| Dimensions |  |  |  |  | DC | DC |  |
| Length |  |  | mm | 440 | 560 | 614 | 614 |
| Width |  |  | mm | 165 | 290 | 400 | 400 |
| Heigh |  |  | mm | 315 | 438 | 580 | 880 |
| Weight |  |  | Kg | 10 | 25 | 46(Without water)50 |  |
| Torch Model |  |  | Class | IP23 | IP23 | IP23 | IP23 |
| Max. Current Carrying Capacity |  |  |  | HI-PRO TIG 201-4/201-8 |  | HI-PRO TIG 301-4/301-8 |  |
| @ 60% DC |  |  | Amps DC/AC | 200 / 150 |  | 300 / 250 |  |
| @ 100 % DC |  |  | Amps DC/AC | 150 / 115 |  | 230 / 180 |  |
| Length |  |  | Mtrs. | 4 / 8 |  | 4 / 8 |  |
| Cooling |  |  | Type | Gas |  | Water |  |
| Electrode Capacity |  |  |  | 1.6, 2.4 & 3.2 |  | 1.6, 2.4, 3.2 & 4.0 |  |

  - Warranty : One year from the date of commissioning. ADOR WELDING LIMITED warrants that all new equipment sold from Plant/Area Offices / Authorised Distributors are free from defects in materials and workmanship and will perform in full accordance with applicable specifications.

  - In view of continuous development, ADOR WELDING LIMITED reserves the right to modify/change the design and /or the specifications without any prior notice.

  - Backed by dedicated customer care package.

AMBALASCIENCEMARKET.COM- VIKRAM MITTAL /M-9812204116	www.modernkitchenequipments.com

90

# CHAMPTIG 220P

The World Class Inverter based DC TIG and DC Pulsed TIG Welder

CC

DC TIG

### SALIENT FEATURES

  - Single phase (240 V AC) inverter based, high efficiency and high power factor pulse TIG/ MMA DC welder

  - Useful for wide variety of material types and thickness.

  - Full featured TIG controls possible

  - HF ignition

  - Intelligent protection: over voltage, line (415V), over Current, temperature

  - Latest PWM inverter technology

  - High efficiency (Up to 87%)

### CONTROL PANEL FEATURES

| Control Panel Features CHAMPTIG 220P |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| CHAMPTIG 220P Control Panel Operations |  |  |  |  |  |
| Item | Symbol |  | Functionality / Description |  | Mode of Operation |
|  |  |  | A | SET CURRENT AND ACTUAL WELDING CURRENT DURING WELDING | MMA-TIG |
| 3 Digit Seven Segment Display |  |  | DISPLAYS PREFLOW, POST FLOW, UP- SLOPE AND DOWN-SLOPE TIME DURING SEC PARAMETER SETTING WITH MENU KEY |  | TIG |
|  |  |  | HZ | Frequency of Pulsing in Hz |  |
|  | A | S E C H% % |  |  | TIG |
|  |  |  | % | DISPLAYS HOT START AND ARC FORCE PERCENTAGE DURING PARAMETER SETTING WITH MENU KEY | MMA |

Cont.

AMBALASCIENCEMARKET.COM- VIKRAM MITTAL /M-9812204116	www.modernkitchenequipments.com

91

# CHAMPTIG 220P

The World Class Inverter based DC TIG and DC Pulsed TIG Welder

Cont.

| Item | Symbol | Functionality / Description | Mode of Operation |
| --- | --- | --- | --- |
| Power Supply On Indicator |  | MAINS ON MAINS ON | MMA-TIG |
| Machine Trip Indication |  | TRIP MACHINE TRIP | MMA-TIG |
| MMA/TIG HF/LIFT ARC Selection | MIMA Prog TIG-HF LIFT ARC | MMA/TIG HF/ LIFT ARC Selection Through Prog Button (Visual Indications Are Proved For The Same) | MMA-TIG |
| Panel Encoder |  | Setting The Welding Current 'A' In MMA Mode & Also The Different Parameters In TIG Mode Clockwise Rotation =>Increase, Anticlockwise Rotation =>Decrease | MMA-TIG |
| Parameter Setting In MMA Mode | Menu HOT A START ARC MMA FORCE | MMA Mode Hot Start, Current Set & ARC MMA Force Mode. Visual Indications Are Provide For The Same | MMA-TIG |
| Parameter Setting In TIG Mode | Ip Is k Menu | TIG Mode Set Pre Flow Time, Starting Current, Unslope Time, Peak Current, Post Flow Time Respectively. Visual Indications Are Provide For The Same | TIG |
| Self-Hold On-Off Selection in TIG Mode (2 Track/ 4Track Operation) | 2T MPreongu 4T | TIG Mode Set 2T/4T Modes Of Operation.Visual Indications are Provided. DisabledIn MMA Mode. | TIG |
| Menu Key | Menu | Menu Key Is Used to Set the Different Parameters In MMA Mode & TIG Mode of Operation in Different Combinations Of 2T, 4T in TIG Mode & Hot Start & ARC Forced In MMA Mode | MMA-TIG |
| Normal Pulse Selction | Tig PULSE Mode NORMAL | TIG Mode Select The Modes Pulse/ Normal, Visual Indications Are Provided For The Same. Disable in MMA Mode. | TIG |

Cont.

AMBALASCIENCEMARKET.COM- VIKRAM MITTAL /M-9812204116	www.modernkitchenequipments.com

92

# CHAMPTIG 220P

CC

The World Class Inverter based DC TIG and DC Pulsed TIG Welder

Cont.

CONTROL PANEL FEATURES

DC TIG

Control Panel Features CHAMPTIG 220P

CHAMPTIG 220P Control Panel Operations

| Parameter | Unit | Value |  | Parameter | Unit | Value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | MMA Mode | TIG Mode |  |  | MMA Mode | TIG Mode |
| Hot Start | % | 0-100% of Set Current | NA | Hot Start | % | 0-100% of Set Current | NA |
| Welding Current Set | A | 10-200 | NA | Welding Current Set | A | 10-200 | NA |
| ARC Force | % | 0-100% | NA | ARC Force | % | 0-100% | NA |
| Gas Pre Flow Time | Sec | NA | 0-5 | Gas Pre Flow Time | Sec | NA | 0-5 |
| Start Current (IS) | A | NA | 5-220 | Start Current (IS) | A | NA | 5-220 |
| Current Unslope Time | Sec | NA | 0-10 | Current Unslope Time | Sec | NA | 0-10 |
| Peck Current (IP) | A | NA | 5-220 | Peck Current (IP) | A | NA | 5-220 |
| Current Down Slope Time | Sec | NA | 0-10 | Base Current (IB) | A | NA | 5-220 |
|  |  |  |  | Pulse Fervency | Hz | NA | 1-200 |
| Crater Current (IC) | A | NA | 5-220 | Duty Cycle (TP) | % | NA | 10-90 |
|  |  |  |  | Current Down Slope Time | Sec | NA | 0-10 |
| Gas Post Flow Time | Sec | NA | 0-20 | Crater Current (IC) | A | NA | 5-220 |
|  |  |  |  | Gas Post Flow Time | Sec | NA | 0-20 |

AMBALASCIENCEMARKET.COM- VIKRAM MITTAL /M-9812204116	www.modernkitchenequipments.com

93

# CHAMPTIG 220P

The World Class Inverter based DC TIG and DC Pulsed TIG Welder

Cont.

TECHNICAL SPECIFICATIONS

CC

DC TIG

| Parameter | Unit |  |  |
| --- | --- | --- | --- |
|  |  | CHAMPTIG 220P |  |
| Input |  |  |  |
| Supply Voltage, Phase Frequency : | Volts AC | 240 V +10%-15%, 1 Phase, 50/60 Hz |  |
| Max output Input KVA @ 240 V Supply | KVA | MMA Mode | TIG Mode |
| @ 100% duty cycle |  | 5.6 | 4 |
| @ 60% duty cycle |  | 6.7 | 5.5 |
| @ 25% duty cycle |  | 9.80 | 7.80 |
| Efficiency | % | Upto 87 |  |
| Output |  |  |  |
| Open Circuit Voltage | Volts DC | 78 V DC (+/- 5V) |  |
| Welding Current Range | Amps DC | MMA Mode | TIG Mode |
|  |  | 10-200 | 5-220 |
| Welding Current (40°C 10 minutes cycle) | Amps DC |  |  |
| @ 100% duty cycle |  | 120 | 130 |
| @ 60% duty cycle |  | 150 | 170 |
| @ 25% duty cycle |  | 200 | 220 |
| General |  |  |  |
| Suitable for Welding Electrode Size | mm | 2.5,3.2 Intermittent 4 |  |
| Ingress Protection | Type | IP23 |  |
| Cooling | Type | Forced Air |  |
| Insulation | Class | F |  |
| Front Panel Functions |  | 1. Menu Switch For Selections Like Gas Preplow, Gas Post Flow, Unslope Time, Downslope Time, Welding Current As Per Selected Mode Operation ( TIG Mode only) |  |
|  |  | 2. MMA/ TIG HF/ LIFT ARC Mode Selection |  |
|  |  | 3. Menu Switch For Selection of Hot Start, Welding Current & ARC Force ( In MMA mode only) |  |
|  |  | 4. Mains On 'Green' Color Indication |  |
|  |  | 5. Trip 'Red' Color LED for indication Machine is Under Protection Mode |  |
|  |  | 6. Encoder for Selected Parameter Vaule Increment/Decrement. |  |
|  |  | 7. 2T/4T Mode Selection (In TIG Mode only) |  |

Cont.

AMBALASCIENCEMARKET.COM- VIKRAM MITTAL /M-9812204116	www.modernkitchenequipments.com

94

# CHAMPTIG 220P

CC

The World Class Inverter based DC TIG and DC Pulsed TIG Welder

Cont.

DC TIG

| Additional Functions Specific to CHAMPTIG 220P only |  | 8. Pulse Normal Mode Selection Switch Base Current, Frequency & Duty Cycle Selection By Menu Switch |
| --- | --- | --- |
| Protections |  | Over Voltage, Under Voltage, Single Phasing, Over Temperature |
| Dimensions L x W x H (Without handle) | mm | 480 x 140 x 255 |
| Weight (Approx) | Kg | 9 |

  - Warranty : One year from the date of commissioning. ADOR WELDING LIMITED warrants that all new equipment sold from Plant/Area Offices / Authorised Distributors are free from defects in materials and workmanship and will perform in full accordance with applicable specifications.

  - In view of continuous development, ADOR WELDING LIMITED reserves the right to modify/change the design and /or the specifications without any prior notice.

  - Backed by dedicated customer care package.

AMBALASCIENCEMARKET.COM- VIKRAM MITTAL /M-9812204116	www.modernkitchenequipments.com

95

# CHAMPTIG 300P

The World Class Inverter based DC TIG and DC Pulsed TIG Welder

CC

DC TIG

### SALIENT FEATURES

  - Three phase inverter based, High efficiency and High Power Factor Pulse TIG/ MMA DC Welder

  - Full featured TIG controls possible

  - Power source with built in HF ignition

  - Intelligent protection: over/under voltage, over current / temperature

  - Option of water cooled Torch with water cooling unit available

  - Smooth and stable arc with spatter less welding

  - HF start ignition

  - Pulse TIG for precision welding

### SPECIFICATIONS

| Technical Specifications |  |  | Unit | CHAMPTIG 300P |  |
| --- | --- | --- | --- | --- | --- |
| Input |  |  |  |  |  |
| Input Supply : |  |  |  |  |  |
| Voltage |  |  | Volt | 415 , +15%, -10% |  |
| Phase |  |  | No | 3 |  |
| Frequency |  |  | Hz | 50/60 |  |
|  |  |  |  | MMA Mode | TIG Mode |
| Efficiency @ 100% duty cycle |  |  | % | 82 | 77 |
| Power Factor @ 100% duty cycle |  |  | OSØ | 0.93 | 0.93 |
| Input KVA |  |  |  |  |  |
| @ 100% duty cycle |  |  | KVA | 7.5 | 7 |
| @ No Load |  |  | KVA | 0.19 | 0.19 |
| Output |  |  |  |  |  |
| Open Circuit Voltage |  |  | Volt | 70 |  |
| Welding Current Range |  |  |  | MMA Mode | TIG Mode |
| Welding Current |  |  | Amps | 50-250 | 5-300 |
| @ 100% duty cycle |  |  | Amps | 195 | 230 |
| @ 60% duty cycle (10 min cycle) |  |  | Amps | 250 | 300 |

Cont.

AMBALASCIENCEMARKET.COM- VIKRAM MITTAL /M-9812204116	www.modernkitchenequipments.com

96

# CHAMPTIG 300P

The World Class Inverter based DC TIG and DC Pulsed TIG Welder

Cont.

| Pulse TIG Parameters |  |  |  |  |
| --- | --- | --- | --- | --- |
| Duty Ratio |  |  | % | 10-90 |
| Pulse Frequency |  |  | Hz | 0.5-10 |
| Base Current |  |  | Amps | 5 - 90% of Ipulse current |
| Pulse Current ( Ipulse) |  |  | Amps | 5-300 |
| General |  |  |  |  |
| Start current |  |  | Amps | 5-300 |
| Current unpslope time |  |  | Sec | 0-10 |
| Current down slope control |  |  | Sec | 0-10 |
| Pre-Flow |  |  | Sec | 0-5 |
| Post-Flow |  |  | Sec | 1-20 |
| Crater current |  |  | Amps | 5-300 |
| Current / Voltage Display |  |  |  | 7 –Segment LED Display for Current & Other parameters |
| Ingress Protection |  |  | Class | IP23 |
| Cooling |  |  | Type | Forced Air |
| Insulation |  |  | Class | H |
| Welding Output Terminals |  |  | - | Cam Lock connections |
| Dimensions |  |  |  |  |
| Length |  |  | mm | 610 |
| Width |  |  | mm | 295 |
| Height |  |  | mm | 480 |
| Weight (approx.) |  |  | Kg | 38 |
| Water Cooling Unit |  |  |  |  |
| Input Supply Voltage (AC) |  |  | Volts | 230 |
| Phase |  |  | - | 1 |
| Frequency |  |  | Hz | 50-60 |
| Water Reservoir Capacity |  |  | Liters | 8 |
| Flow rate |  |  | Ltr/min | 10 |
| Dimensions |  |  |  |  |
| Length |  |  | mm | 560 |
| Width |  |  | mm | 325 |
| Height |  |  | mm | 285 |
| Weight (approx.) |  |  | Kg | 18 |
| Trolley |  |  |  |  |
| Dimensions |  |  |  |  |
| Length |  |  | mm | 660 |
| Width |  |  | mm | 525 |
| Height |  |  | mm | 415 |
| Weight (approx.) |  |  | Kg | 10 |

  - Warranty : One year from the date of commissioning. ADOR WELDING LIMITED warrants that all new equipment sold from Plant/Area Offices / Authorised Distributors are free from defects in materials and workmanship and will perform in full accordance with applicable specifications.

  - In view of continuous development, ADOR WELDING LIMITED reserves the right to modify/change the design and /or the specifications without any prior notice.

  - Backed by dedicated customer care package.

AMBALASCIENCEMARKET.COM- VIKRAM MITTAL /M-9812204116	www.modernkitchenequipments.com

97

# CHAMPTIG 300 SP

Synergic DC TIG and DC Pulsed TIG Welder with Intelligent Control

CC

DC TIG

### TYPICAL APPLICATION

  - Latest Inverter based technology.

  - IGBT based operating at high frequency.

  - Suitable for both DC TIG (Pulse & Normal) and MMA welding operation.

  - Available with Synergic mode in TIG for pulse/ normal mode. Also suitable for scratch start in TIG mode.

  - Built in HF ignition for easy arc striking.

### SPECIFICATIONS

  - User friendly front panel interface with LCD display and Multi-functional encoder.

  - Anti-stick function to protect the machine from short circuit condition in MMA mode.

  - Option of Remote control unit with digital display, for setting current remotely in MMA and TIG mode

  - Option of Foot switch control regulator for setting current in TIG mode

| Technical Specifications |  |  | Unit | ICVR 250 |
| --- | --- | --- | --- | --- |
| Input |  |  |  |  |
| Input Supply : |  |  |  |  |
| Voltage |  |  | Volts, Ac | 415, +15%, -10% |
| Phase |  |  |  | 3 |
| Frequency |  |  | HZ | 50/ 60 |
| Characteristics |  |  |  | CC - CV |
| Input KVA @ 100% duty cycle |  |  | KVA | MMA Mode-10.3, TIG Mode-7.1 |
| Input KVA @ 60% duty cycle |  |  | KVA | MMA Mode-13.1, TIG Mode-9.5 |
| Input KVA @ 35% duty cycle |  |  | KVA | MMA Mode-16.9, TIG Mode-11.9 |
| Input Current 415V Supply @ 100% duty cycle |  |  | Amps, Ac | MMA Mode-14.4, TIG Mode-9.8 |
| Input Current 415V Supply @ 60% duty cycle |  |  | Amps, Ac | MMA Mode-18.3, TIG Mode-13.2 |
| Input Current 415V Supply @ 35% duty cycle |  |  | Amps, Ac | MMA Mode-23.5, TIG Mode-16.5 |
| Power Factor |  |  |  | Up to 0.75 |
| Efficiency |  |  | % | Up to 84 |
| Output |  |  |  |  |
| Open Circuit Voltage @415V Input Supply |  |  | Volts, DC | 80 V DC (+/-5V) |
| Welding Current Range |  |  | Amps, DC | MMA Mode-50-300, TIG Mode-10-300 |
| Welding Current (40°C 10 minutes cycle) |  |  |  |  |
| @ 100% duty cycle |  |  | Amps, DC | 200 |
| @ 60% duty cycle |  |  | Amps, DC | 250 |
| @ 35% duty cycle |  |  | Amps, DC | 300 |

Cont.

AMBALASCIENCEMARKET.COM- VIKRAM MITTAL /M-9812204116	www.modernkitchenequipments.com

98

# CHAMPTIG 300 SP

Synergic DC TIG and DC Pulsed TIG Welder with Intelligent Control

Cont.

| General | Unit | Value |
| --- | --- | --- |
| Remote Controller | 10 Meter | Provide as optional for Current Setting |
| Foot Switch Control | 5 Meter | Provide as optional for Current Setting |
| Welding Electrode Size (MMA Mode) | Dia in mm | 2.5,3.15,4 |
| Protection |  | Over Voltage, Under Voltage, Single Phasing, Over Temeprature |
| Front Panel Functions |  | - MMA/HF TIG/ LIFT ARC TIG selection switch - 2T/4T/Spot/Cycle Switch - Pulse/Normal Mode selection - Normal/Foot switch selection switch - Menu switch for selecting all functions for pulse - Gas pre flow, Gas post flow, pulse frequency, current unslope time current downslope time, welding current (peak current for pulsing mode), Background current (for pulsing mode), set current time - as per selected mode of operation - Mains on ‘Green’ color indication - Trip ‘Red’ color LED for indication of machine is under protection mode - Encoder for selection parameter Value Increment / Decrement - Auto key making synergic mode on/off - Para/Gas switch to set the parameters in synergic mode & to check gas in normal mode - Remote controller - Gas Out - Water/ Gas cooled selection switch - Torch switch connector - Foot switch connector - Cam look output connector - 10 sets of parameter setting can be saved & recalled |

Cont.

AMBALASCIENCEMARKET.COM- VIKRAM MITTAL /M-9812204116	www.modernkitchenequipments.com

99

# CHAMPTIG 300 SP

Synergic DC TIG and DC Pulsed TIG Welder with Intelligent Control

Cont.

| Auxiliary power supply for water cooling unit | VAC, W | 240V, 300W |
| --- | --- | --- |
| Cooling | Type | Forced Air |
| Ambient Temperature rating | °C | 40 |
| Class of Insulation | - | H |
| Degree of Protection | - | IP23S |
| Dimensions (L x W x H) | mm | 600 x 215 x 470 |
| Weight (approx.) | Kg. | 27 |

DIGITAL PANEL DETAILS FOR MODEL: CHAMPTIG 300SP

Up Slope Time

Start Current

Gas Pre- Flow Time

Main Current

Cycle Current

I1

I2

Base Current

PULSE

IB

Pulse Current

IP

TP

Duty Cycle/Spot Time in spot Mode

Down Slope

Time

Final Current

Gas Post- Flow Time

CYCLE	F

Pulse Frequency

Cont.

AMBALASCIENCEMARKET.COM- VIKRAM MITTAL /M-9812204116	www.modernkitchenequipments.com

100

# CHAMPTIG 300 SP

Synergic DC TIG and DC Pulsed TIG Welder with Intelligent Control

DIGITAL PANEL DETAILS FOR MODEL: CHAMPTIG 300SP

Cont.

| Key | Legend |  | Description |
| --- | --- | --- | --- |
| MMA/TIG HF/LIFT ARC Selection Key | Prog | MIMA TIG-HF LIFT ARC | Prog key can be used to select MMA / TIG (With HF & Without HF) Process. Visual indications are provide for the same |
| Encoder |  |  | With the help of Encoder we can set the welding current ‘A’ in MMA TIG mode & also the different parameters in TIG mode. (Encoder rotation clockwise = +, I.E. Increment Encoder Rotation anticlockwise =-, I.E. Decrement |
| Pulse / Normal Selection | Tig Mode | PULSE NORMAL | This can be used TIG mode only. In TIG mode, with the help of this button can toggle between Pulse / Normal TIG mode. Visual indication are provide for the same |
| 2T / 4T / Spot / Cycle Selection Key | MPernogu | 2T 4T SPOT CYCLE | This key can be used in TIG mode to select the mode 2T / 4T / Spot / Cycle. Visual indications are provided for the same. This key is inactive in MMA mode. |
| Para / Gas Selection Key | Para Gas |  | This key can be used in TIG mode to select different parameters when synergic mode is on. This same key can be used to check Gas in TIG mode when synergic mode is off. |
| Auto Switch | Auto |  | This key can be used to enabled or disable the synegic TIG mode. Visual indiction is also provided. |
| Gas Cooled / Water Cooled Selection Switch | GAS COOLED WATER COOLED |  | This key is enabled in TIG mode only to select the cooling way depending upon the TIG Torch to be used. |
| TIG Parameter Selection Switch | Menu |  | Menu Key Is Used to Set the Different Parameters In MMA Mode & TIG Mode of Operation in Different Combinations Of 2T, 4T, Spot, Cycle with pulse & normal. Visual indiction is also provided for all TIG parameters |

Cont.

AMBALASCIENCEMARKET.COM- VIKRAM MITTAL /M-9812204116	www.modernkitchenequipments.com

101

# CHAMPTIG 300 SP

Synergic DC TIG and DC Pulsed TIG Welder with Intelligent Control

DIGITAL PANEL DETAILS FOR MODEL: CHAMPTIG 300SP

Cont.

| Key | Legend | Description |
| --- | --- | --- |
| LED Display |  | 7 - SEG LED Display shows actual Welding & Current |
| Mains on Indication | Mains On | This Green LED Indicates that Machine is ready for use. |
| Trip Indication | Trip | This Red LED Indicates that Machine is under one of the protection mode via. under Voltage / Over Voltage / Single Phase |
| Normal Foot Switch | NORMAL FOOT SWITCH | Mini Rocker switch is provided for Normal / Foot switch selection |
| Remote | REMOTE | This is 4 pin connector provided for interfacing Remote |
| Gas Out | GAS OUT | Gas Nippal is Provided on panel to interface the Gas connection of torch in TIG mode. |
| LED Display | LED Display shows the selected parameter value which can be set |  |

Cont.

AMBALASCIENCEMARKET.COM- VIKRAM MITTAL /M-9812204116	www.modernkitchenequipments.com

102

# CHAMPTIG 300 SP

Synergic DC TIG and DC Pulsed TIG Welder with Intelligent Control

SPECIFICATION OF WATER COOLING UNIT

Cont.

| Parameter | Unit | Value |
| --- | --- | --- |
| Supply Voltage, Phase, Frequency | Volts, Ac | 230 V, 1 Phase, 50 / 60 HZ |
| Cooling Capacity | W | 750 - 1000 |
| Max Motor Power | W | 220 |
| Max Pump Pressure | BAR | 2.5 |
| Max Pump Flow Rate | L | 10 |
| Tank Capacity | Min | 8 |
| Connector Size | In / Out | 1/4 ² BSP, 19TPI |
| Dimensions (L X W X H) | mm | 580 X 215 X 270 |
| Weight (Approx.) | Kg. | 14 |

- Warranty : One year from the date of commissioning. ADOR WELDING LIMITED warrants that all new equipment sold from Plant/Area Offices / Authorised Distributors are free from defects in materials and workmanship and will perform in full accordance with applicable specifications.

- In view of continuous development, ADOR WELDING LIMITED reserves the right to modify/change the design and /or the specifications without any prior notice.

- Backed by dedicated customer care package.

AMBALASCIENCEMARKET.COM- VIKRAM MITTAL /M-9812204116	www.modernkitchenequipments.com

103

# CHAMPTIG 400P

The World Class Inverter based DC TIG and DC Pulsed TIG Welder

3 CC

DC TIG

SALIENT FEATURES

• Three phase inverter based, High efficiency and High Power Factor Pulse TIG/ MMA DC Welder

• Useful for wide variety of material types and thickness.

• Full featured TIG controls possible

• HF ignition

• Intelligent protection: over/under voltage, over current / temperature

• Water cooled Torch with water cooling unit option.

• Latest PWM inverter technology

• High efficiency (>85%)

• Smooth and stable arc with spatter less welding

### CONTROL PANEL FEATURES

Current Upslope Start Current

Pre-flow

Pulse Current/ Main Current I

N Mma Mode (ip/ I1)

Cycle Current (I2) Base Current (Ib)

### SPECIFICATIONS

Main curernt in MMA mode

Crater Current

Post-flow

Current Downslope

Pulse Frequency (f)

Duty Ratio / Spot Time In Spot Mode (tp)

| Technical Specifications |  |  | Unit | CHAMPTIG 400P |
| --- | --- | --- | --- | --- |
| Input |  |  |  |  |
| Input Supply : |  |  |  |  |
| Voltage |  |  | Volt | 415 , +15%, -10% |
| Phase |  |  | No | 3 |
| Frequency |  |  | Hz | 50/60 |
| Efficiency @ 100% duty cycle |  |  | % | 85 |
| Power Factor @ 100% duty cycle |  |  |  | 0.94 Max |

Cont.

AMBALASCIENCEMARKET.COM- VIKRAM MITTAL /M-9812204116	www.modernkitchenequipments.com

104

# CHAMPTIG 400P

The World Class Inverter based DC TIG and DC Pulsed TIG Welder

Cont.

| Input KVA |  |  |  | MMA Mode | TIG Mode |
| --- | --- | --- | --- | --- | --- |
| @ 100% duty cycle |  |  | KVA | 13 | 10 |
| @ No Load |  |  | KVA | 0.13 | 0.13 |
| Output |  |  |  |  |  |
| Open Circuit Voltage |  |  | Volt | 70 |  |
| Welding Current Range |  |  |  | MMA Mode | TIG Mode |
| Welding Current |  |  | Amps | 50-400 | 10-400 |
| @ 100% duty cycle |  |  | Amps | 310 |  |
| @ 60% duty cycle (10 min cycle) |  |  | Amps | 400 |  |
| Pulse TIG Parameters |  |  |  |  |  |
| Duty Ratio |  |  | % | 10-90 |  |
| Pulse Frequency |  |  | Hz | 0.5-10 |  |
| Base Current |  |  | Amps | 10 - 90% of Ipulse current |  |
| Pulse Current ( Ipulse) |  |  | Amps | 10-400 |  |
| General |  |  |  |  |  |
| Start current |  |  | Amps | 10-400 |  |
| Current unpslope time |  |  | Sec | 0-10 |  |
| Current down slope control |  |  | Sec | 0-10 |  |
| Pre-Flow |  |  | Sec | 0-5 |  |
| Post-Flow |  |  | Sec | 0.1-20 |  |
| Crater current |  |  | Amps | 10-400 |  |
| Current / Voltage Display |  |  |  | 7-Segment LED Display for Current & Other parameters |  |
| Ingress Protection |  |  | Class | IP23 |  |
| Cooling |  |  | Type | Forced Air |  |
| Insulation |  |  | Class | H |  |
| Welding Output Terminals |  |  | - | Cam Lock connections |  |
| Dimensions |  |  |  |  |  |
| Length |  |  | mm | 660 |  |
| Width |  |  | mm | 315 |  |
| Height |  |  | mm | 485 |  |
| Weight (approx.) |  |  | Kg | 46 |  |
| Water Cooling Unit |  |  |  |  |  |
| Input Supply Voltage (AC) |  |  | Volts | 230 |  |
| Phase |  |  | - | 1 |  |
| Frequency |  |  | Hz | 50-60 |  |
| Water Reservoir Capacity |  |  | Liters | 8 |  |
| Flow rate |  |  | Ltr/min | 10 |  |
| Dimensions |  |  |  |  |  |
| Length |  |  | mm | 560 |  |
| Width |  |  | mm | 325 |  |
| Height |  |  | mm | 285 |  |

Cont.

AMBALASCIENCEMARKET.COM- VIKRAM MITTAL /M-9812204116	www.modernkitchenequipments.com

105

# CHAMPTIG 400P

The World Class Inverter based DC TIG and DC Pulsed TIG Welder

Cont.

| Weight (approx.) |  |  | Kg | 18 |
| --- | --- | --- | --- | --- |
| Trolley |  |  |  |  |
| Dimensions |  |  |  |  |
| Length |  |  | mm | 1005 |
| Width |  |  | mm | 500 |
| Height |  |  | mm | 109 |
| Weight (approx.) |  |  | Kg | 35 |

- Warranty : One year from the date of commissioning. ADOR WELDING LIMITED warrants that all new equipment sold from Plant/Area Offices / Authorised Distributors are free from defects in materials and workmanship and will perform in full accordance with applicable specifications.

- In view of continuous development, ADOR WELDING LIMITED reserves the right to modify/change the design and /or the specifications without any prior notice.

- Backed by dedicated customer care package.

AMBALASCIENCEMARKET.COM- VIKRAM MITTAL /M-9812204116	www.modernkitchenequipments.com

106

# CHAMPTIG 300 AD

The World Class Inverter based AC / DC Pulsed / Mixed Pulsed TIG Welding system

3 CC

DC TIG

AC TIG

SALIENT FEATURES

• Large input supply voltage range with under/over voltage

• Single phase protection

• Higher efficiency & power factor

• Lower power consumption

• In built spot TIG welding facility, AC/DC as well as mixed TIG Welding facility. In built pulse TIG welding controls with Independent settings of all parameters from front panel. Optionally welding current variation possible by using remote Control (hand held) or foot control regulator

• Cleaning width control in ac TIG welding enables user to adjust Oxide cleaning as well as penetration of weld puddle according to his choice

• Constant current characteristics irrespective of arc length variation

• High efficiency (≈82%)

• Smooth and stable arc with spatter less welding

• Light weight and compact TIG welding system

• Latest PWM inverter technology

### TECHNICAL SPECIFICATIONS

| Parameter | Unit | Value |  |
| --- | --- | --- | --- |
| Input |  |  |  |
| Supply Voltage, Phase Frequency : | Volts, AC | 415 (+15%-10%) 3 Phase, 50/60 Hz |  |
| Max output Input KVA @ 416 V Supply | KVA | MMA Mode | TIG Mode |
| @ 100% duty cycle |  | 7 | 7.5 |
| @ 60% duty cycle |  | 10 | 11 |
| Power Factor |  | Upto 0.92 |  |
| Efficiency | % | Upto 82 |  |
| Output |  |  |  |
| Open Circuit Voltage @ 416 V Supply | Volts, DC | 75 V DC (+/- 3V) |  |
| Welding Current Range |  | AC TIG Mode 20-300 DC TIG Mode 10-300 | MMA Mode 50-250 |
| Welding Current (40°C 10 minutes cycle) | Amps |  |  |
| @ 60% duty cycle |  | TIG Mode 300 Amps | MMA Mode 250 Amps |
| @ 100% duty cycle |  | TIG Mode 230 Amps | MMA Mode195 Amps |
| General |  |  |  |
| Remote Controller | 10 Meter | Provide as Optional for Current Settings |  |
| Foot Switch Control | 10 Meter | Provide as Optional for Current Settings |  |

Cont.

AMBALASCIENCEMARKET.COM- VIKRAM MITTAL /M-9812204116	www.modernkitchenequipments.com

107

# CHAMPTIG 300 AD

The World Class Inverter based AC / DC Pulsed / Mixed Pulsed TIG Welding system

Cont.

| Protections |  | Over Voltage, Under Voltage, Single Phasing, Over Temperature |
| --- | --- | --- |
| Front Panel Functions |  | MMA TIG Selection Switch |
|  |  | 2T/ 4T / Spot/ Cycle Selection Switch |
|  |  | DC+/ DC-/ AC Welding Method Selection Switch |
|  |  | Gas Check Switch |
|  |  | Pulse/ Normal Mode Selection Switch |
|  |  | HF On/ HF Off Selection Switch |
|  |  | Normal/ Foot Selection Switch |
|  |  | Menu Switch for Selecting All Functions Via Gas Pre Flow, Start Current, Up slope Time, Base Current, Pulse Current, Down Slope Time, Crater Current, Gas Post Flow Time, Cleaning for AC, AC Frequency, AC Offset, Pulse Width/Spot Time, Pulse Frequency, As Per Mode Selection Operation |
|  |  | Torch Switch Connector |
|  |  | Foot Switch Connector |
|  |  | Remote Connector |
|  |  | Gas Out |
|  |  | CAMLOCK output Connector |
|  |  | Mains On 'Green' Color Indication |
|  |  | Trip 'Red' Color LED for indication Machine is Under Protection Mode |
|  |  | Water/Gas Cooled Selection Switch |
|  |  | Encoder for Selected Parameter Vaule Increment/Decrement. |
| Cooling | Type | Forced Air |
| Ambient Temperature Rating | °C | 40 |
| Class of Insulation | - | H |
| Degree of Protection | - | IP23 |
| Dimensions L x W x H (Without handle) | mm | 650 x 425 x 515 |
| Weight (Approx) | Kg | 50 |

Cont.

AMBALASCIENCEMARKET.COM- VIKRAM MITTAL /M-9812204116	www.modernkitchenequipments.com

108

# CHAMPTIG 300 AD

The World Class Inverter based AC / DC Pulsed / Mixed Pulsed TIG Welding system

TIG Welding Parameter Setting

SPECIFICATIONS

| Parameter | Unit | Value |
| --- | --- | --- |
| Gas Pre Flow Time | Sec | 0-5 |
| Start Current | Amp | 5-300 |
| Current Unslope Time | Sec | 0-10 |
| Base Current Pulse Current | A A | 5-300 5-300 |
| Pulse Width | % | 10-90% of Width of Pulse Current |
| Pulse Frequency | Hz | 0.5 to 500 |
| Current Down Slope TIme | Sec | 0-10 |
| Crater Current Gas Post Flow Time | A Sec | 5-300 0.1-20 |
| Spot Time | Sec | 1-10 |
| Cleaning Control | % | (-40) - (+40) |
| AC Frequency Control | Hz | 20-100 |
| AC Offset | % | (-50) - (+30) |

- Warranty: One year from the date of commissioning. ADOR WELDING LIMITED warrants that all new equipment sold from Plant/Area Offices / Authorised Distributors are free from defects in materials and workmanship and will perform in full accordance with applicable specifications.

- In view of continuous development, ADOR WELDING LIMITED reserves the right to modify/change the design and /or the specifications without any prior notice.

- Backed by dedicated customer care package.

AMBALASCIENCEMARKET.COM- VIKRAM MITTAL /M-9812204116	www.modernkitchenequipments.com

109

# CHAMPTIG 500 AD

The World Class Inverter based AC / DC Pulsed / Mixed Pulsed TIG Welding system

3 CC

DC TIG

AC TIG

SALIENT FEATURES

• Latest Inverter based technology with constant output current

• IGBT based operating at high frequency

• Suitable for both TIG(AC & DC) and MMA welding operation

• Positive (DC+) & negative (DC-) polarity selection in MMA mode

• Option of HF ON & HF OFF in TIG mode, hence also suitable for scratch start in TIG mode Built HF ignition for easy arc striking.

• Welding process, mode selection and parameter adjustment by using keypad and multi function encoderprovided on digital front panel

• 7 segment LED display for actual current display & 7 segment LED display for actual voltage display & LCD display for other parameter display setting

• Anti stick function to protect the machine from short circuit condition in MMA mode.

• CRATER OFF (2T), CRATER ON (4T), SPOT, CYCLE mode of operation (TIG).

• Available with Cleaning width control and AC offset adjustment and AC frequency setting for AC TIG welding application.

• Adjustment of pulse current, pulse frequency, pulse width, AC current, AC frequency, cleaning control and AC offset, it can meet the requirement of seam depth, width and ripple, which can prolong the using life of tungsten electrode.

• Option of Remote control unit for setting current remotely in MMA and TIG

• Mode Option of Foot switch control regulator for setting current in TIG mode

• The outfit is provided with Water Cooling Unit and 500 Amp capacity Water cooled TIG Torch for heavy- duty TIG Welding

AMBALASCIENCEMARKET.COM- VIKRAM MITTAL /M-9812204116	www.modernkitchenequipments.com

110

# CHAMPTIG 500 AD

The World Class Inverter based AC / DC Pulsed / Mixed Pulsed TIG Welding system

TECHNICAL SPECIFICATIONS

| Parameter | Unit | Value |
| --- | --- | --- |
| Input |  |  |
| Supply Votage,phase,freqeuncy | Volts, AC | 415 V +15%, -10%, 3 Phase, 50 / 60 Hz |
| Input Power @ 415ac | KVA |  |
| @ 100 % Duty Cycle | KVA | MMA Mode - 19, TIG Mode - 15 |
| @ 60 % Duty Cycle | KVA | MMA Mode - 26, TIG Mode - 20 |
| @ 35 % Duty Cycle | KVA | MMA Mode - 34, TIG Mode - 27 |
| Input Current @ 415vac | Amps, AC |  |
| @ 100 % Duty Cycle | Amps, AC | MMA Mode - 26, TIG Mode - 21 |
| @ 60 % Duty Cycle | Amps, AC | MMA Mode - 36, TIG Mode - 28 |
| @ 35 % Duty Cycle | Amps, AC | MMA Mode - 47, TIG Mode - 38 |
| Efficiency | % | UP TO 82 |
| Power Factor |  | 0.8 Max |
| Output |  |  |
| Open Current Voltage @ 415v Input Supply | Volts, AC | 78 V DC (+/-5 V)-DC MMA, TIG; 78 V AC (+/-5 V)-AC TIG |
| Welding Current Range | Amps | MMA Mode 50 - 500, AC / DC TIG Mode 20 - 500 |
| Welding Current At 40deg C, 10 Minute Cycle |  |  |
| @ 100 % Duty Cycle |  | 310 |
| @ 60 % Duty Cycle |  | 400 |
| @ 35 % Duty Cycle |  | 500 |
| General |  |  |
| Remote Controller / Foot Switch Controller | 10 Meter | Provided As Optional For Current Setting. |
| Protections |  | Over Voltage, Under Voltage, Single-phase, Over-temperature Protection. |
| Front Panel Functions |  | 1. Selector Switches For Selection of: MMA / TIG, 2T / 4T / Spot / Cycle, Dc+/dc-/ac Welding Method, NormPulse / Normal Mode, Hf On / Off,al / Food Switch Control,Water / Gas Cooled Torch Selection |

Cont.

AMBALASCIENCEMARKET.COM- VIKRAM MITTAL /M-9812204116	www.modernkitchenequipments.com

111

# CHAMPTIG 500 AD

The World Class Inverter based AC / DC Pulsed / Mixed Pulsed TIG Welding system

Cont.

|  |  | 2. Gas Check Switch |
| --- | --- | --- |
|  |  | 3. Menu Switch For Selecting All Functions Via. Gas Pre Flow Start Current, Upslope Time, Base Current, Pulse Current, Down Slope Time, Crater Current, Gas Post Flow Time, Cleaning For Ac, Ac Frequency, Ac Offset, Pulse Width/ Spot Time, Pulse Frequency As Per Selected Mode of Operation. |
|  |  | 4. Connect For Torch Switch, Foot Switch, Remote, Gas Out, Camlock Power Output |
|  |  | 5. Mains On ‘green’ Colour And Trip ’red’’ Indication |
|  |  | 6. Encoder For Selected Parameter Value Increment / Decrement |
| Cooling | Type | Forced Air |
| Class Of Insulation | - | H |
| Degree Of Protcetion | - | IP23(S) |
| Dimension (L x W x H) | mm | 780 X 355 X 620 |
| Weight (Approx.) | Kg. | 66 |

THE WELDING PARAMETER SETTINGS

| Parameter | Unit | Value |
| --- | --- | --- |
| Gas Pre Flow Time | Sec. | 0 - 5 |
| Initial Welding Current - TIG | Amps | 20 - 500 |
| Current Upslope Time | Sec. | 0 - 10 |
| Base Current TIG | Amps | 20 - 500 |
| Pulse Current (TIG) | Amp. | 20 - 500 |
| Pluse Width | % | 10 - 90% Of Pulse Time Period (1 Second - 2 Milisecond) |
| Pluse Frequency | Hz | 1 - 500 |
| Cuurent Downslope Time | Sec. | 0 - 10 |
| Crater Current (tig) | Amps | 20 - 500 |
| Gas Post-flow Time | Sec. | 0.1 - 20 |
| Sopt Time | Sec. | 1 - 10 |
| Cleaning Control |  | -40 to +40 |
| Ac Frequencey Control |  | 20 - 50 |
| Ac Frequencey Control | % | -50 to +30 |

Cont.

AMBALASCIENCEMARKET.COM- VIKRAM MITTAL /M-9812204116	www.modernkitchenequipments.com

112

# CHAMPTIG 500 AD

The World Class Inverter based AC / DC Pulsed / Mixed Pulsed TIG Welding system

SPECIFICATIONS OF WATER COOLING UNIT

| Parameter | Unit | Value |
| --- | --- | --- |
| Input Supply | V ac | 240 |
| Phase | No. | 1 |
| Frequency | Hz | 50 |
| Cooling Capacity | W | 750 - 1000 |
| Maximum Motor Power | W | 220 |
| Maximum Pump Pressure | Bar | 2.5 |
| Maximum Pump Flow Rate | L/Min | 10 |
| Tank Capacity | L | 8 |
| Connector Size | In/Out | 1/42 X9 |
| Dimensions (L x W x H) | Mm | 560 X 325 X 285 |
| Weight | Kg. | 18 |

• Warranty : One year from the date of commissioning. ADOR WELDING LIMITED warrants that all new equipment sold from Plant/Area Offices / Authorized Distributors are free from defects in materials and workmanship and will perform in full accordance with applicable specifications.

• In view of continuous development, ADOR WELDING LIMITED reserves the right to modify/change the design and

/or the specifications without any prior notice.

• Backed by dedicated customer care package.

AMBALASCIENCEMARKET.COM- VIKRAM MITTAL /M-9812204116	www.modernkitchenequipments.com

113

# MAESTRO SERIES 800, 1000, 1200

A range of SUBMERGED Arc Welding Equipment to suit a wide range of applications

SALIENT FEATURES

• Choice of Boom mounted or Tractor mounted Welding head models offered with Diode or Thyristorized Power Source

• Boom mounted head comes with a choice of manual, semi – motorized and fully motorized cross slides

• Programmed sequential operations of Power Source, Wire feed and carriage through built- in solid state circuitry

• Twin Wire Attachment : Twin Wire Attachment is available for the Tractor as well as boom mounted welding heads. The arrangement consists of one set of each of wire drive roll and nozzle for 2 and 2.5 mm size wire to feed two wires in tandem and increase

the deposition rate substantially. These arrangements is ideal for applications like cladding of hard surfaces as well as for filing weld metal in extra large size preparation grooves. Twin 2.5 mm wires can deliver up to 20% higher deposition rates than a single 5 mm wire at the same welding current and arc voltage parameters.

• Fillet Welding Attachment : To enhance the usability of SAW Outfits, a Fillet Welding attachment can be provided to increase shop-floor productivity.

• Spot Light Projector : This facility is especially useful for welding operations inside cylindrical jobs where a light source is required to examine the seam

##### The Welder - friendly tractor or boom mounted welding head with Diode / Thyristorized Power sources perfectly match; Adopted to local conditions; offering a range of models to suits every budget and need

AMBALASCIENCEMARKET.COM- VIKRAM MITTAL /M-9812204116	www.modernkitchenequipments.com

114

# MAESTRO SERIES 800, 1000, 1200

A range of SUBMERGED Arc Welding Equipment to suit a wide range of applications

Cont.

TECHNICAL SPECIFICATIONS DOIDE BASED SYSTEMS	CV

| Power Source | Unit | PS 800 (F) | PS 1000 (F) | PS 1200 (F) |
| --- | --- | --- | --- | --- |
| Input |  |  |  |  |
| Input Supply: |  |  |  |  |
| Voltage | Volts, AC | 415 | 415 | 415 |
| Phase | No | 3 | 3 | 3 |
| Frequency | Hz | 50 | 50 | 50 |
| Input Kva @ 100% Duty Cycle | KVA | 37 | 45 | 54 |
| Recommended Switch Fuse Rating | Amps, AC | TP-60 | TP-80 | TP-100 |
| Output |  |  |  |  |
| Static Characteristics | Type | CV | CV | CV |
| Open Circuit Voltage Range (max. With IPT) | Volts, DC | 35-63 | 35-63 | 35-63 |
| Open Circuit Voltage Range (Actual) |  | 29-54 | 29-54 | 29-54 |
| Welding Voltage |  | 21.15-44 | 24-44 | 24-44 |
| Welding Current Range | Amps, DC | 150-800 | 200-100 | 200-1200 |
| Welding current @ 60% duty cycle |  | 800 | 100 | 1200 |
| Welding current @ 100% duty cycle |  | 650 | 800 | 1000 |
| General |  |  |  |  |
| Insulation | Class | H | H | H |
| Cooling | Type | Forced Air | Forced Air | Forced Air |
| Dimensions L x W x H | mm | 1225x800x900 | 1225x800x900 | 1225x800x900 |
| Weight (Approx) | Kg | 350 | 365 | 390 |

TECHNICAL SPECIFICATIONS THYRISTOR BASED SYSTEMS	CV

| Power Source | Unit | PS 800 T(F) | PS 1000 T(F) | PS 1200 T(F) |
| --- | --- | --- | --- | --- |
| Input |  |  |  |  |
| Input Supply: |  |  |  |  |
| Voltage | Volts, AC | 415 | 415 | 415 |
| Phase | No | 3 | 3 | 3 |
| Frequency | Hz | 50 | 50 | 50 |
| Input Kva @ 100% Duty Cycle | KVA | 44 | 50 | 63 |
| Recommended Switch Fuse Rating | Amps, AC | TP-70 | TP-80 | TP-100 |
| Output |  |  |  |  |
| Static Characteristics | Type | CV | CV | CV |
| Open Circuit Voltage Range (max.) | Volts, DC | 62 | 62 | 62 |
| Open Circuit Voltage Range |  | 150-800 | 200-1000 | 200-1200 |
| Welding current @ 60% duty cycle | Amps, DC | 800 | 100 | 1200 |
| Welding current @ 100% duty cycle |  | 650 | 800 | 1000 |

AMBALASCIENCEMARKET.COM- VIKRAM MITTAL /M-9812204116

www.adorwelding.coCmont.

115

# MAESTRO SERIES 800, 1000, 1200

A range of SUBMERGED Arc Welding Equipment to suit a wide range of applications

Cont.

| General |  |  |  |  |
| --- | --- | --- | --- | --- |
| Insulation | Class | H | H | H |
| Cooling | Type | Forced Air | Forced Air | Forced Air |
| Dimensions L x W x H | mm | 1225x800x1060 | 1225x800x1060 | 1225x800x1060 |
| Weight (Approx) | Kg | 350 | 365 | 390 |

ORDERING INFORMATION OPTIONAL ACCESSORIES

| Description | Product Code |
| --- | --- |
| Set Of Welding Cables SAC 800, For PS 800 (F), 800 T (F) | S10.36.011.0005 |
| Set Of Welding Cables SAC 1000, For PS 800 (F), 1000 T (F) | S10.36.011.0006 |
| Set Of Welding Cables SAC 1200, For PS 800 (F), 1200 T (F) | S10.36.011.0004 |
| Description | Product Code |
| WAS : Set Of Welders Accessories - All Models | S10.36.041.0001 |
| SLP: Spotlight Projector - All Models | S10.36.031.0001 |
| FWA: Fillet Welding Attachment - All Models | S10.36.021.0001 |
| TWA: Twin Wire Attachment (2 And 2.5 Mm) - All Models | S10.36.021.0003 |

TECHNICAL SPECIFICATIONS - WELDING HEADS

| Saw Head | Unit | WH -15(F) | WH -15-01(F) | WH -15-02(F) | WH -15-03(F) |
| --- | --- | --- | --- | --- | --- |
| Max. Welding Current @ 60% Duty Cycle | Amps, DC | 1500 | 1500 | 1500 | 1500 |
| Max. Welding Current @ 60% Duty Cycle |  | 1250 | 1250 | 1250 | 1250 |
| Recommended Power Source |  | PS 800 (F) / PS 1000 (F) / PS 1200 (F) |  |  |  |
| Recommended Wire Diameter Single Wire | mm | 2, 2.5, 3.2 For 800 (F); 2, 2.5, 3.2, 4.5 For 1000 (F) & 1200 (F) |  |  |  |
| Recommended Wire Diameter Twin Wire |  | 2, 2.5 (Optional) |  |  |  |
| Speed Range Wire Feed | M / Min. | 0.5-4 | 0.5-4 | 0.5-4 | 0.5-4 |
| Speed Range Carriage |  | 0.1-1.5 | NA | NA | NA |
| Input To Control Unit | Volts, AC | 42 V, 1 PH, 50 Hz From Power Source |  |  |  |
| Range Of Adjustment |  |  |  |  |  |
| Vertical (Z) | mm | 140 (Manual) | 140 (Manual) | 100 (Motorized) | 100 (Motorized) |
| Horizontal (X)-Transverse To Travel Direction |  | 140 (Manual) | 140 (Manual) | 100 (Motorized) | 100 (Motorized) |
| Horizontal (7)-In Direction Of Travel |  | - | 140 (Manual) | 140 (Manual) | 100 (Motorized) |
| Maximum Swivel |  |  |  |  |  |
| Transverse To Head Travel | Degree | 45 |  |  |  |
| Parallel To Head Travel |  | 30 |  |  |  |
| Horizontal Swing |  | 270 |  |  |  |
| Standard Bore For Spool | mm | 285 - 315 (Adjustable) |  |  |  |
| Weight Of Spool (max.) | Kg | 25 |  |  |  |
| Flux Hopper Capacity (max.) | Kg/Ltrs | 10/7 |  |  |  |
| Dimensions (L X W X H) | mm | 1500x1030x580 | 1500x1030x580 | 1500x1030x600 | 1500x1030xC6o0n0t. |
| Weight (without Flux And Wire) | Kg | 80 | 80 | 110 | 110 |

AMBALASCIENCEMARKET.COM- VIKRAM MITTAL /M-9812204116	www.modernkitchenequipments.com

116

# MAESTRO SERIES 800, 1000, 1200

A range of SUBMERGED Arc Welding Equipment to suit a wide range of applications

• Warranty : One year from the date of commissioning. ADOR WELDING LIMITED warrants that all new equipment sold from Plant/Area Offices / Authorised Distributors are free from defects in materials and workmanship and will perform in full accordance with applicable specifications.

• In view of continuous development, ADOR WELDING LIMITED reserves the right to modify/change the design and /or the specifications without any prior notice.

• Backed by dedicated customer care package.

AMBALASCIENCEMARKET.COM- VIKRAM MITTAL /M-9812204116	www.modernkitchenequipments.com

117

INTERFACE UNIT WITH INTERCONNECTION CABLE

INTERFACE UNIT WITH INTERCONNECTION CABLE

UNI - MAESTRO RETROFIT UNIT FOR SAW

The customized Retrofit unit (complete with new welding head) for Interfacing new welding head (diode based) with old model of SAW Power Source

- The UNI-MAESTRO retrofit box is a “BORN AGAIN PACKAGE (Rejuvenate)” to upgrade productivity from existing old working power source, Model: CPR-1204 / CPR-1205 / PS-1200, but with welding head which is in non-working condition or the customer is not having welding head.

- Old version of “UP-15-CP / WH-15” series Welding Heads are upgraded with New “Feather-Touch controlled WH-15 (F)” series (Diode based) Welding Heads, as old version of “UP-15-CP / WH-15” are discontinued from AWL production range.

- The complete outfit consists of Retrofit box, tractor mounted or boom mounted submerged arc welding unit, inter-connection control cable between Power source and Retrofit box. These are interfaced with suitable capacity of old power source CPR-1204 / CPR-1205 / PS- 1200, which is already available with the customer.

COMPLETE SYSTEM

##### The complete system or outfit consists of:

- Retrofit Box (Interface Box)

- Welding Head type WH 15 (F) or WH 15-01 (F) => suitable for Diode based systems only

- Interconnection control cable between power source & Welding Head - 10 meters length

AMBALASCIENCEMARKET.COM- VIKRAM MITTAL /M-9812204116	www.modernkitchenequipments.com

118

# UNI - MAESTRO RETROFIT UNIT FOR SAW

#### The customized Retrofit unit (complete with new welding head) for

Interfacing new welding head (diode based) with old model of SAW Power Source

CONNECTION SYSTEM

EXISTING OLD DIODE BASED SAW POWER SOURCE - CUSTOMER

WELDING HEAD

### ORDERING INFORMATION

| UNI - MAESTRO RETROFIT UNIT (Comprising of Interface box, Welding Head Assembly type WH – 15 (F) Tractor mounted, 10 Meter Interconnection cable between Interface Box and Welding Head) | S15.01.001.0601 |
| --- | --- |
| UNI - MAESTRO RETROFIT UNIT (Comprising of Interface box, Welding Head Assembly type WH – 15 01 (F) Boom mounted, 10 Meter Interconnection cable between Interface Box and Welding Head) | S15.01.001.0602 |

AMBALASCIENCEMARKET.COM- VIKRAM MITTAL /M-9812204116	www.modernkitchenequipments.com

119

# THREE WHEEL TRACTOR WELDING HEAD

The new welding head for flexible operations

CV

### SALIENT FEATURES

•		The Three wheel tractor is more versatile compared to the 4 wheel tractor; it shortens the welding spacing and has a wider welding range for wires from Φ2.0 to Φ5.0 mm.

• Besides linear welding, the three wheel tractor can turn left and right and weld around circular curve.

### SPECIFICATIONS

• Easy and flexible regulation, integrated up and down, rotation of the tractor head with the torch.

• Easy adjustable flux container.

• Double driven wire feeding with straightening mechanism, stable wire feeding, good centering, strong drawing force and low dissipation power.

| Technical Specifications | Unit | VALUE |
| --- | --- | --- |
| Input |  |  |
| Rated Input Voltage Of Traveling Mechanism | Volts DC | 42 V |
| Rated Input Current Of Traveling Mechanism | Amps DC | 5.5 Amps |
| Rated Input Voltage Of Wire Feeding Mechanism | Volts DC | 42 V |
| Rated Input Current Of Wire Feeding Mechanism | Amps DC | 0.75 Amp |
| Welding Speed | Meters / Minute | 0.2 - 1.5 / 0.2 - 2 M / Min |
| Wire Feeding Speed | Meters / Minute | 0.6 - 6 M / Min |
| Rated Power Source | Capacity | 1200 Amps @ 60% Duty Cycle |
| Wire Diameters | mm | 2, 3, 4, 5, |
| Type Of Wires Applicable |  | Soft Steel, Solid Core |
| Adjustable Height Of Tractor Head | mm | 70 MM |
| Adjustable Distance Of Tractor Head | mm | 100 X 100 (longitudinal X Transversal) |
| Rotatable Angle Of Tractor Head Around Cross Beam | Degree | ± 90˚ |
| Tractor Head Inclination | Degree | ± 45˚ |
| Torch Inclination | Degree | ± 45˚ |
| Flux Container Volume | Litres | 6 L |
| Inner Diameter Of Wire Spool | mm | Ø 300 mm |
| Wire Spool Capacity | Kg | 25 KG |
| Dimension (L X W X H) | mm | 1020 X 480 X 780 |
| Weight | Kg | 50 Kg (Does Not Include Wire) |
| Insulation | Class | F |
| Welding Output Terminals |  | Stud type / Camlock |

AMBALASCIENCEMARKET.COM- VIKRAM MITTAL /M-9812204116	www.modernkitchenequipments.com

120

# MAESTRO 1200 (I)

The world Class Inverter based Submerged Arc Welder

SALIENT FEATURES

• Inverter based digitally controlled Welding head and Power source.

• High efficiency and high power factor resulting in substantial energy saving over conventional SAW outfits

• Enhanced Reliability due to SMD technology.

• Power source is CC-CV type and capable for MMA/GOUGING and SAW processes.

• *****SAW Welding can be done in either CV or CC modes. In the Constant Current mode of SAW welding, advanced Adaptive control technology is utilized to obtain the most stable arc parameters, highest level of penetration, and excellent weld bead finish******

### FRONT PANEL FUNCTIONS

• Mains On 'Green' Colour Led Indication

• Operation Mode Indications (MMA / Gouging, Saw, Remote)

• Trip 'Red' Colour Led Indication of machine is Under Protection Mode.

• The complete system consists of inverter based power source; tractor / boom mounted welding head and interconnecting cables. SAW flux is used for shielding weld metal against external atmosphere.

• Inverter based power source is energy efficient giving almost 30% energy saving over conventional type of machines.

• User can save and recall up to ten programs.

• Better user interface having 128 X 64 character LCD display for selecting various modes and button, LED and indicator lamp, which helps in easy operation of the equipment.

• Automatically selects the MMA or SAW mode based on welding head connections to power source.

• 4 digit digital displays for Voltage and Current.

•  Current adjustment Encoder .

AMBALASCIENCEMARKET.COM- VIKRAM MITTAL /M-9812204116	www.modernkitchenequipments.com

121

# MAESTRO 1200 (I)

The world Class Inverter based Submerged Arc Welder

WELDING HEAD CONTROL PANEL :

##### The Welding Head Control panel consists of four encoders for selecting welding parameters selection, welding current selection, welding voltage selection and carriage speed selection respectively.

- Welding parameters can be seen on 128*64 character LCD as

  - MATERIAL: MS/SS

  - DIAMETER: 2-5mm

  - WIRESPEED: 0-4 M/MIN

  - UP TIME: 01-10 sec

  - SAVE: 01-10

  - RECALL: 01-10

- Proper parameters can be selected by rotating the respective encoders.

- A facility has been given for saving the welding parameters settings in at most 10 memory locations. So the same settings can again be recalled even after turning On the machine, by rotating RECALL option.

- Another special feature of UP TIME is given to the user in between 1 second to 10 second, so that after welding is OFF, wire won't stick in to the weld pool. In this case wire feeder motor will rotate in opposite direction to pull out the wire for the specified seconds.

- The Front panel consists of seven tact switches, for selecting various functions such as carriage movement direction (FORWARD / REVERSE) selection, carriage motor ON / OFF command, wire inch UP / DOWN switch and welding ON / OFF switch.

- CC-CV Mode Selection switch for SAW is provided on panel to suit specific SAW Welding requirements

### TECHNICAL SPECIFICATIONS WELDING HEAD (WH 15 I)

| Input | Unit | Value |
| --- | --- | --- |
| Input Supply: |  |  |
| Voltage | Volt | 42 V (From Power Source) |
| Output |  |  |
| Welding Current Range | Amps | 100-1200 |
| Duty Cycle 100% Continues | Amps | 1000 |
| Welding Voltage Range | Volts, DC | 26-44 |
| Welding Carriage Speed | M/Min | 0.1-1.6 |
| Wire Feed Speed | M/Min | 0-4 |
| General |  |  |
| Range Of Adjustment Vertical | mm | 140 |
| Transverse To Head Travel - Horizontal |  | 140 |
| Parallel To Head Travel - Horizontal |  | 140 |
| Max Swivel : |  |  |
| Transverse To Hand Travel | Degree | 45° |

Cont.

AMBALASCIENCEMARKET.COM- VIKRAM MITTAL /M-9812204116	www.modernkitchenequipments.com

122

# MAESTRO 1200 (I)

The world Class Inverter based Submerged Arc Welder

Cont.

| Parallel To Head Travel | Degree | 30° |
| --- | --- | --- |
| Horizontal Open Swing |  | 270° |
| Standard Bored Spool | mm | 285-315 (Adjustable) |
| Welding Flux-Hooper Capacity | Kg./Ltr. | 10/7 |
| Wire Spool Weight | Kg | 25 |
| Suitable Welding wire Diameter | mm | 2.0,2.5,3.15,4.0,5.0 |
| Dimension (L x W x H) | mm | 1500 x 1030 x 580 |
| Total Weight Without Flux & Wire | Kg. | 95 |

TECHNICAL SPECIFICATIONS - POWER SOURCE

#### CC/CV

GOUGING

| Parameter | Unit | Value |
| --- | --- | --- |
| Input |  |  |
| Supply Voltage, Phase Frequency : | Volts AC | 415 V +15% -10%, 1 Phase, 50/60 Hz |
| Input Power @ 415 VAC @ 100% duty cycle | KVA | 55 |
| Input Power @ 415 VAC @ 60% duty cycle |  | 66 |
| Input Supply Current @ 415 VAC @ 100% duty cycle (1000) | Amps DC | 76 |
| Input Supply Current @ 415 VAC @6 0% duty cycle(1200) |  | 92 |
| Efficiency | % | > 85 |
| Power Factor |  | Upto 0.93 |
| Output |  |  |
| Open Circuit Voltage @415 V Input Supply | Volts DC | 90 V |
| Welding Current Range (CC-GOUGING Mode) | Amps | 100-1200 |
| Welding Voltage Range (CC-SAW Mode) |  | 26-44 |
| Welding Current (40°C) @ 100% duty cycle | Amps DC | 1000 |
| Welding Current (40°C) @ 60% duty cycle (10 min DC) |  | 1200 |
| Welding Electrode Sizes (Dia.) In MMA Aplication | mm | 3.2,4,5,6.3 mm |
| Welding Electrode Sizes (Dia.) In GOUGING Aplication |  | Upto 12 mm |
| SAW Welding Wire Size (Dia.) | mm | 2.5,3.2,4,5 mm |
| Remote Controller Optional (Can be Used In MMA/ GOUGING Process only) | 10 Meter | Provide as Optional for Remote Current Detting |
| General |  |  |
| Front Panel Functions |  | Mains On 'Green' Coloir Indication |
|  |  | Operation Mode Indications (MMA/ GOUGING, SAW, REMOTE) |
|  |  | Trip 'Red' Colour LED for indication Machine is Under Protection Mode |
|  |  | 4 Digit Display for Voltage & Current |
|  |  | Current Adjustment Encoder |

Cont.

AMBALASCIENCEMARKET.COM- VIKRAM MITTAL /M-9812204116	www.modernkitchenequipments.com

123

# MAESTRO 1200 (I)

The world Class Inverter based Submerged Arc Welder

Cont.

| Protections | - | Over Voltage, Under Voltage, Single Phasing, Over Temperature |
| --- | --- | --- |
|  |  | Over Temperature Protection |
| Cooling | Type | Forced Air |
| Class Of Insulation | - | H |
| Degree Of Protection | - | IP23 |
| Dimensions L x W x H (Without handle) | mm | 930 x 525 x 950 |
| Weight (Approx) | Kg | 115 |

• Warranty : One year from the date of commissioning. ADOR WELDING LIMITED warrants that all new equipment sold from Plant/Area Offices / Authorised Distributors are free from defects in materials and workmanship and will perform in full accordance with applicable specifications.

• In view of continuous development, ADOR WELDING LIMITED reserves the right to modify/change the design and /or the specifications without any prior notice.

• Backed by dedicated customer care package.

AMBALASCIENCEMARKET.COM- VIKRAM MITTAL /M-9812204116	www.modernkitchenequipments.com

124

# ELECTRODE AND FLUX

DRYING OVENS

For Welding Electrodes and Submerged Arc Welding Fluxes

TYPICAL APPLICATIONS

Reliable Ovens which works hour after hour, day after day the same consistency.

Temperature Control : Provided with thermostat control for precise setting of drying temperature required for various types of electrodes.

### SALIENT FEATURES

•		Heavy duty drying ovens for welding electrode & submerged arc welding fluxes

•	Robust constructions

•	ADOR HANDY model portable Electrode drying oven for quick shifting from one place to the other

Flawless Welds : Totally dry welding consumables yield radiographic quality welds.

•	User friendly

•	Uniform heating of charged materials

• Heavy duty thermostats for accurate temperature controls throughout range

•	Heavy duty metallic body

### TECHNICAL SPECIFICATIONS

| Models | ADOR HANDY | ADORDRY-I | KING OVEN-II | ADORDRY-III | ADORFLUX |
| --- | --- | --- | --- | --- | --- |
| Input |  |  |  |  |  |
| Voltage | 230V AC | 230V AC | 230V AC | 230V AC | 415V AC |
| Current (Amps) | 2 | 7.5 | 10 | 20 | 10 |
| Phase | 1 Phase | 1 Phase | 1 Phase | 1 Phase | 3 Phase |
| Frequency | 50 Hz | 50 Hz | 50 Hz | 50 Hz | 50 Hz |
| Watts | 0.5 KW | 1.75 KW | 2.25 KW | 4.5 KW | 12 KW |
| Output |  |  |  |  |  |
| Temperature | 50˚C - 300˚C | 50˚C - 250˚C | AMBIENT + 5° C - 370° C | 50°C - 500°C | 50°C - 400°C |

AMBALASCIENCEMARKET.COM- VIKRAM MITTAL /M-9812204116	www.modernkitchenequipments.com

125

# ELECTRODE AND FLUX DRYING OVENS

For Welding Electrodes and Submerged Arc Welding Fluxes

Cont.

| Models | ADOR HANDY | ADORDRY-I | KING OVEN-II | ADORDRY-III | ADORFLUX |
| --- | --- | --- | --- | --- | --- |
| General Specifications |  |  |  |  |  |
| Capacity | 5 KG ELECTRODES | 25 KG ELECTRODES | 25 KG ELECTRODES | 25 KG ELECTRODES | 100 KG FLUX |
| DIMENSIONS IN MM (L X W X H) | EXTERNAL: 515 X 190 X 190 INTERNAL: 435 (L) X 62 (Ø) | 490 X 450 X 440 | 490 X 450 X 440 | 490 X 450 X 440 | 750 X 600 X 600 |
| No.of Shelves | - | Five | Five | Five | Six |
| Air Circulation |  | NATURAL CONVECTION | NATURAL CONVECTION | NATURAL CONVECTION | FORCED AIR |
| Temperature Control | THERMOSTAT | THERMOSTAT | THERMOSTAT WITH ELECTRONIC CONTROLLER & DIGITAL TEMPERATURE INDICATOR |  |  |

- Warranty : One year from the date of commissioning. ADOR WELDING LIMITED warrants that all new equipment sold from Plant/Area Offices / Authorised Distributors are free from defects in materials and workmanship and will perform in full accordance with applicable specifications.

- In view of continuous development, ADOR WELDING LIMITED reserves the right to modify/change the design and /or the specifications without any prior notice.

- Backed by dedicated customer care package.

AMBALASCIENCEMARKET.COM- VIKRAM MITTAL /M-9812204116	www.modernkitchenequipments.com

126

Our Global Footprints

Head Office Manufacturing Facilities

Corporate Marketing Office